3GPP TSG-RAN WG4#114	R4-2501581

Athens, Greece, February 17th – 21st, 2025

Agenda item:	4.2.2

Source: 	Qualcomm France

Title: 	On UL RMC’s for SRS transmissions

Document for:	Approval

# Introduction

Considerations on adding UL RMC’s for SRS transmissions are provided in this contribution.

- Discussion

In RAN4#113, CR’s for Rel-17 [1] and Rel-18 [2] were proposed which would add UL RMC’s to verify SRS transmitter power requirements and the ΔTRxSRS for antenna switching.

Considering the extensive discussion on “SRS IL imbalance reporting” during the past several years both in RAN1 and RAN4 without any conclusions so far, some clarifications from RAN4 perspective are necessary so that RAN5 can make the test case properly. Current RAN4 specification does not define UE behavior for SRS antenna switching at lower than maximum output power levels. Hence SRS Tx power for antenna switching should be verified only with maximum output power level. RAN4 should send an LS to RAN5 describing the conditions in which the SRS Tx power for antenna switching shall be verified.

If the intention in [1],[2] would be to test the SRS Tx power at lower than maximum power level, RAN4 should liaise with RAN1 on what kind of UE behavior to expect.

Proposal 1: The following is agreed for SRS Tx power verification from Rel-17 onwards:

- Current RAN4 specification defines UE behavior for SRS Tx power in antenna switching only for maximum output power, i.e. in case where PEMAX,c is not limiting the output power

- The requirements shall be verified only at maximum output power (PCMAX,f,c)

- PEMAX,c shall not limit the PCMAX,f,c  in the test case i.e. it should be 23dBm for PC2, 26dBm for PC2 and 29dBm for PC1.5

- Note: In case RAN4 agrees to amend Rel-19 or later specifications to include UE behavior for SRS Tx power in antenna switching at lower than maximum output power, it will inform RAN5

- Send an LS to RAN5 including the information in bullets above

- Conclusion

Considerations on adding UL RMC’s for SRS transmissions were provided with the following proposal.

Proposal 1: The following is agreed for SRS Tx power verification from Rel-17 onwards:

- Current RAN4 specification defines UE behavior for SRS Tx power in antenna switching only for maximum output power, i.e. in case where PEMAX,c is not limiting the output power

- The requirements shall be verified only at maximum output power (PCMAX,f,c)

- PEMAX,c shall not limit the PCMAX,f,c  in the test case i.e. it should be 23dBm for PC2, 26dBm for PC2 and 29dBm for PC1.5

- Note: In case RAN4 agrees to amend Rel-19 or later specifications to include UE behavior for SRS Tx power in antenna switching at lower than maximum output power, it will inform RAN5

- Send an LS to RAN5 including the information in bullets above

- Reference

[1] R4-2418870, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, Ericsson

[2] R4-2418871, “(NR_RF_TxD-Core) Reference measurement channels for SRS transmissions”, Ericsson