3GPP TSG-RAN WG4#114	R4-2501583

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.1.1.3.4

Source: 	Qualcomm France

Title: 	About SRS IL Imbalance indication

Document for:	Discussion

# Introduction

SRS IL imbalance indication considered in this contribution.

- WF from RAN4#113

WF was agreed in RAN4#113 [1]

## Sub-topic 3-1:	General considerations for SRS IL imbalance issue

Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1.

Issue 3-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1.

## Sub-topic 3-2:	SRS IL imbalance issue solutions

Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed..

Candidate solution framework:

-	UE performs self-compensation of SRS IL up to its configured maximum output power.

-	FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact.

-	UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g.

-	Per SRS resource power headroom reporting

-	Configured maximum output power per SRS resource reporting

-	SRS insertion loss value reporting (per SRS resource or per UE)

-	Other

-	ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting.

-	Others

-	Details of SRS IL self-compensation and assistance framework are FFS

- Conclusion

After numerous discussions during the past years, RAN4 should stop discussions on this topic.

After looking into this in very detail we note that the benefits, if any, would be very small especially for UE’s compensating the SRS IL. In far-cell scenario where compensation may not be possible, SRS may not be that beneficial anyhow independent of indication or no indication. In any kind of indication there would be quite a bit of additional complexity, and no quarantee that NW would use any of the additional information.

Proposal 1: RAN4 should not continue the discussion on SRS IL imbalance indication.

- Conclusion

SRS IL imbalance indication considered in this contribution, with the following proposal.

Proposal 1: RAN4 should not continue the discussion on SRS IL imbalance indication.

- Reference