# 3GPP TSG-RAN WG4 Meeting #114  		R4-2501598

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on RRM requirements for UE initiated Beam reporting in R19 MIMO

Agenda Item:	7.22.3.1

Document for:	Discussion

Introduction

In RAN4 #113, the WF [1] is approved. Moreover, an incoming LS from RAN1 is received [2].

Based on all above information, we provide our views on RRM requirements for UE initiated beam reporting.

Discussion

<On how to define requirements for event-triggered L1 reporting>

In last meeting, RAN4 has also discussed whether UE shall update the L1 measurement results during the time UE wait for uplink resources. According to RAN1 agreements, UE may not know when gNB will send the CSI request field in DCI, after first PUCCH is reported. RAN1 is also discussing first PUCCH retransmission. It is possible from UE perspective that, at the time when actual L1 measurement report shall be sent, the event that triggers the reporting is already exited. In our view, this is related to the report cancelling mechanism, which shall be discussed in RAN1/2 first. On the other hand, RAN4 may proceed without considering further updates to the results reported by UE after first PUCCH is sent. For simplicity, it is proposed to consider a testable scenario, i.e. the condition over the air is fixed after the event is triggered.

Proposal 1  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions:

- the condition which triggers the event is remained fixed during the time UE performs L1 measurements.

- the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting..

RAN4 may further discuss other cases if time allows.

Moreover, RAN1 has also agreed at least Event-2. Some details about agreements Event-2 is cited as follows.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, there is no RAN1 consensus on the following enhancement for handling the case that only one TRS is configured in the indicated TCI state in RAN1#118bis - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). Note 3: When only one TRS is configured in the indicated TCI state, either Scheme-1(working assumption) or Scheme-2 is used where enabling one of either Scheme-1 or Scheme-2 is selected by NW. When the Scheme-1 is used, the UE assumes that the CSI-RS resource in the indicated TCI state is configured in a CSI-RS resource set configured with repetition.

RAN1 has agreed to use a serving cell RS, which is derived based on indicated TCI state, as the RS for current beam. This RS can be either SSB or CSI-RS, and RAN1 has already completed the rule for determine the RS from indicated TCI state. Based on definition in TS 38.214, indicated TCI state may change due to either MAC CE based TCI activation, or DCI based TCI switching. Therefore, the TCI used for current beam measurement can be dynamically changed.

On the other hand, the RS for new beam is configured in a resource set. In our understanding this resource set can be either SSB set or CSI-RS set. At least for the case of CSI-RS, UE may not be always able to perform CSI-RS based L1 measurement on the corresponding CSI-RS, if the SNR of QCL-source SSB is not kept high enough, e.g. over 2dB. Therefore, theoretically, the new beam where UE will measure may also change dynamically. Note that RAN1 is still FFS on the semi-persistent CSI-RS for this case.

Based on above, it is also proposed to focus on the simplest case in RAN4 first, i.e. RAN4 firstly discuss the RRM requirements for event triggered L1 reporting considering no change to the indicated TCI state, and no change on the RS for the new beam measurements.

Proposal 2  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions:

- during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell

- during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam.

RAN4 may further discuss other cases if time allows.

According to incoming LS from RAN1, at least the case when the periodicity of the current beam RS is the same as that of the new beam RS is supported. Note that R19 MIMO is the extension of R17/R18 MIMO feature, and in MIMO WI discussion, normally only intra-DU deployment is considered. Therefore, gNB would be able to coordinate the RS periodicity between current beam and new beam. From RRM requirement perspective, keeping only Alt-1 is also beneficial for the simplicity of the requirements. For example, if Alt-2 in RAN1 LS is also considered, RAN4 may need to discuss RRM requirements for the case when the periodicity of target RS is 20ms, but the periodicity of source RS is 160ms. In that case UE comparing a latest target beam L1-RSRP results to an out-of-date source beam result would be possible, which would made UE unnecessarily trigger the event. In our view, if RAN4 needs to proceed with Alt-2 also, RAN4 may need to discuss some restrictions to the requirement applicability.

Proposal 3  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions (Alt-1 in RAN1 LS R4-2500009):

- The periodicity of the current beam RS should be the same as that of the new beam RS(s)

RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions:

- the condition which triggers the event is remained fixed during the time UE performs L1 measurements.

- the condition which triggers the event is remained fixed during the time after UE sends the first uplink transmission and before UE is scheduled with the second uplink transmission for the L1 reporting..

RAN4 may further discuss other cases if time allows.

Proposal 2  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions:

- during the time UE performs L1 measurements, no change on the indicated TCI state of current serving cell

- during the time UE performs L1 measurements, no change on the to-be-measured RS of the new beam.

RAN4 may further discuss other cases if time allows.

Proposal 3  As the first step, RAN4 to discuss the event-triggered L1 reporting requirements based on the following assumptions (Alt-1 in RAN1 LS R4-2500009):

- The periodicity of the current beam RS should be the same as that of the new beam RS(s)

RAN4 may further discuss RRM requirements for Alt-2 and the potential restrictions to the requirement applicability if time allows, based on RAN1 further agreements.

- References

[1]  R4-2420096  WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung.  RAN4 #114

[2]  R4-2500009  LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, ZTE.  RAN4 #114