# 3GPP TSG-RAN WG4 Meeting #114  		R4-2501601

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES

Agenda Item:	7.25.2.1

Document for:	Discussion

Introduction

In RAN4 #114, the WF and LS to RAN1 [1], [2] are approved.

Moreover, RAN1 has provided feedback to RAN4 in the reply LS [3].

Based on all above information, we provide our views on RRM requirement impacts for on-demand SSB SCell operation in R19 NES.

Discussion

<On cell search after OD-SSB is activated>

In RAN4 #113, the follow issue was discussed and the agreements are achieved as follows.

Issue 1-1-1: RAN1 LS Agreement: - RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective. - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective. - Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement. Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement - Option 1: The same CSSF as legacy - Option 2: - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. Other option is not precluded.

In legacy L3 measurement requirements in connected mode, CSSF is defined for both measurement outside gaps and measurement within gaps. For measurement within gaps, as cited from Table 9.1.5.2.1-1 as follows, one searcher is assumed for PCC, and one searcher is shared between PSCC, SCC and inter-frequency SSB measurements without gaps, if any. Considering PCC and SCC, an illustration of UE measurement behaviour for outside gap case is illustrated in Figure 1. Note that UE may not always perform cell search between layers in alternative manner, i.e. search on another SCC layer after it collect enough samples on one frequency layer. A round robin manner would be assumed for UE measurement scheduler.

Table 9.1.5.1.2-1: CSSFoutside_gap,i scaling factor for SA mode Scenario; CSSFoutside_gap,i for FR1 PCC; CSSFoutside_gap,i for FR1 SCC; CSSFoutside_gap,i for FR2 PCC; CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required; CSSF outside_gap,i for FR2 SCC where neighbour cell measurement is not required; CSSFoutside_gap,i for inter-frequency MO with no measurement gap; CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap [TBD]. FR1 only CA; 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; N/A; N/A; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS+ NSCC_CCA_RSSI/CO; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only intra band CA; N/A; N/A; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. FR2 only inter band CA; N/A; N/A; 1+NPCC_CSIRS; 2*(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR1 PCell) Note 1; 1+NPCC_CSIRS; 2×( NSCC_SSB +Y+Z+2* NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); N/A; 2x(1+ NSCC_CSIRS_FR2_NCM) Note 3,5; 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM); 2×( NSCC_SSB +Y+Z+2x NSCC_CSIRS -1-NSCC_CSIRS_ FR2_NCM). FR1 +FR2 CA (FR2 PCell) Note 1; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; 1+NPCC_CSIRS; N/A; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS; NSCC_SSB +Y+Z+2x NSCC_CSIRS. Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. Note 3: CSSFoutside_gap,i =1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i =2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS =0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO= 1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO = Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. Note 11: Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0..

Figure 1  Illustration of connected mode CSSF for L3 measurement performed outside gaps. For SCC, UE performs cell search on each SCC in a round-robin manner

For OD-SSB based L3 measurement in case 1, RAN4 has agreed to use SSB periodicity as to define RRM requirements in last meeting. The purpose is to allow gNB to only transmit SSB in a short interval. This would have a lot of impacts to existing measurement scheduler, if cell identification for OD-SSB based SCell is needed. For example, gNB may have no information on which carrier UE is performing cell search, when it indicates UE to perform OD-SSB-based L3 measurements on this SCell. To achieve higher network energy saving gain, UE shall prioritize such cell search on the SSB frequency layer of OD-SSB-based SCell.

In this case UE would need some time to re-schedule its searcher, and prepare RF for this carrier. Such time would be different, depending on different cases. For example, for case 1, RF retuning is anyway needed. However, for case 2, if OD-SSB and AO-SSB(always-on SSB) have the same SSB frequency, and the spatial relation of the two kinds of SSBs are the same, then UE may not need to retune its RF, but still needs to perform cell search if the SCell is unknown, hence time for re-scheduling its searcher is needed. To deal with the worst case without complicating the spec, it is proposed to consider 1 ms for UE processing time.

As discussed above, it is not good for network energy saving if UE performs OD-SSB based cell identification by sharing searcher with legacy SCell measurement. Therefore, if cell identification is needed, it is also proposed to prioritize the OD-SSB based L3 measurements over legacy L3 measurements on PSCCs and other SCCs when applicable.

Regarding the scenario when OD-SSB based cell search is needed, the analysis is as follows:

- If there is always-on SSB (i.e. case 2), UE might not need to perform OD-SSB based cell identification if the SCell is known. However, if the SCell is unknown, OD-SSB based cell identification might also be needed even if there is AO-SSB.

- If there is no always-on SSB (i.e. case 1), and the SCell is unknown based on OD-SSB-based L3 measurement reporting, UE also need to perform OD-SSB based cell identification.

However, if the SCell with OD-SSB is known (for both case 1 and case 2), or if UE obtains rough timing of the SCell based on referenceCell indication, UE may perform OD-SSB based L3 measurement in parallel with other frequency layers, since cell search is not needed. RAN2 is still discussing the related UE reporting after OD-SSB is activated. It is not yet clear whether UE L3 reporting can be triggered the MAC CE based OD-SSB activation. It is somehow beneficial for the network power saving, if UE report the L3 results immediately after the MAC CE based OD-SSB activation, since it would extend the possibility of the known scenario. However, we do not see any issue if RAN4 re-use legacy L3 measurement requirements.

Note that RAN1/2 have not yet precluded the case that UE may need to perform OD-SSB based L3 measurements on more than one SSB frequencies. For example, UE may receive 2 OD-SSB indication MAC CEs in the same DL slot. In this case, RAN4 may need to discuss whether an OD-SSB specific CSSF is needed.

Proposal 1  Additional UE processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay.

Proposal 2  OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement.

- This applies to both Case 1 and Case 2

- This only applies to SCell unknown case.

- RAN4 further discuss CSSF if UE receives OD-SSB activation for multiple SSB frequencies.

Proposal 3  If the SCell is known, or if UE obtains rough timing of the SCell based on referenceCell indication, UE performs OD-SSB based L3 measurement following legacy SCell L3 measurement requirements.

- UE may report L3 results of the SCell based on OD-SSB triggering MAC CE if RAN1/2 agrees, but it is up to UE whether/how to perform L3 measurement in this case. No additional impact to L3 measurement requirements.

<On known condition of OD-SSB>

As discussed above, OD-SSB based L3 measurement procedure shall be impacted by the known condition of the SCell. Therefore, it would be important to clarify the known condition of the SCell. In legacy, known condition of the SCell will only impact the SCell activation requirements.

In last two meetings, the following is agreed.

RAN4 #112b Sub-topic 1-4: OD-SSB based SCell activation Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. RAN4 #113 Sub-topic 1-4: OD-SSB based SCell activation Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

The legacy known cell condition is cited as follows.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

The above known condition can be re-used for the applicability of OD-SSB-based L3 measurement requirements with only slight modification. One important issue is that, in legacy, the SSB reported shall remain detectable, as cited above. In our understanding, this is to clarify that UE will not lost sync during this procedure, and UE may always check its synchronization if needed, since the SSB from gNB is always available. However, for OD-SSB based L3 measurement, the SSB would be even de-activated and not available. UE may only perform OD-SSB based L3 measurement at the next time with stored timing.

Observation 1  In legacy, there are always-on SSBs and UE is allowed to perform rough synchronization based on SSB at any time it lost sync, e.g. due to low SNR. However, for OD-SSB based SCell, after UE reports L3 measurement results, UE may not have SSB to perform sync tracking. Therefore, UE may only perform OD-SSB based sync tracking after OD-SSB is activated.

Therefore, firstly, we propose to revise the known conditions for OD-SSB based L3 measurements as follows.

Proposal 4  For the requirement applicability of OD-SSB based de-activated SCell measurement requirements, SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of OD-SSB activation command:

-	the UE has sent a valid measurement report for the SCell to be measured, and

-	the SSB measured, if available, remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before the reception of OD-SSB activation command:

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	During the period from L3-RSRP reporting to the reception of OD-SSB activation command, the reported SSBs with indexes, if available, remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3

Otherwise, the first SCell in FR2 band is unknown.

Note that in above proposal, there is no differentiation between AO-SSB and OD-SSB. For case #2, a valid measurement report and the SSB measured can be based on either AO-SSB or OD-SSB.

Secondly, for case 1, in [2] the procedure is clarified as follows.

Figure 2  OD-SSB-based L3 measurement procedure for case 1

The related agreements and FFSs are cited as follows.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? FFS Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2? FFS.

As discussed in observation 1, because the SSB could be unavailable during T2, the UE may only start the L3 measurement based on stored sync information. If the stored rough sync info is out-of-date, e.g. due to UE movement, UE would treat the SSB as un-detectable, and would need cell identification procedure. In this case, it is proposed allow UE using some additional OD-SSB samples to check the SCell is known or not.

Note that if UE provided rough timing of the SCell based on referenceCell indication, such additional OD-SSB samples would also be beneficial to extend the applicable scenario. For example, if referenceCell indication is provided, but the actual propagation condition, including Rx timing difference and power gain difference, exceed the range of defined in requirement applicability in TS 38.133 in previous release, UE may still fall back to legacy OD-SSB based SCell activation for the unknown case.

Proposal 5  For case 1, besides the samples for OD-SSB-based L3 measurement, UE is also allowed to use X additional SSB samples so as to check whether the known condition is met or not, e.g. whether the rough sync information is still maintained. Such X samples applies to both known case and unknown case. Prefer X = 1.

<On intra-band CA>

In last meeting the following issue is agreed.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

Firstly, for intra-band co-located CA case, how much NW energy saving gain can be achieved for the OD-SSB case would be unclear, and such scenario significantly complicates the UE RRM requirements for SCell activation. As discussed in the SI phase, most NW energy comes from gNB TxRUs to support large antenna array, and with intra-band collocated active serving cell, such TxRUs shall be already activated. Since the power consumption is already high, periodic SSB transmission, if needed, on an intra-band collocated SCell, will not cause a significant power consumption issue. On the other hand, no matter SSB is transmitted or not for an intra-band collocated SCell, spec impacts would be quite large for both cases, at least in FR1. Therefore, RAN4 can consider deprioritize such scenario, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

Proposal 6  RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

For intra-band non-collocated CA in FR1, as discussed in R18, it shall be up to UE capability whether AGC can be adjusted for two intra-band CCs with non-collocated serving cells separately. For simplicity, it is possible to only consider the case UE have such capability.

Proposal 7  For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  Additional UE processing time is needed after OD-SSB is activated by RRC or MAC CE. Prefer [1] ms for this delay.

Proposal 2  OD-SSB based L3 measurement is prioritized over legacy L3 measurement on PSCC and other SCCs, i.e. RRM requirements on PSCC and other SCCs are not applicable during the time-window when UE performs OD-SSB based L3 measurement.

- This applies to both Case 1 and Case 2

- This only applies to SCell unknown case.

- RAN4 further discuss CSSF if UE receives OD-SSB activation for multiple SSB frequencies.

Proposal 3  If the SCell is known, or if UE obtains rough timing of the SCell based on referenceCell indication, UE performs OD-SSB based L3 measurement following legacy SCell L3 measurement requirements.

- UE may report L3 results of the SCell based on OD-SSB triggering MAC CE if RAN1/2 agrees, but it is up to UE whether/how to perform L3 measurement in this case. No additional impact to L3 measurement requirements.

Observation 1  In legacy, there are always-on SSBs and UE is allowed to perform rough synchronization based on SSB at any time it lost sync, e.g. due to low SNR. However, for OD-SSB based SCell, after UE reports L3 measurement results, UE may not have SSB to perform sync tracking. Therefore, UE may only perform OD-SSB based sync tracking after OD-SSB is activated.

Proposal 4  For the requirement applicability of OD-SSB based de-activated SCell measurement requirements, SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of OD-SSB activation command:

-	the UE has sent a valid measurement report for the SCell to be measured, and

-	the SSB measured, if available, remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before the reception of OD-SSB activation command:

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	During the period from L3-RSRP reporting to the reception of OD-SSB activation command, the reported SSBs with indexes, if available, remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3

Otherwise, the first SCell in FR2 band is unknown.

Proposal 5  For case 1, besides the samples for OD-SSB-based L3 measurement, UE is also allowed to use X additional SSB samples so as to check whether the known condition is met or not, e.g. whether the rough sync information is still maintained. Such X samples applies to both known case and unknown case. Prefer X = 1.

Proposal 6  RAN4 to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1.

Proposal 7  For intra-band non-collocated CA in FR1, only consider UE with similar behaviour as supporting intraBandNR-CA-non-collocated-r18

- References

[1]  R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson.  RAN4 #113

[2]  R4-2420192	Reply LS on timeline for On-demand SSB operation on SCell, Ericsson.  RAN4 #113

[3]  R4-2500010	Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, RAN1.  RAN4 #114