3GPP TSG-RAN WG4 Meeting #114 	R4-2501610

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.1

Source: 	Ericsson

Title:	On LCM requirements for AI/ML

Document for:	Discussion

# Introduction

The following was agreed [4] in RAN4#113 on general issues/post-deployment testing for AI/ML:

## Scope of LCM discussion at the current stage

- The discussion should consider use-cases specific aspects

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

The following was agreed [3] in RAN4#112-bis on general issues/post-deployment testing for AI/ML:

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance. - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

# LCM RRM requirements for AI/ML

In the previous meetings, it was agreed that RAN4 will consider defining RRM requirements for selected LCM procedures, one of which is the fallback procedure to non-AI operation, and that RAN4 has to assess what RRM requirements are needed for each case.

According to the RAN1/RAN2 discussions, the fallback procedure is related to the concepts of applicable/non-applicable functionalities and functionality activation/deactivation mechanisms. RAN2 has agreed on the following definitions:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

- Activated functionalities refers to functionalities already enabled for performing inference.

For a functionality to be applicable at least there should at least one model available within it. RAN2 also agreed that the UE will indicate to the gNB/LMF whether the AI/ML functionality is available/applicable.

Although the granularity of functionality (e.g., whether it is a use case such as beam management, whether it is a sub-use case such as beam management Case 1, or others) is still under discussion in RAN1, “RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.”

## Functionality activation/deactivation for beam management

For the beam management use case, UE-sided model:

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.

- Applicable periodic CSI Report (for beam management) is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported, e.g., by L1 and MAC signalling.

- When a functionality becomes non-applicable the UE reports this to NW, but does not autonomously deactivate the functionality. NW is expected to deactivate active functionality upon receiving the UE’s report.

Upon receiving the deactivation command, the UE may need to go to non-AI/ML operation, and in that case it is important that this is completed as soon as possible. Therefore, a deactivation delay requirement is justified in this case. Functionality activation delay requirement may also potentially be needed for the beam management use case, which can be different for periodic, aperiodic, and semi-persistent reports.

- Proposal 1: Functionality activation delay requirement is considered for the beam management use case, and it can differ for periodic, aperiodic, and semi-persistent reports.

  - FFS whether for the case when going from the non-AI/ML operation mode (when no AI/ML functionality activated) to the AI/ML operation mode the requirement is different from the case when there is already at least one functionality activated prior to the activation.

- Proposal 2: RAN4 will define functionality deactivation delay requirement for the beam management use case.

  - FFS whether for the case when going back to the non-AI/ML operation mode (when no active AI/ML functionality remains) the requirement is different from the case when one of multiple active functionalities is being deactivated and at least AI/ML functionality remains.

## Functionality activation/deactivation for positioning

For the positioning use case, UE-sided model:

- For Case 1, the baseline LCM procedure is:

  - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP RequestCapabilities message.

  - Step 2: UE sends LPP ProvideCapabilities message to LMF with the supported functionalities at the UE side.

  - Step 3: LMF sends the LPP ProvideAssistanceData message (which may contain network side additional condition).

  - Step 4: UE reports the applicable functionality to the LMF by the LPP ProvideCapabilities message in response to Step 3 (“reactive” way).

    - The UE can report the applicable functionality also in “proactive” way by sending an unsolicited LPP ProvideCapabilities message to LMF when the applicability changes.

  - Step 5: The LMF requests the inferred location information using the LPP RequestLocationInformation message.

  - Step 6: UE reports the inferred location using LPP ProvideLocationInformation message.

- As a baseline, if a functionality (e.g., AIML-based positioning method) becomes non-applicable in the UE, when LMF requests UE location estimation, the UE cannot perform the AIML-based positioning and reply with LPP ProvidelocationInformation message with error cause. FFS in RAN2 if other fallback options are considered.

For the positioning use case 1, there is no deactivation procedure yet agreed and the fallback procedure is still under discussion in other groups.

- Proposal 3: FFS: functionality deactivation delay requirement for the positioning use case 1, depending on the agreements in other groups, including the fallback to the non-AI/ML operation mode.

## Data collection for training

For beam management data collection configuration for UE-side model training, the UE can send a request for data collection.  The following methods for network control of the initiation and configuration for data collection were agreed in RAN2:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection.

When the network decides the start/stop of the data collection, it is important that the data collection is not delayed by the UE, which can justify a requirement ensuring the timely start of UE measurements for data collection. One reason for this is that the network may need to configure radio signals for these measurements, and they should not be wasted. Another reason could be that the network is interested in that the UE is measuring during a specific time, for the data collection to serve the purpose properly.

- Proposal 4: A requirement for the maximum delay until the UE actually starts the data collection after receiving the network configuration can be considered for data collection, to ensure that the data collection is not delayed. Alternatively, a UE behaviour can be specified that the UE shall start, e.g., from a first signal occasion available after receiving and decoding the command.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: Functionality activation delay requirement is considered for the beam management use case, and it can differ for periodic, aperiodic, and semi-persistent reports.

  - FFS whether for the case when going from the non-AI/ML operation mode (when no AI/ML functionality activated) to the AI/ML operation mode the requirement is different from the case when there is already at least one functionality activated prior to the activation.

- Proposal 2: RAN4 will define functionality deactivation delay requirement for the beam management use case.

  - FFS whether for the case when going back to the non-AI/ML operation mode (when no active AI/ML functionality remains) the requirement is different from the case when one of multiple active functionalities is being deactivated and at least AI/ML functionality remains.

- Proposal 3: FFS: functionality deactivation delay requirement for the positioning use case 1, depending on the agreements in other groups, including the fallback to the non-AI/ML operation mode.

- Proposal 4: A requirement for the maximum delay until the UE actually starts the data collection after receiving the network configuration can be considered for data collection, to ensure that the data collection is not delayed. Alternatively, a UE behaviour can be specified that the UE shall start, e.g., from a first signal occasion available after receiving and decoding the command.

# References

[1]	RAN4#112 Chairman’s notes, main session, Aug. 2024.

[2] R4-2414323, WF on AI/ML, Qualcomm Inc., Aug. 2024.

[3] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Inc. Oct. 2024.

[4] R4-2420336, WF on WF on AI/ML air interface part 2, Ericsson, Nov. 20224.