3GPP TSG-RAN WG4 Meeting #114 	R4-2501612

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.28.2

Source: 	Ericsson

Title:	On RRM requirements for XR

Document for:	Approval

# Introduction

The RAN4#113 WF is in [6]. The RAN4#112-bis WF is in [5]. The RAN4#112 agreements and WF are in [4]. The RAN4-related objectives of the Rel-19 XR Phase 3 WI are captured in [1].

The following agreements were made in RAN4#113 [6]:

- On scenarios for measurement skipping:

  - Deployment scenarios

    - EN-DC and NE-DC for DCI-based solution

      - Is NOT supported for measurement gap skipping by the WID

  - Types of measurements to be considered

    - The following measurement types are considered for XR measurement skipping

      - Intra-frequency

      - Inter-frequency

      - LTE-inter- RAT (NR-EUTRAN FDD/TDD)

    - The scenario, where the UE is required to measure SRVCC and perform XR measurement skipping, is not supported from RAN4 point of view. The details about how to capture this agreement is FFS.

- General approach for defining requirements for measurements with gaps

  - For the type of measurement gap, as a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other measurement gap types are deprioritized, but the discussion on other measurement gap types will not impact the completion of the WI.

- XR skipping requirements impact

  - Reuse existing measurement accuracy requirements.

  - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping

    - For the delay requirements, FFS whether and how to extend the delay

- Time offset requirement for measurement gap skipping

  - Baseline feature 5 ms

  - Optional capability 3 ms, FFS if any condition is needed.

RAN4 also received RAN1 LS [2], where RAN4 is requested to take the below information from [2] into account and decide whether or not to introduce any UE assistance information related to measurement occasions:

RAN1 Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Below we further discuss the issues related to RAN4 requirements impact for UE operating XR.

# Discussion

## Scenarios

The following was agreed in RAN4#112-bis [5] on the scenarios:

- Frequency ranges

  - Agree on FR1 and FR2-1

- Scenarios

  - Start the discussion for

    - NR-SA

    - Carrier aggregation

Frequency range FR2-2 does not seem to be so relevant for XR and therefore should not be considered in this WI.

- Proposal 1: FR2-2 is not considered in this WI.

## Types of MGs

In [6], it was agreed that Type 1 MGs are used as a baseline, while the relevance for cancellation of other MG types and MG purposes needs further discussion in RAN4. For example, since the configured PRS (positioning) measurements may be intended for emergency positioning, and also because positioning measurements are handled by the core network node (LMF), it is proposed that MGs associated with PRS measurements shall not be cancelled. For MUSIM MGs, the MGRP can be already very long, so applying MG cancellation would even further delay all the measurements. As of concurrent gaps, it is difficult to discuss MG cancellation for UE configured with concurrent MGs, until the single MG pattern framework is settled.

- Proposal 2: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF).

- Proposal 3: MUSIM MGs shall not be cancelled.

- Proposal 4: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 5: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

## On the impact of UL XR operation

The UL XR operation can also be scheduled during the cancelled MGs, therefore the impact of scheduling the UL XR transmissions during the cancelled MGs needs to be discussed due to the potential impact on the RAN4 requirements.

At least the following issues need RAN4 discussion and their relation to the cancelled MGs:

Correct UL timing:

- the UE needs to have the correct UL timing prior staring the UL XR operation, which may result in delayed UL operation and lost resources of the cancelled MGs.

A guard period,

- which is needed for DL-to-UL switching for UL XR operation within the target MG, to avoid UL-to-DL interference.

RF switching:

- needed for the UE to switch from DL to UL and back.

- Proposal 6: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

  - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

  - Issue 2: Guard period for DL-to-UL switching,

  - Issue 3: Impact of RF switching from DL to UL and back.

## On UE assistance information for XR

RAN1 has been discussing UE assistance data to suggest which and/or how RRM measurement occasions (e.g., in measurement gaps) are to be reallocated to XR. According to [2], RAN1 discussed whether the impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information:

- Information about the max number of MGs/SMTC with restrictions that can be skipped within a time period,

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

As correctly also pointed out in [2], from the specification point of view, there is no mandated gNB behaviour in response to any of the UE assistance information. This means that there cannot be any explicit or implicit requirement on gNB behaviour upon receiving such assistance data, including no UE measurement requirements dependent on UE XR UAI. Relying on UAI may also limit network scheduling flexibility. Furthermore, different UEs may have different UE preferences, while the network will have to support all UEs, which will ultimately result in high complexity in the network. This is in addition to that gNB should not be expected to follow the UEs recommendation, anyway.

With the above observations, it is not possible to specify UE requirements, based on any XR UAI, since this otherwise would be an implicit requirement on the network to follow the UAI.

- Proposal 7: RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

On the information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period (mentioned in RAN1 LS): this is more related to RRM measurement performance and therefore can be pre-defined in the specification (TS 38.133). Furthermore, such number may depend on MGRP and DRX cycle length, so it can be more relevant to specify the maximum time between the remaining occasions available for RRM measurements.

- Proposal 8: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133.

On patterns in UAI (mentioned in RAN1 LS): There can be ambiguity in patterns sent by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle.

- Proposal 9: Inform RAN1 about potential ambiguity in patterns or number of measurement occasions indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle.

A draft LS is provided in a companion contribution.

## DRX impact on MG cancellation

In addition to all the above considerations, we also note that a UE can be configured with DRX, which also has own periodicities (which can be longer or shorter than MGRP) and DRX ON duration, not to mention that DRX and XR can be configured by different network nodes (e.g., with NR-DC). When configured with DRX, the UE is not required to perform measurements more than once per DRX cycle (which may make measurements less frequent than with MGRP without DRX), and the UE is not expected to monitor control channel outside DRX ON time period. Hence, not all MG occasions are relevant for cancelling in favour of XR.

- Proposal 10: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

## RRM requirements enhancements for XR

This WI aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions reserved for RRM measurements, while keeping impact to mobility performance limited. This was studied in TR 38.835 B.1.7.

The RRM measurement core and performance requirements are tightly linked to the number of measurement occasions and the minimum number of samples needed to meet the accuracy requirements. If some of the measurement occasions are not available for the measurements, the existing requirements may not be met, at least at the existing side conditions. Hence, to maintain the RRM measurement performance, two approaches could be envisioned:

- Approach 1: The simplest approach would be to limit the XR operation to unused measurement occasions, e.g., in good radio conditions, when RRM measurements could be completed by UE faster. No extension of the RRM measurement period would be needed in this case, but the availability of the unused measurement occasions may not be always guaranteed, e.g., at the cell edge, resulting in no time-frequency resources available for XR for UEs in poor radio conditions, e.g., when the interference is high and/or along cell edges.

- Approach 2: A more practical approach is to compensate for the configured measurement occasions reallocated to the XR operation by extending the RRM measurement period during the XR operation (the compensation means a longer measurement period compared to the UE operation without XR), to ensure the availability of the sufficient number of measurement occasions to complete the RRM measurements at the required accuracy level for any UE, regardless of the experienced radio conditions.

- Approach 3: Combination of Approach 1 and Approach 2, i.e., selectively (upon the need) extend the measurement period only for UEs in poor radio conditions.

With Approach 2 or Approach 3, the extension of the measurement period can be fixed or pre-defined or can be dynamically extendable based on the availability of the measurement occasions during the UE XR operation. The choice of how the extension is defined depends on the RAN1/RAN2 solution and the specified signalling, but the dynamically extendable RRM measurement period may be more suitable for XR due to the dynamic nature of the XR traffic.

- Proposal 11: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation.

- Proposal 12: RAN4 to choose between the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation:

  - pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor),

  - dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

Type 1 MGs are configured by several configuration parameters, including MGL and MGRP and can be per-UE or per-FR. Not all MG configurations are equally important for XR, when MGs are cancelled to reallocate resource to XR.

- Proposal 13: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

At the same time, it is also noted that when some of the measurement occasions are used for the UE XR operation, there will also be impact on RRM measurement samples and how they are combined, since the samples may become non-equally spaced or disrupted, resulting in inaccurate or lost measurements, timing, etc., even if the total measurement period is met. This may result in the RRM measurement accuracy degradation.

- Proposal 14: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: FR2-2 is not considered in this WI.

- Proposal 2: MGs associated with PRS (positioning) measurements shall not be cancelled, e.g., because positioning can be used for emergency and also because these measurements are handled by the core network node (LMF).

- Proposal 3: MUSIM MGs shall not be cancelled.

- Proposal 4: Network controlled pre-configured MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 5: Concurrent MGs are not considered at least until the core requirements impacts are settled for Type 1 MGs.

- Proposal 6: FFS the impact of the following issues for UL XR operation and their relation to the cancelled MGs:

  - Issue 1: Ensuring that the UE has the correct UL timing prior to cancelling MGs,

  - Issue 2: Guard period for DL-to-UL switching,

  - Issue 3: Impact of RF switching from DL to UL and back.

- Proposal 7: RAN4 will not specify any UE requirements dependent on XR UAI (if such is to be defined).

- Proposal 8: The information related to the maximum number of measurement gap occasions allowed for skipping/reallocated to XR over a time period (mentioned in RAN1 LS on UAI) or the maximum time between any consecutive samples can be defined in TS 38.133.

- Proposal 9: Inform RAN1 about potential ambiguity in patterns or number of measurement occasions indicated by UEs and their interpretation by the network, when the UE is configured with DRX, since the measurement occasions and RAN4 measurement period depend on the DRX cycle.

- Proposal 10: DRX configuration needs to be considered in discussions on MG cancellation, e.g., because:

  - UE is not required to perform RRM measurements more frequently than once per DRX cycle, therefore depending on whether or not a UE is configured with DRX, the resource selection for XR may need to be performed differently, e.g., this may impact which measurement gaps are relevant for cancellation in favour of XR and how the measurement gaps occasions are counted when the selection is being decided.

  - UE is not expected to monitor control channel beyond DRX on (and changing this assumption for XR may not be justified),

  - UE can be configured with more than one DRX configurations,

  - Cancelling MGs under some DRX configurations may be less preferred compared to other DRX configurations.

- Proposal 11: To avoid limiting the XR operation to good network radio conditions only and hereby limiting the XR service coverage, RRM measurement requirements are enhanced for UEs under XR operation.

- Proposal 12: RAN4 to choose between the following enhancement methods to the legacy RRM measurement period requirements to account for the UE XR operation:

  - pre-defined fixed extended RRM measurement period for UE under XR operation (e.g., based on a fixed scaling factor),

  - dynamically extendable RRM measurement period (e.g., similar to NR-U), depending on the number of actually cancelled MGs.

- Proposal 13: RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

  - minimum MGL, e.g., MGL≥3 ms,

  - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Proposal 14: Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined.

# References

[1] RP-240791, WID on XR (eXtended Reality) for NR Phase 3, March 2024.

[2] R4-2411007/R1-2405736, LS on UE assistance information, RAN1, Aug./May 2024.

[3] R4-2414910/ R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1 LS, Oct./Aug. 2024.

[4] R4-2414043, WF on RRM requirements for XR_Ph3, Nokia, Aug. 2024.

[5] R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, Oct. 2024.

[6] R4-2420103, WF on RRM requirements for XR_Ph3, Qualcomm, Nov. 2024.