3GPP TSG-RAN WG4 Meeting #114	R4-2501621

Athens, Greece, 17-21 February, 2024

Source:	Ericsson

Title:	Mobile VSAT gradual timing adjustment

Agenda item:	7.11.6

Document for:	Discussion

# 1	Introduction

When assessing UL timing System as part of parameters analysis the following remark was made [1]:

… Moreover, an additional problem refers to the autonomous adjustment rate that can be performed by NTN UEs. The value of Tq_NTN imposes an upper bound on how much the UE is allowed to adjust its timing advance autonomously within 200 ms. For FR1-NTN, the value of 5.5 Ts allows for a total aggregate adjustment rate corresponding to a UE relative speed of 966 km/h, which is enough to cover for most airplane cases. However, the value of 2.5 Ts chosen for FR2-NTN would allow a maximum aggregate adjustment rate corresponding to a speed of 439 km/h, which is not enough to provide autonomous adjustment in flight conditions. …

In this contribution we discuss the Gradual Adjustment parameters Tq_NTN and Tp_NTN for ESIM.

# 2	Discussion

## 2.1 	The history of the 3GPP baseline

The current requirements look like this:

#### … 7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms. Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in Table 7.1C.2.1-1. Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq_NTN; Tp_NTN. FR1-NTN; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; N/A; N/A. FR2-NTN; 60; 2.5*64*Tc; 2.5*64*Tc. 120; 2.5*64*Tc; 2.5*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211 [6]. …

The NTN parameters are the same as for terrestrial NR, i.e, 5.5 and 2.5 Ts. The reason for this is that the deterministic terms related to the orbit are not part of the gradual adjustment procedure and what is left is the UE mobility:

The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN.

The UE mobility was considered to be similar in 3GPP rel-17 NTN and rel-18 NTN as corresponding non-NTN UE. This lead to the fact that NTN gradual timing requirement inherited the corresponding terrestrial requirements.

Observation 1: 	The UE mobility was considered to be similar in 3GPP rel-17 NTN and rel-18 NTN as corresponding non-NTN UE. This lead to the fact that NTN gradual timing requirement inherited the corresponding terrestrial requirements.

… Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq; Tp. 1; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; 5.5*64*Tc; 5.5*64*Tc. 2-1; 60; K*64*Tc; 2.5*64*Tc. 120; K*64*Tc; 2.5*64*Tc. 2-2; 120; 2.5*64*Tc; 2.5*64*Tc. 480; 0.8*64*Tc; 0.8*64*Tc. 960; 0.8*64*Tc; 0.8*64*Tc. NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5.. …

### 2.1.1	Derivation of the terrestrial gradual adjustment requirements

In preparation of release-15 for NR, the Gradual Adjustment requirement was revisited and an example calculation, which correspond to current values in NR and NTN specification can be found in [2]:

… The requirement on maximum autonomous timing adjustment step size Tq is related to the time drift of reference timing (i.e. downlink frame timing of the reference cell). The downlink timing might change due to UE movement and frequency error of operating carrier. Table 1 shows the calculation of time drift with different UE speeds, where ±0.1 PPM frequency error is assumed. Table 1: Calculation of time drift due to UE movement and frequency error Time Drift. Frequency error; ±0.1 PPM. Time drift due to frequency error per 200ms; 20 ns. Supported max speed of UE movement; 30 km/h; 60 km/h; 120 km/h; 250 km/h; 350 km/h. Time drift due to UE movement per 200ms; 5.6ns; 11.1 ns; 22.2 ns; 46.3 ns; 64.8 ns. Total time drift per 200ms; 25.6 ns (0.78 Ts); 31.1 ns (0.95 Ts); 42.2 ns (1.29 Ts); 66.3 ns (2.04 Ts); 84.8 ns (2.61 Ts). Note: The time length of Ts equals to 1/30720000 second (≈ 32.55 ns). As shown in Table 1, if the supported mobile speed in NR is up to 350 km/h, the total time drift due to UE movement and frequency error is 84.8 ns (≈ 2.61 Ts) per 200ms. If the supported mobile speed in NR is up to 250 km/h, the total time drift due to UE movement and frequency error is 66.3 ns (≈ 2.04 Ts) per 200ms. However, for above 6GHz, UE maximum speed may not be very high and more likely be a medium speed, e.g. 30km/h, in which case the total time drift is 25.6 ns (≈ 0.78 Ts) per 200ms. In baseband, the UE transmit signals is quantized in time domain and the sampling rate relies on uplink bandwidth. The UE in typical implementation will use lower sampling rate for smaller bandwidths. The time adjustment step in baseband shall be an integral number of sampling period. Besides, the RF margin shall also be considered and the DigRF error in LTE (i.e. 1.5Ts) could be used as start point. Table 2: Restricts of maximum autonomous timing adjustment step size (Tq) Parameters; Values. Frequency range; Sub 6GHz; Above 6GHz. UL SCS; 15KHz; 30 KHz; 60 KHz; 60 KHz; 120 KHz. Minimum Bandwidth of UL transmission; 5MHz; 5MHz; 10MHz; 50MHz; 50MHz. Sample period of UL transmission; 4Ts; 4Ts; 2Ts; 0.5Ts; 0.5Ts. Supporting Max UE speed; 30 km/h; -; -; -; 0.78 Ts; 0.78 Ts. 250 km/h; 2.04 Ts; 2.04 Ts; 2.04 Ts; -; -. Tq (without DigRF error); 4Ts; 4Ts; 4Ts; 1Ts; 1Ts. Tq (with 1.5Ts DigRF error); 5.5 Ts; 5.5 Ts; 5.5 Ts; 2.5 Ts; 2.5 Ts. Note: The time length of Ts equals to 1/30720000 second (≈ 32.55 ns). For sub-6GHz, if UE supported speed is up to 250km/h, then the total timing drift is 66.3ns (≈ 2.04 Ts) per …

Basically the maximum UE speed, the minimum UE sample rate and the DigRF quantization determined the NR Gradual Adjustment requirements, which where then copied over into NTN.

Please note that the NR range of UE speeds 30-250(350) km/h are used here. Basically the NR requirements and therefore the NTN requirements as well, cover up to and including 250(350) km/h.

Observation 2: 	The maximum UE speed, the minimum UE sample rate and the DigRF quantization determined the NR Gradual Adjustment requirements, which where then copied over into NTN.

Observation 3: 	In NR terrestrial a range of UE speeds from 30 km/h to 250(350) km/h were used. Therefore the NTN requirements as well, cover up to and including 250(350) km/h.

## 2.2 	NTN UE speed requirements

In SA1, TS 22.261, Service requirements for the 5G system, Stage 1 (Release 18) a UE speed of 1000 km/h is stated.

… Table 7.4.2-1: Performance requirements for satellite access Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL) (note 1); Area traffic capacity (UL) (note 1); Overall user density; Activity factor; UE speed; UE type. Pedestrian (note 2); [1] Mbit/s; [100] kbit/s; 1,5 Mbit/s/km2; 150 kbit/s/km2; [100]/km2; [1,5] %; Pedestrian; Handheld. Public safety; [3,5] Mbit/ss; [3,5] Mbit/s; TBD; TBD; TBD; N/A; 100 km/h; Handheld. Vehicular connectivity (note 3); 50 Mbit/s; 25 Mbit/s; TBD; TBD; TBD; 50 %; Up to 250 km/h; Vehicle mounted. Airplanes connectivity (note 4); 360 Mbit/s/ plane; 180 Mbit/s/ plane; TBD; TBD; TBD; N/A; Up to 1000 km/h; Airplane mounted. Stationary; 50 Mbit/s; 25 Mbit/s; TBD; TBD; TBD; N/A; Stationary; Building mounted. Video surveillance (note 4a); [0,5] Mbit/s; [3] Mbit/s; TBD; TBD; TBD; N/A; Up to 120km/h or stationary (note 4b); Vehicle mounted or fixed installation. Narrowband IoT connectivity; [2] kbit/s; [10] kbit/s; 8 kbit/s/km2; 40 kbit/s/km2; [400]/km2; [1] %; [Up to 100 km/h]; IoT. Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 4a: Refer to video surveillance data transmitted (in UL) from a UE on the ground (e.g. picture or video from a camera) using satellite NG-RAN to connect to 5GC, and video surveillance-related configuration or control data sent (in DL) to the UE/device. 0.5 Mbit/s for DL experienced data rate is based on MAVLINK protocol that is widely used for UAV control. 3 Mbit/s for UL experienced data rate is based on the assumed sum from 2.5 Mbit/s for video streaming and 0.5 Mbit/s for data transmission. Note 4b: Up to 120km/h applies to vehicle mounted while stationary applies to fixed installation. Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario.. …

<Editor’s note: NTN TR rel-16 requirement proposals where UE speed has been discussed, but not in normative way >

The table from SA1 TS 22.261 is repeated in TR 38.821 together with some other references to 1000 km/h UE speed.

<Make argument that 1200 km/h has not been actively pursued by any 3GPP party so far in RAN4.>

As stated above Basically the maximum UE speed, the minimum UE sample rate and the DigRF quantization determined the NR Gradual Adjustment requirements, which where then copied over into NTN.

Please note the NR range of UE speeds 30-250(350) km/h used here. Basically the NR requirements and therefore the NTN requirements as well, cover up to and including

Observation 4: 	In SA1 TS 22.261 and in TR 38.821 up to 1000 km/h of UE speed is assumed.

## 2.3 	ESIM requirements for 3GPP rel-19

A timing change of Tq seconds for the UL signal corresponds to Tqc meters traveled distance for UE. The UE speed, vUE during the last TPeriod seconds then becomes:

vUE<TqcTPeriod

If we instantiate with different Tq and set TPeriod = 200 ms we get:

| Tq | TPeriod | vUE |
| --- | --- | --- |
| 5.5 Ts | 200 ms | 970 km/h |
| 2.5 Ts | 200 ms | 440 km/h |

Observation 5: 	A UE speed of 1000 km/h would require Tq = 5.5 Ts.

# 3 	Summary

Observation 1: 	The UE mobility was considered to be similar in 3GPP rel-17 NTN and rel-18 NTN as corresponding non-NTN UE. This lead to the fact that NTN gradual timing requirement inherited the corresponding terrestrial requirements.

Observation 2: 	The maximum UE speed, the minimum UE sample rate and the DigRF quantization determined the NR Gradual Adjustment requirements, which where then copied over into NTN.

Observation 3: 	In NR terrestrial a range of UE speeds from 30 km/h to 250(350) km/h were used. Therefore the NTN requirements as well, cover up to and including 250(350) km/h.

Observation 4: 	In SA1 TS 22.261 and in TR 38.821 up to 1000 km/h of UE speed is assumed.

Observation 5: 	A UE speed of 1000 km/h would require Tq = 5.5 Ts.

# References

[1] 		R4-2418252, Discussion on NR NTN system parameters for Ku band, Nokia

[2]		R4-1713400, Further considerations on UE timing adjustment requirements in NR, Huawei, HiSilicon, RAN4#85