3GPP TSG-RAN WG4 Meeting #114                                                    	R4-2501642

Athens, Greece, February 17 – 21, 2025

Source:	ZTE Corporation, Sanechips

Title:	TP for PRD 01 on delta TIB and RIB tables

Agenda item:			6.2.6

Document for:	Approval

# 1. Introduction

Some guidelines on the usage of delta TIB and RIB have been introduced in [1] for specifying band combinations. It is useful to also capture the related rules into PRD 01 [2].

Proposal 1.  It is suggested to approve the following TP on the usage of delta TIB and RIB for PRD 01.

2. Reference

[1] TR 38.846, Study on simplification of band combination specification for NR and LTE.

[2] R4-2418328, PRD 01 v0.2.0, Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, Ericsson.

# 3. Text Proposal

<Start of Text Proposal>

5.4	Guidelines on delta TIB and RIB

Regarding to the tables for ΔTIB,c and ΔRIB,c, only the configurations having the same component E-UTRA / NR bands can be grouped into one cell (row). For example, in Table 5.4-1 for the ΔTIB,c of the inter-band EN-DC configurations, two rows should be specified separately in the table due to the different component bands. However, for the configurations “DC_3-7-8_n1-n78”, “DC_3-3-7-8_n1-n78”, “DC_3-7-7-8_n1-n78” and “DC_3-3-7-7-8_n1-n78” having the same component bands, they should be in one cell.

Table 5.4-1: Example for ΔTIB,c for Inter-band EN-DC configurations

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 or n8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |

Guideline 5.4-1: It is supposed that only the configurations having the same component E-UTRA / NR bands can be grouped into one cell (row) for the ΔTIB,c and ΔRIB,c tables.

With regard to the values for a band combination in the ΔTIB,c / ΔRIB,c table, considering that a statement of ‘Unless otherwise stated, ΔTIB,c / ΔRIB,c is set to zero’ having been specified in the general part of specification, it is reasonable to remove the combination in the ΔTIB,c / ΔRIB,c table with all component band having the value of ‘-’ (zero). For example in the following ΔRIB,c Table 5.4-2, the CA combinations CA_n1-n3-n5, CA_n1-n3-n18 and CA_n1-n3-n20 do not need to be listed in the table.

Table 5.4-2: Example for ΔRIB,c for Inter-band CA configurations

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n3-n5 | - | - | - |
| CA_n1-n3-n8 | 0.2 | 0.2 | 0.5 |
| CA_n1-n3-n18 | - | - | - |
| CA_n1-n3-n20 | - | - | - |

Guideline 5.4-2: For the band combination with all the component bands having the ΔTIB,c / ΔRIB,c values as ‘-’ (zero), there is no need to be listed in the ΔTIB,c / ΔRIB,c tables.

There are some special delta TIB and RIB values for the band combinations having SUL, SDL, immediately close component band, band combination with overlapping component band, and EN-DC combination with LTE LAA component band in the specifications. However, the denotations for the delta TIB and RIB values for these special constituent band are inconsistent among different band combinations, some of which having the value of “-”, some of which having the value of “N/A”, while some of others having the value of non-zero number. To avoid inconsistencies, the following guideline is applied to the band combinations having special component band.

Guideline 5.4-3: For the component bands which are special bands such as SDL band, SUL band, immediately close band, band combination with overlapping band, and EN-DC combination with LTE LAA band, etc.

-	Non-zero value is not allowed for the special bands in the delta TIB/RIB tables.

-	If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta TIB requirements for that constituted band of the band combination.

-	If downlink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta RIB requirements for that constituted band of the band combination.

<End of Text Proposal>