3GPP TSG-RAN WG4 Meeting #114                                        R4-2501676

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              Discussion on NES common signal adaptation

Agenda item:			7.25.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update

The latest progress for NES common signal/channel adaptation is duplicated as below

<SSB adaptation> Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Working Assumption For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details

Based on the agreements achieved in RAN1, we have the following observations and proposals:

Observation 1: Based on RAN1 agreements on SSB adaptation:

- PCell (connected mode) are not supported for adaptation mechanisms of SSB in time-domain

- Frequency domain adaptation (e.g., inter-frequency) is precluded in RAN1

Proposal 1:

- There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in Pcell

- There is no need to discuss the RRM requirements for the scenarios of frequency domain adaptation

3 Discussions on SSB adaptation

3.1 Measurement period/behaviour at transition

From our understanding, the discussions of measurement period/behavior at transition may occur at L3 measurement (L3 beam level measurement is not considered here) and L1-RSRP measurement based on more SSB samples. We take L3 intra-frequency measurement as example, below

Fig 1. Illustration of L3 intra-frequency measurement before and after transition

- Applicability: SSB adaptation happens during one measurement period (at instance A) for multi-sample measurement

(Some intra-frequency measurements will be changed)

- Solution: Measurement period is the maximum {Period of the first mode before transition, Period of the second mode after transition}. Thereinto, the first mode and the second mode corresponding to SSB/SMTC before and after adaptation, respectively

  - Benefits: Long measurement period to cover sufficient samples. The measurement can be performed with reliable measurement accuracy at transition.

The similar interpretation can be found in multi-sample L1 measurement (i.e, more SSB samples, M=3). While, for L1-RSRP measurement based on one SSB sample, there is no need to discuss this issue 3.1, since uncompleted measurement would not exist during one measurement period. The next measurement opportunity after transition can be waited for and is available for the NES capable UE.

Besides, from our understanding, such max{∙} rule is not applicable to RLM/BFD, since as the current spec. says: the minimum requirement at transition for such case is “no less than the minimum of…”, captured as below

## 8.1 Radio Link Monitoring ### 8.1.4 Minimum requirement at transitions When the UE transitions from a first configuration of RLM resources to a second configuration of RLM resources that is different from the first configuration, for each RLM resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

Proposal 2: The measurement period is the maximum of the period corresponding to the SSB before and after adaptation

- Applicability: SSB adaptation happens during one measurement period for L3 cell-level measurements and L1-RSRP measurement based on more SSB samples (i.e., M=3)

Regarding how to select the multiple measurement samples to guarantee the measurement accuracy, from our understanding, one of the candidate options is that to prioritize adapted resources measurement (i.e., SSB sources after transition). After transition, the UE ready to receive adapted SSB/SMTC resource configuration, it is reasonable to measure adapted measurement resources with the highest priority and perform X samples L1 measurement filter. While, whether to release the measurement results based on legacy resources up to UE implementation, since due to the un-complete measurement, the measurement accuracy based on the SSB sources before transition would be sacrificed.

Proposal 3: To guarantee the measurement accuracy at transition

- UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation

Subsequent to the measurement period above mentioned, the cell identification and the corresponding measurement periods are corresponding to the second mode after transition.

3.2 SSB adaptation – L3 Requirements

3.2.1 Mis-match of adapted SSB transmission and SMTC window

From our understanding, for NES L3 measurement, the mis-match issue between adapted SSB transmission and SMTC window may arise, please find the illustration below. The essential reason is that the MO/SMTC is configured per frequency rather than per cell, so it is difficult for NW to configure one single SMTC window to cover all SSBs (including adapted SSBs and legacy SSBs) without causing UE extra power consumption, especially for the legacy UEs. The applicable cases for adaptation of SSB in time domain, we assume here is: Serving cell with both legacy and Rel-19 NES-capable UE.

Fig. 2. The mis-match issue of adapted SSB and SMTC window

If such issue happens, the L3 measurement behavior (s) of NES capable UE cannot be defined accurately by following the current RAN1, RAN2 and RAN4 spec.. Considering the specification changes, majority suggest to consider the possibility of optimizing/adjusting the SMTC configuration for NES instead of using adapted SSB to perform L3 measurement directly.

From our understanding, the main ongoing solutions from RAN2 perspective are at least (combining with OD-SSB discussions) following:

Fig. 3. The Alts. to optimize SMTC configuration for NES discussed in RAN2

Observation 2:  Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs

Observation 3: Based on RAN2 discussions, the methods to optimize SMTC configurations for NES can be:

- NES dedicated Measurement Object (e.g., MeasObjectNES) with/without new MeasId

- Separate/new optional IE for NES configured within the existing MeasObjectNR

- Separate/New serving cell configurarions for NES configured within the existing ServingCellConfig/ServingCellConfigCommon

- New optional IE configured in SCellConfig to support Scell supporting NES indication

3.2.2 RRM requirement impact

No matter to perform L3 measurement/cell identification by using SMTC configurations for NES or adapted SSB pattern, the L3 requirements involving such novel changes shall be enhanced, the requirements include 9.2 NR intra-frequency measurements (including cell identification, measurement);9.3 NR inter-frequency measurements (including cell identification, measurement), but how to enhance the corresponding requirement can wait RAN2’s concrete conclusions. However, based on RAN1 agreements and discussion preference, among such requirements, we prefer to deprioritize deactivated SCell measurement/detection enhancement for SSB adaptation, since such corresponding periods are related to measCycleSCell, irrelevant to SSB/SMTC periodicity for adaptation.

Proposal 4: RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurment, and inter-frequency SCell identification and measurment after receiving RAN2 reply LS on SMTC configuration mechanism (s) for NES

Proposal 5: For adaptation of SSB transmissions, suggest RAN4 deprioritize the discussion on RRM requirement impact on deactivated SCell measurement/detection

- If deactivated SCell measurement/detection is needed based on SSB sources for adaptation, RAN4 to discuss whether the principle defined for OD-SSB can be reused

3.3 SSB adaptation – L1 Requirements

3.3.1 Measurement priority with relation to DRX

The following agreement was achieved in last meeting regarding L1 requirements subsequent to “transition period”, copied below

Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the adapted SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues.

And if we take a look at the measurement period for FR1/FR2, below (taking L1-RSRP measurement as example),

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

the SSB based L1-RSRP measurement is based on M SSB samples (M=1/M=3), sweeping factor N, SSB periodicity, DRX cycle length, configured periodicity for reporting, measurement gap related sharing factor P, relaxation of additional 1.5, and the computation rule is max(A, B*max (TDRX,TSSB)) for DRX cycle<=320 ms.

However, we have some concerns on DRX cycle <=320 ms configuration case. Since from our understanding, once the SSB adaptation activation has been triggered, one of the most important tasks for the UE is to finish the L1 measurement and to complete Tx beam selection as fast as possible (i.e., with minimum delay), in this sense, if we to consider L1-RSRP measurement under DRX mode, the measurement period will be always limited by DRX cycle, even Tadapted SSB<< TDRX, we cannot see the benefits of SSB adaptation with dense periodicity. From this, we proposal

- Proposal 6: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode

3.4 SSB adaptation – SCell activation

3.4.1 SCell activation based on different location UE receives SSB adaptation indication

In practice, for SSB adaptation SCell operation, the UE can receive the adaptation indication at #1 Before SCell activation command reception #2 Along with MAC-CE activation command; #3 During SCell activation procedure but after MAC-CE activation command reception; #4 After/When SCell activation is completed and SCell is activated. The similar procedure as on-demand SSB based SCell activation. From our understanding, there is no need to consider # 3 SSB time adaptation indication case. Since I. the SSB adaptation activation/deactivation is opportunistic; II. When UE can complete AGC+SSB detection is left to UE implementation, we have concerns on whether additional complexities will be introduced for requirements definition if SSB time adaptation happens during the SCell activation procedure. Even one of the worst cases, e.g., FR2 intra-band CA SCell activation, if SSBs are not in the same half frame (half frame indication after SSB adaptation is discussed in RAN1), UE will fail the fine time tracking, and depending on UE implementation it may start detecting the adapted SSB again (in this sense, the SCell activation timeline shall be extended), or it may just abort the activation. From this, we have

Proposal 7: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception

Fig. 4. SCell activation based on different location UE receives SSB adaptation indication (FR2)

For #2, UE receive SCell activation command and SSB adaptation indication command at the same time, from our understanding:

- A separate MAC CE to activate SSB adaptation along with SCell activation

- For FR2 unknown SCell activation, if adaptation based on one SSB configuration, and SSB periodicity adaptation is considered, the SCell activation delay can be shorten by reducing the AGC settling time, cell detection time, and L1 measurement time based on dense SSB periodicity for adaptation instead of SMTC periodicity

- Since there is no active FR2 serving cell (PCell/PSCell) supporting SSB adaptation, the assumption of using SSB resources for adaptation to measure the SCell is acceptable, that is TSMTC_MAX is not required for AGC in unknown SCell activation, which is in terms of all the active serving cells in the band

Proposal 8: For FR2 unknown SCell activation, if there is no active FR2 serving cell supporting SSB adaptation in the band, it can be assumed that UE to use SSB resources for adaptation to active the SCell (w.r.t. obtain AGC) instead of SMTC

Proposal 9: When UE receive SCell activation command and SSB adaptation indication command at the same time, for FR2 unknown SCell activation, the activation delay can be shorten by reducing the AGC settling time, cell detection time, and L1 measurement time based on dense SSB periodicity for adaptation

For #1, since there is a possibility that UE sent a valid L3-RSRP measurement result with SSB index based on SMTC/SSB resource for adaptation within [X] s before receiving the SCell activation command, the unknown SCell can become a known cell, and the SCell activation delay can be shorten.

Proposal 10: If UE receive SSB adaptation indication command before receiving SCell activation command, RAN4 to discuss whether to/how to define the known cell condition for FR1/FR2 for NES

At same time, considering the commonality between on-demand SSB and adaptation SSB, we think the MAC CE based SCell activation case can also be considered as the starting point for SSB adaptation SCell operation. Actually, if DCI based case is considered, we have some concerns on whether/how to handle the case when UE miss one of DCI, which is used to indicate SSB adaptation.

However, since the following issues were discussed but without any clear conclusions in RAN1: How NW to indicate SSB adaptation, by RRC/MAC-CE/DCI; Unified/Common MAC-CE framework for on-demand SSB and adaptation SSB

From this, we have

Proposal 11: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

4 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on NES adaptation. The following observations and proposals are obtained:

Observation 1: Based on RAN1 agreements on SSB adaptation:

- PCell (connected mode) are not supported for adaptation mechanisms of SSB in time-domain

- Frequency domain adaptation (e.g., inter-frequency) is precluded in RAN1

Observation 2:  Due to the mis-match of adapted SSB and SMTC window, if the adapted SSB triggered by MAC-CE/DCI does not fall into the configured SMTC window, the UE cannot perform L3 measurement based on the adapted SSBs

Observation 3: Based on RAN2 discussions, the methods to optimize SMTC configurations for NES can be:

- NES dedicated Measurement Object (e.g., MeasObjectNES) with/without new MeasId

- Separate/new optional IE for NES configured within the existing MeasObjectNR

- Separate/New serving cell configurarions for NES configured within the existing ServingCellConfig/ServingCellConfigCommon

- New optional IE configured in SCellConfig to support Scell supporting NES indication

Proposal 1:

- There is no need to discuss the RRM requirements for the scenarios of Rel-19 NES-capable UEs in Pcell

- There is no need to discuss the RRM requirements for the scenarios of frequency domain adaptation

Proposal 2: The measurement period is the maximum of the period corresponding to the SSB before and after adaptation

- Applicability: SSB adaptation happens during one measurement period for L3 cell-level measurements and L1-RSRP measurement based on more SSB samples (i.e., M=3)

Proposal 3: To guarantee the measurement accuracy at transition

- UE can to perform measurement based on the SSB/SMTC resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation

Proposal 4: RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurment, and inter-frequency SCell identification and measurment after receiving RAN2 reply LS on SMTC configuration mechanism (s) for NES

Proposal 5: For adaptation of SSB transmissions, suggest RAN4 deprioritize the discussion on RRM requirement impact on deactivated SCell measurement/detection

- If deactivated SCell measurement/detection is needed based on SSB sources for adaptation, RAN4 to discuss whether the principle defined for OD-SSB can be reused

- Proposal 6: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode

Proposal 7: No need to define RRM requirements for the case: UE receive the adaptation indication during SCell activation procedure but after MAC-CE activation command reception

Proposal 8: For FR2 unknown SCell activation, if there is no active FR2 serving cell supporting SSB adaptation in the band, it can be assumed that UE to use SSB resources for adaptation to active the SCell (w.r.t. obtain AGC) instead of SMTC

Proposal 9: When UE receive SCell activation command and SSB adaptation indication command at the same time, for FR2 unknown SCell activation, the activation delay can be shorten by reducing the AGC settling time, cell detection time, and L1 measurement time based on dense SSB periodicity for adaptation

Proposal 10: If UE receive SSB adaptation indication command before receiving SCell activation command, RAN4 to discuss whether to/how to define the known cell condition for FR1/FR2 for NES

Proposal 11: For Rel-19 NES-capable UE’s SCell

- RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation

5 References

[1] R4-2420099 “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei