3GPP TSG-RAN WG4 Meeting #114                                        R4-2501677

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              Discussion on L3 measurement enhancement by optimizing CSSF

Agenda item:			7.17.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had extensively discussions on L3 measurement enhancement by optimizing CSSF. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Tentative solution 1: measure one serving CC per band

The reason why we have some concerns on the existing WF and prefer NW control solution is that:

- We think compare to neighbour cell measurement, serving cell measurement is more important, and NW to indicate which ServingCellMO (s) to be measured based on its demand/data traffic/preference, and what UE can do is to follow NW decision and measure the indicated MO. And we slightly think, there are no strong benefis if NW only indicate to enable/disable CSSF enhancement (i.e., the feature is supported/enabled) although signalling overhead is smaller (straightforward and maybe just 0/1 indication and may be not involved in MO configuration), since UE cannot know the intention of NW to configure those MOs without any indication. From this, NW-UE mismatch issue may arise, that is what UE measured MO/carrier depending on its implementation is not NW expection. Of course, based on the offline discussion, we agree that the mismatch issue may highly happen when UE only has SCells in the FR2 band.

Obervation 1: When the UE only has SCells in the FR2 band, if NW indicates to enable this CSSF enhancement, NW-UE mismatch issue may arise, that is what UE measured MO/Carrier depending on its implementation is not NW expection, less benefits

To solve this issue, based on the offline discussions, we have two directions:

- Direction 1: Keep discussing and trying to find an enhanced way to Issue 2-4-1: measure one serving CC per band,

  - Direction 1-1: Enable CSSF enhancement feature indication + UE to measure one serving CC per band reusing the legacy principle for neighbour cell measurement (i.e., the solution captured in the current WF)

  - Direction 1-2: Compromised solution, if it is acceptable:

    - The NW indication is not limited to indicate enable/disable CSSF enhancement feature, but to select/indicate one serving SCC to measure if multiple serving SCCs are in the same FR2 band and when such FR2 band has no spCell. But for other cases, the pre-defined rules (i.e., legacy principle for meighbour cell measurment can be reused, that is:

      - Measure PCC if PCC in band or

      - Measure PSCC if PSCC in band,

rather than all serving CCs

    - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements

- Direction 2:  Remove objective solution 1 and use legacy NW implemention, i.e, following the existing procedure to implement one serving CC per band measurement by considering servingCellMO, sCellToAddModList and sCellToReleaseList configurations

If NW-UE mismatch issue cannot be solved, we suggest to consider the following solution to measure one serving CC per band

Proposal 1 (the compromised solution on top of the tentative solution 1):

- Based on the new NW indication, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured

  - The NW indication is not limited to indicate enable/disable CSSF enhancement feature, but to indicate one serving SCC to measure if multiple serving SCCs are in the same FR2 band and when such FR2 band has no spCell.

  - For other cases, reuse the legacy principle for neighbour cell measurement:

    - Measure PCC if PCC in band or

    - Measure PSCC if PSCC in band,

  - FFS how to design the NW indication

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements

2.2 Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

From our understanding, for some BC defined in the 38.101-1/2/3, changing the searcher occupancy ratio of measurement among the measured CCs is a promising way, and the NW to indicate the ratio between PCC, PSCC and SCC (e.g., x/2% is from PCC, x/2%from SCCs) and even indicate the prioritized carrier

Proposal 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions,

- NW to indicate the ratio between PCC, PSCC and SCC

  - Tentative solution 3: 3 searchers for CSSF

The 3 seachers for CSSF scenario we can image is that FR1+FR2, per FR-gap based. To be specific, assume that: Two serving CCs, CC1 and CC2 in FR1; One serving CC3 in FR2; Two intra-frequency layers to be monitored without gap on CC1 and CC2, respectively, and FR2 inter-frequency layer f3 to be monitored with gap at CC3. While, the SMTC occasions of the CCs are collided in time domain. Since all measurements in FR2 are gap based, if the UE has less than 3 searchers, the corresponding measurement delay will be increased due to the limitation number of searcher and there is no parallel mesurement in FR2. However, we have concerns on such UE implementation.

Proposal 3: One possible scenario of using 3 searchers for CSSF reduction is FR1 + FR2 scenario (CA and NR-DC) only

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 CSSF enhancement. The following observations and proposals are obtained:

Obervation 1: When the UE only has SCells in the FR2 band, if NW indicates to enable this CSSF enhancement, NW-UE mismatch issue may arise, that is what UE measured MO/Carrier depending on its implementation is not NW expection, less benefits

Proposal 1 (the compromised solution on top of the tentative solution 1):

- Based on the new NW indication, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured

  - The NW indication is not limited to indicate enable/disable CSSF enhancement feature, but to indicate one serving SCC to measure if multiple serving SCCs are in the same FR2 band and when such FR2 band has no spCell.

  - For other cases, reuse the legacy principle for neighbour cell measurement:

    - Measure PCC if PCC in band or

    - Measure PSCC if PSCC in band,

  - FFS how to design the NW indication

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements

Proposal 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions,

- NW to indicate the ratio between PCC, PSCC and SCC

Proposal 3: One possible scenario of using 3 searchers for CSSF reduction is FR1 + FR2 scenario (CA and NR-DC) only

4 References

[1] R4-2420093 “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple