3GPP TSG-RAN WG4 Meeting #114                                        R4-2501679

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:              Discussion on T_uncertainty for eEMR based fast SCell activation

Agenda item:			7.17.3

Document for:	Discussion

2 Discussion

In FR2, the activation delay consists time needed for the following step especially:

- Uncertainties:

  - Tuncertainty_MAC includes the waiting time for PDCCH TCI info and can with the waiting time of MAC-command for SP CSI-RS when semi-persistent CSI-RS is configured for CSI report, below:

    - Tuncertainty_MAC =0 if UE receives the SCell activation command and TCI state activation commands at the same time

    - Tuncertainty_MAC =0 and Tuncertainty_SP =0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time,

  - Tuncertainty_RRC

  - Tuncertainty_SP

For known and unknown case, the timeline can be found below

Fig. 1 Illustration of known case for FR2

Fig. 2 Illustration of unknown case for FR2,

To be specific, the time period for the three uncertainies are

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case. Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case. Tuncertainty_SP is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case.

However, for our eEMR specific unknown case, the applicability on above description of time period of the three uncertainties are not satisfied any more, since based on our agreed timeline, we do not have first valid L1-RSRP reporting but consider AGC.

- 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. - TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

From this, the updates on Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP should be considered in order to make the uncertainties applicable to our eEMR based fast SCell activation unknown case, but the corresponding discussions are missing in the draft CR pre-meeting sync-up email proposed.

Proposal 1: The existing time period definition of Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP are not applicable to eEMR based fast SCell activation unknown case, in which the first valid L1-RSRP reporting is not considered, while such updates are not captured in the draft CR.

Proposal 2: RAN4 to discuss how to define the time period of : Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP for eEMR based fast SCell activation unknown case without considering first valid L1-RSRP reporting

- FFS: AGC

- FFS: eEMR valid L3-RSRP measurement reporting

- Other options are not precluded

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on Tuncertainty for eEMR based fast SCell activation. The following observations and proposals are obtained:

Proposal 1: The existing time period definition of Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP are not applicable to eEMR based fast SCell activation unknown case, in which the first valid L1-RSRP reporting is not considered, while such updates are not captured in the draft CR.

Proposal 2: RAN4 to discuss how to define the time period of : Tuncertainty_MAC/ Tuncertainty_RRC/ Tuncertainty_SP for eEMR based fast SCell activation unknown case without considering first valid L1-RSRP reporting

- FFS: AGC

- FFS: eEMR valid L3-RSRP measurement reporting

- Other options are not precluded