3GPP TSG-RAN4 Meeting #114	R4-2501690

# Athens, Greece, 17th – 21st February, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2693 | rev |  | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_SL_enh-Core) CR on A-MPR for NS_33 (R18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A-MPR, A-SEM, and A-SE for NS_33 need to be updated based on ETSI EN 302 571. The regulation is as follows. A-MPR, A-SEM, and A-SE for NS_33 should be updated to align with updated regulation in Europe. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PC3 & PC2 A-MPR for NS_33 is updated for carrier frequency of 5920MHz. - Unit for RBstart and Lcrb is changed from RB to MHz to cover all SCSs for PSSCH/PSCCH. - A-SEM, and A-SE for NS_33 are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR, A-SEM, and A-SE for NS_33 requirements are not aligned with updated regulation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3.2, 6.5E.2.3, 6.5E.3.3, 6.5E.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

6.2E.3.2	A-MPR for V2X UE by NS_33

When NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the additional maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector* A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2E.3.2-1, 6.2E.3.2-2 is allowed when network signalling value is provided. A-MPRBase is the default A-MPR value when no Gpost connector is declared. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I in [11]. The A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector.

For the contiguous PSSCH and PSCCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-1 and 6.2E.3.2-2 for power class 3. And A-MPR requirements specified in Table 6.2E.3.2-2a and 6.2E.3.2-2b for power class 2 are allowed for NR V2X UE.

Table 6.2E.3.2-1: PC3 A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz, and 5920MHz)

| Carrier frequency [MHz] | Region |  | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | LCRB* 12*SCS [MHz] | RBstart*12*SCS [MHz] | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 1.8 and ≤ 2.7 | 0 | ≤ 24.0 |  |  |
|  |  | ≥ 0.18 and ≤ 0.54 | ≤19.0 |  |  |
|  |  | ≥ 4.68 | ≤ 6.0 |  |  |
|  | ≥ 1.8 and ≤ 3.6 | ≥ 2.16 and ≤ 2.52 | ≤11.0 |  |  |
|  |  | ≥ 2.7 and ≤ 3.42 | ≤ 9.5 |  |  |
|  |  | ≥ 3.6 and ≤ 4.5 | ≤ 8.0 |  |  |
|  | > 2.7 and < 4.5 | ≥ 4.5 | ≤ 8.0 |  |  |
|  | ≥ 1.8 and < 7.2 | ≥ 0.72 and ≤1.26 | ≤ 16.0 |  |  |
|  |  | ≥ 1.44 and ≤ 1.98 | ≤ 13.5 |  |  |
|  | ≥ 3.6 and < 7.2 | ≥ 0 and ≤ 0.54 | ≤ 22.0 |  |  |
|  | ≥ 4.32 and < 7.2 | ≥ 2.88 and ≤ 3.78 | ≤ 9.5 |  |  |
|  |  | ≥ 3.96 and ≤ 4.86 | ≤ 8.0 |  |  |
|  | ≥ 4.32 and ≤ 7.2 | ≥ 2.16 and ≤ 2.7 | ≤ 12.0 |  |  |
|  | 7.2 and 8.1 | ≥ 0 and ≤ 0.18 | ≤ 19.0 |  |  |
|  |  | ≥ 0.36 and ≤ 0.9 | ≤ 16.0 |  |  |
|  |  | ≥ 1.08 and ≤ 1.98 | ≤ 13.5 |  |  |
|  | > 8.1 | ≥ 0 | ≤ 16.0 |  |  |
| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBend*12*SCS [MHz] | QPSK/16QAM | 64QAM | 256QAM |
| 5920 | ≥ 1.8 and ≤ 2.7 | 9.36 | ≤ 24.0 |  |  |
|  |  | ≥ 8.82 and ≤ 9.18 | ≤19.0 |  |  |
|  |  | ≤ 4.68 | ≤ 6.0 |  |  |
|  | ≥ 1.8 and ≤ 3.6 | ≥ 6.84 and ≤ 7.2 | ≤11.0 |  |  |
|  |  | ≥ 5.94 and ≤ 6.66 | ≤ 9.5 |  |  |
|  |  | ≥ 4.86 and ≤ 5.76 | ≤ 8.0 |  |  |
|  | > 2.7 and < 4.5 | ≤ 4.86 | ≤ 8.0 |  |  |
|  | ≥ 1.8 and < 7.2 | ≥ 8.1 and ≤8.64 | ≤ 16.0 |  |  |
|  |  | ≥ 7.38 and ≤ 7.92 | ≤ 13.5 |  |  |
|  | ≥ 3.6 and < 7.2 | ≥ 8.82 and ≤ 9.36 | ≤ 22.0 |  |  |
|  | ≥ 4.32 and < 7.2 | ≥ 5.58 and ≤ 6.48 | ≤ 9.5 |  |  |
|  |  | ≥ 4.5 and ≤ 5.4 | ≤ 8.0 |  |  |
|  | ≥ 4.32 and ≤ 7.2 | ≥ 6.66 and ≤ 7.2 | ≤ 12.0 |  |  |
|  | 7.2 and 8.1 | ≥ 9.18 and ≤ 9.36 | ≤ 19.0 |  |  |
|  |  | ≥ 8.46 and ≤ 9.0 | ≤ 16.0 |  |  |
|  |  | ≥ 7.38 and ≤ 8.28 | ≤ 13.5 |  |  |
|  | > 8.1 | ≤ 9.36 | ≤ 16.0 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.2-2: PC3 A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.2 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

Table 6.2E.3.2-2a: PC2 A-MPR for PSCCH/PSSCH by NS_33 (at Fc=5860MHz, and 5920MHz)

| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBstart*12*SCS [MHz] | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 1.8 and ≤ 2.7 | ≥ 0 and ≤ 0.18 | ≤ 24 |  |  |
|  |  | ≥ 0.36 and ≤ 0.54 | ≤ 22 |  |  |
|  |  | 0.72 | ≤ 20 |  |  |
|  | ≥ 1.8 and ≤ 4.5 | ≥ 0.9 and ≤ 1.26 | ≤ 17.5 |  |  |
|  | ≥ 1.8 and ≤ 5.4 | 1.8 | ≤ 16 |  |  |
|  | ≥ 1.8 | ≥ 1.44 and ≤ 1.62 | ≤ 16 |  |  |
|  |  | ≥ 1.98 and ≤ 2.52 | ≤ 14.5 |  |  |
|  |  | ≥ 2.7 and ≤ 3.42 | ≤ 13 |  |  |
|  |  | ≥ 3.6 and ≤ 4.32 | ≤ 11.5 |  |  |
|  |  | ≥ 4.5 and ≤ 5.22 | ≤ 10 |  |  |
|  |  | ≥ 5.4 | ≤ 8.5 |  |  |
|  | ≥ 3.6 and ≤ 4.32 | 0.18 | ≤ 22 |  |  |
|  | ≥ 3.6 and ≤ 5,4 | 0 | ≤ 22 |  |  |
|  |  | ≥ 0.36 and ≤ 0.54 | ≤ 20 |  |  |
|  |  | 0.72 | ≤ 17.5 |  |  |
|  | ≥ 25 and ≤ 40 | 0.18 | ≤ 20 |  |  |
|  | ≥ 5.4 | ≥ 0.9 and ≤ 1.26 | ≤ 16 |  |  |
|  | ≥ 6.48 | 0 | ≤ 20 |  |  |
|  |  | ≥ 0.36 and ≤ 0.72 | ≤ 17.5 |  |  |
|  |  | 1.8 | ≤ 14.5 |  |  |
|  | ≥ 8.1 | 0.18 | ≤ 17.5 |  |  |
| Carrier frequency [MHz] | LCRB* 12*SCS [MHz] | RBend*12*SCS [MHz] | A-MPR(dB) |  |  |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5920 | ≥ 1.8 and ≤ 2.7 | ≥ 9.18 and ≤ 9.36 | ≤ 24 |  |  |
|  |  | ≥ 8.82 and ≤ 9.0 | ≤ 22 |  |  |
|  |  | 8.64 | ≤ 20 |  |  |
|  | ≥ 1.8 and ≤ 4.5 | ≥ 8.1 and ≤ 8.46 | ≤ 17.5 |  |  |
|  | ≥ 1.8 and ≤ 5.4 | 7.56 | ≤ 16 |  |  |
|  | ≥ 1.8 | ≥ 7.74 and ≤ 7.92 | ≤ 16 |  |  |
|  |  | ≥ 6.84 and ≤ 7.38 | ≤ 14.5 |  |  |
|  |  | ≥ 5.94 and ≤ 6.66 | ≤ 13 |  |  |
|  |  | ≥ 5.04 and ≤ 5.76 | ≤ 11.5 |  |  |
|  |  | ≥ 4.14 and ≤ 4.86 | ≤ 10 |  |  |
|  |  | ≥ 3.96 | ≤ 8.5 |  |  |
|  | ≥ 3.6 and ≤ 4.32 | 9.18 | ≤ 22 |  |  |
|  | ≥ 3.6 and ≤ 5,4 | 9.36 | ≤ 22 |  |  |
|  |  | ≥ 8.82 and ≤ 9.0 | ≤ 20 |  |  |
|  |  | 8.64 | ≤ 17.5 |  |  |
|  | ≥ 25 and ≤ 40 | 9.18 | ≤ 20 |  |  |
|  | ≥ 5.4 | ≥ 8.1 and ≤ 8.46 | ≤ 16 |  |  |
|  | ≥ 6.48 | 9.36 | ≤ 20 |  |  |
|  |  | ≥ 8.64 and ≤ 9.0 | ≤ 17.5 |  |  |
|  |  | 7.56 | ≤ 14.5 |  |  |
|  | ≥ 8.1 | 9.18 | ≤ 17.5 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 6.2E.3.2-2b: PC2 A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 5870,5910 | outer | ≤ 8.5 |  |  | ≤ 8.5 |
|  | inner | ≤ 6.0 |  |  |  |
| 5880,5890,5900 | outer | ≤ 6.0 |  |  | ≤ 6.5 |
|  | inner | ≤ 3.5 |  | ≤ 4.5 |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.1 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

For the simultaneous PSFCH transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-3 for power class 3 and in Table 6.2E.3.2-3a for power class 2.

Table 6.2E.3.2-3: PC3 A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860, 5920 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

Table 6.2E.3.2-3a: PC2 A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.2 | 0.2≤ NGap / NRB < 0.4 | 0.4≤ NGap / NRB ≤ 1 |  |
| 10 | 5860, 5920 | NRB =1 | 25.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 16.5 | 12 | 20 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

For the S-SSB transmission when NS_33 is indicated by the network or pre-configured radio parameters for NR V2X UE, the NR UE allow the follow A-MPR requirements specified in Table 6.2E.3.2-4 for power class 3 and in Table 6.2E.3.2-5 for power class 2.

Table 6.2E.3.2-4: PC3 A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |
| Carrier Frequency (MHz) | RBend * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| 5920 | ≥ 8.36 | ≤ 25 | 0.6 |
|  | ≥ 7.36 and < 8.36 | ≤ 19 |  |
|  | ≥ 6.12 and < 7.36 | ≤ 12 |  |
|  | ≥ 5.76 and < 6.12 | ≤ 10 |  |
|  | < 5.76 | ≤ 9 |  |

Table 6.2E.3.2-5: PC2 A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |
| Carrier Frequency (MHz) | RBend * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| 5920 | ≥ 8.36 | ≤ 25 | 0.6 |
|  | ≥ 7.36 and < 8.36 | ≤ 19 |  |
|  | ≥ 6.12 and < 7.36 | ≤ 12 |  |
|  | ≥ 5.76 and < 6.12 | ≤ 10 |  |
|  | < 5.76 | ≤ 9 |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

6.5E.2.3	Additional Spectrum emission mask

6.5E.2.3.1	Requirements for network signalling value "NS_33"

The additional spectrum mask in Table 6.5E.2.3.1-1 applies for NR V2X UE within 5855 MHz to 5925 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" is indicated in the cell or pre-configured radio parameters, the power of any V2X UE emission shall not exceed the levels specified in Table 6.5E.2.3.1-1.

Table 6.5E.2.3.1-1: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for NR V2X overrides any other requirements in frequency range 5855-5925MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11].

< END OF CHANGE #2 >

< START OF CHANGE #3 >

6.5E.3	Spurious emissions for V2X

6.5E.3.1	General spurious emissions

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the general spurious emission requirements in clause 6.5.3.1 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

6.5E.3.1A	Spurious emissions for sidelink CA

For SL intra-band contiguous CA, the general NR CA general SE for CA Bandwidth specified in subclause 6.5A.3.1 shall be applied to the aggregated channel bandwidth with SL CA bandwidth class B.

6.5E.3.2	Spurious emissions for UE co-existence

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the requirements in clause 6.5.3.2 shall apply for NR V2X sidelink transmission.

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with the SL MIMO configurations described in clause 6.2D.1.

6.5E.3.2A	Spurious emissions band UE co-existence for sidelink CA

For SL intra-band contiguous CA, the protection operating band lists for n47 transmission is defined in Table 6.5.3.2-1 which shall be applied to NR SL intra-band contiguous CA UE.

6.5E.3.3	Spurious emissions for UE co-existence for V2X concurrent operation

This clause specifies the additional requirements for inter-band concurrent V2X operation with the single CC uplink assigned to two NR bands for coexistence with protected bands for the specified simultaneous transmission of the inter-band concurrent V2X configurations in Table 6.5E.3.3-1. The intersection of the requirements for the individual bands specified in clause 6.5.3.2 shall also apply for the specified simultaneous transmission of the inter-band concurrent V2X. Intersection of a requirement means that both UL or sidelink transmission constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply.

For the inter-band concurrent NR V2X operation, the UE-coexistence requirements in Table 6.5E.3.3-1 apply for the corresponding inter-band concurrent operation with transmission assigned to both uplink in licensed band and sidelink in Band n47.

NOTE:	For inter-band concurrent V2X operation with uplink assigned to NR band and slidelink transmission assigned to NR V2X operating bands, the requirements in Table 6.5E.3.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5E.3.3-1 and in clause 6.5.3.2 would be considered to be verified by the measurements verifying the one uplink inter-band concurrent UE to UE co-existence requirements.

Table 6.5E.3.3-1: Requirements for inter-band concurrent V2X operation

| V2X | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| concurrent operating band configuration | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_n1A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n3A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n8A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n34A-n47A | E-UTRA Band 1, 3, 7, 8, 22, 26, 28, 39, 40, 41, 42, 44, 45, 65, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n34A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n40A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n41A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n71A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| V2X_n78A-n47A | Frequency range | 5925 | - | 5945 | -30 | 1 | 3 |
|  | Frequency range | 5835 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Applicable when NS_33 is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: Void. |  |  |  |  |  |  |  |

For the intra-band NR V2X transmission where Uu and SLoverlap in time , the UE-coexistence requirements in Table 6.5A.3.2.1-1 apply for the corresponding intra-band concurrent operation for the both uplink and sidelink transmission in licensed band.

6.5E.3.4	Additional spurious emissions requirements for V2X

6.5E.3.4.1	General

This clause specifies additional spurious emission requirements for V2X operation

6.5E.3.4.2	Requirements for network signalling value "NS_33"

Table 6.5E.3.4.2-1: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5945 | -30 | 1 |  |
| Frequency range | 5835 |  | - | 5855 | -30 | 1 |  |
| NOTE 1: Void. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Void. |  |  |  |  |  |  |  |

When "NS_33" is configured from pre-configured radio parameters or the cell, and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any NR V2X UE emission shall fulfil either one of the two sets of conditions.

Table 6.5E.3.4.2-2: Requirements for spurious emissions to protect CEN DSRC for V2X UE

|  | Maximum Transmission Power (dBm EIRP1) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP1) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |
| NOTE 1: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. |  |  |

< END OF CHANGE #3 >