3GPP TSG-RAN WG4 Meeting #114		 			                        R4-2501691
Athens, Greece, 17th – 21st February, 2025
 
Agenda Item: 7.11.4

Source:	SKY Perfect JSAT

Title: Discussion on UE RF requirements for NR NTN Ku band

Document for:	Discussion

- Introduction

In the RAN4#113 WF in R4-2419790 [1], how to proceed with specifying off-axis EIRP requirement and on-axis spurious and off-axis spurious requirements for CEPT Ku band has agreed.

- Issue 1-2-4: Off-axis EIRP requirements for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the off-axis EIRP density limit based on EN 301 428, EN 302 340 and EN 302 977 as a starting point, the ECC decision can be further checked whether additional updated is needed.

- Issue 1-2-5: On-axis spurious and off-axis spurious requirement for the CEPT Ku band

- For the UL band 14.0 - 14.5 GHz (E-to-S) concerning requirements from ECC regulation and ETSI standards:

  - Specify the same on-axis spurious limits in “Carrier-on” state for both outside and inside as specified in EN 303 980, EN 303 981 as the starting point

  - Specify the same requirement as Ka band for the general and off-axis spurious limits in “Emissions disabled” state as starting point.

  - FFS on off-axis spurious limits in “Carrier-on” and “Carrier-off” state

In this contribution, we provide our views on UE RF requirements perspective.

Additionally, the following is an excerpt from RAN4#113 WF R4-2419790 [1], summarizing the agreed items regarding the Maximum EIRP and TRP of Ku-band VSAT UE.

- Issue 1-2-2: Maximum EIRP and TRP requirement - The following aspects will be consider if setting the maximum EIRP & TRP of the NTN Ku band in 38.101-5 - An absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separating consideration for non-GSO scenarios - Upper EIRP limits in the regulation - Enable flexibility for different application and scenarios - Issue 1-2-3: Maximum EIRP requirements and the minimum antenna size for the CEPT Ku band - For the CEPT Ku band, the following requirements from ECC regulation will be considered, FFS on how to reflect to the specifications and the VSAT types. - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(17)04) - NTN VSAT operating with GSO 14.25 – 14.5 GHz Max EIRP: 50 dBW (from ECC Decision(03)04) - NTN VSAT operating with GSO 14.0 – 14.25 GHz Max EIRP: 60 dBW (from ECC Decision(06)03) - Mobile VSAT / Land ESIM 14.0 – 14.5 GHz Max EIRP: 54.5 dBW (from ECC Decision(18)04) - Mobile VSAT / Maritime ESIM 14.0 – 14.5 GHz minimum antenna size: 0.6 m (from ECC Decision (05)10)

At RAN#106, it was agreed to include NTN VSAT Type 4 and Type 5 in the standardization scope for Ku-band. This document proposes requirements for Maximum EIRP and TRP, considering NTN VSAT Types.

- Analysis and Discussion

Off-axis EIRP requirements

In each ETSI standard, the chapter that specifies the off-axis e.i.r.p. emission density within the band contains:

- EN 301 428 (14.0-14.5 GHz, for VSAT communicating with GEO)

the following descriptions in addition to the above regulations.

- EN 302 340 (14.0-14.5 GHz, for ESV communicating with GEO)

the following descriptions in addition to the above regulations.

- EN 302 977 (14.0-14.5 GHz, for VMES communicating with GEO)

the following descriptions in addition to the above regulations.

As confirmed above, EN 301 428 (14.0-14.5 GHz, for VSAT communicating with GEO), EN 302 340 (14.0-14.5 GHz, for ESV communicating with GEO), and EN 302 977 (14.0-14.5 GHz, for VMES communicating with GEO) contain relaxation conditions for angles greater than 70 degrees.

Observation 1: EN 301 428 (14.0-14.5 GHz, for VSAT communicating with GEO), EN 302 340 (14.0-14.5 GHz, for ESV communicating with GEO), and EN 302 977 (14.0-14.5 GHz, for VMES communicating with GEO) contain relaxation conditions for angles greater than 70 degrees.

If the off-axis EIRP density limit is based on the specifications of EN 301 428, EN 302 340, and EN 302 977, we propose the following values for the off-axis EIRP density limits in any 40 kHz band, taking the above conditions into account.

33 - 25 logθ - 10logN		dBW		for		2.5° ≦ θ ≦ 7.0°

+12 - 10logN 					dBW		for		7.0° < θ ≦ 9.2°

36 - 25 logθ - 10logN		dBW		for		9.2° < θ ≦ 48°

-6 - 10logN					dBW 		for		48° < θ ≦ 70°

4 - 10logN					dBW 		for		70° < θ

Proposal 1: If the off-axis EIRP density limit is based on the specifications of EN 301 428, EN 302 340, and EN 302 977, we propose the following values for the off-axis EIRP density limits in any 40 kHz band, taking the above conditions into account.

33 - 25 logθ - 10logN		dBW		for		2.5° ≦ θ ≦ 7.0°

+12 - 10logN 					dBW		for		7.0° < θ ≦ 9.2°

36 - 25 logθ - 10logN		dBW		for		9.2° < θ ≦ 48°

-6 - 10logN					dBW 		for		48° < θ ≦ 70°

4 - 10logN					dBW 		for		70° < θ

On-axis spurious and off-axis spurious requirements

- ETSI standard

In each ETSI standard for Ku-band, the on-axis spurious and off-axis spurious requirement is specified based on radiated power (EIRP) as below.

On-axis “Carrier-on” radio state limits in EN 303 980 (for Fixed and in-motion Earth Stations communicating with NGSO) and EN 303 981 (for Fixed and in-motion Wide Band Earth Stations communicating with NGSO)

K is the factor that accounts for a reduction on the on-axis spurious radiation level in case of multiple earth stations operating on the same frequency.

The limit (14-K)dBW/1MHz is equivalent to those limits which specified in EN 301 428, EN 302 340, and EN 302 977 for GEO.

On-axis “Carrier-off” and “Emission disabled” radio state limits in EN 303 980, 981

The limit -11dBW/1MHz is equivalent to those limits which specified in EN 301 428, EN 302 340, and EN 302 977 for GEO.

Off-axis limits of radiated field strength at 10m in EN 303 980, 981

The limits are same as those limits which specified in EN 301 428 and EN 302 977 for GEO while are different from EN 302 340 for GEO.

Off-axis “Emission disabled” radio state limits in EN 303 980, 981

The limits are different from those limits which specified in EN 301 428, EN 302 340, and EN 302 977 for GEO.

Off-axis “Carrier-on” and “Carrier-off” radio state limits in EN 303 980, 981

The limits are different from those limits which specified in EN 301 428, EN 302 340, and EN 302 977 for GEO.

- Japan’s regulation

In Japan's regulations, the unwanted emissions requirement for GEO are divided into out-of-band and spurious domains, and both requirements are specified based on conducted power. These are stipulated in Radio Equipment Regulations Appendix No. 3 and the Ministry of Internal Affairs and Communications Notification No. 1228 of 2005.

Japan's regulations are based on ITU Radio Regulations Appendix 3, which in turn is based on ITU-R SM.1541 (Out-of-band domain) and ITU-R SM.329 (Spurious domain).

Values for frequency separation between the centre frequency and the boundaries of out-of-band and spurious domains

BN < 300 kHz					: 750 kHz

300 kHz £ BN £ 250 MHz	: 2.5 BN

250 MHz < BN					: 1.5 BN + 250 MHz

fc: the center frequency of the emission

BN: 	the necessary bandwidth

Out-of-band domain

The less stringent of the following conditions for a reference bandwidth of 4 kHz:

- 40log(2F/BN+1) dBsd

- For P > 50W : 60dBc or higher

For P £ 50W : 50μW or less

F: Frequency separation between BN edge and unwanted emission (Hz)

P: Average power of fundamental frequency (W)

Spurious domain

For a reference bandwidth of 4 kHz:

For P > 50W: 60dBc or higher

For P £ 50W: 50μW or less

P: Average power of fundamental frequency (W)

Observation 2: While spurious emissions of ETSI standards are specified based on radiated power (EIRP), Japan's regulations for unwanted emissions, which is based on ITU Radio Regulations Appendix 3, are specified based on conducted power.

Proposal 2: When deciding whether to align the UE emission requirements of TS 38.101-5 with the values of the ETSI standard, we propose considering the regulations of other countries.

Maximum EIRP and TRP requirements

SKY Perfect JSAT calculated link budget and estimate the link quality and the achievable throughput in case 5G NR waveform carrier is transmitted from Ku-band VSAT UE to GSO Software Defined Satellite and legacy GSO satellite. The details are provided in the contribution R4-2419558[2]. This contribution states that, as a result of the link budget analysis, an EIRP of 88.2 dBm is required.

In the link budget analysis assumed in R4-2419558 for SDS (Software Defined Satellite) HTS (High Throughput Satellite), interference between spot beams was not considered. SDS (HTS) satellites achieve high throughput by using smaller spot beams and reusing frequencies compared to legacy satellites. However, due to frequency reuse, inter-beam interference occurs, and considering this interference in link budget leads to a more realistic analysis.

Therefore, in this document, a link budget analysis was conducted considering inter-beam interference, assuming the VSAT UE EIRP of 88.2 dBm as agreed in the WF. Additionally, a link budget analysis was also conducted using an EIRP of 76.2 dBm, which was the baseline at the beginning of the Ku-band study and also the requirement value for Ka-band, 76.2 dBm.

For inter-beam interference, a C/I of 10 dB was assumed, while all other conditions remains the same as those in R4-2419558. The link budget results are presented below.

Table 2.3-1: Link Budget Analysis for GSO Software Defined Satellites (SDS)

(VSAT UE EIRPMAX =88.2dBm) with Consideration of inter-beam interference

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x 1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Channel Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.1523 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 | 58.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 17.99 | 17.99 | 17.99 | 17.99 | 17.99 | 17.99 |
| Downlink | Satellite EIRP | dBW | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 |
|  | Output back-off | dB | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 |
| Total | Total C/N | dB | 17.75 | 17.75 | 17.75 | 17.75 | 17.75 | 17.75 |
|  | C/I: inter-beam Interference | dB | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 |
|  | C/(N+I) | dB | 9.33 | 9.33 | 9.33 | 9.33 | 9.33 | 9.33 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 18.80 | 15.98 | 11.12 | 6.90 | 2.96 | -0.94 |

Table 2.3-2: Link Budget Analysis for Software Defined Satellites(SDS)

(VSAT UE EIRPMAX=76.2dBm) with Consideration of inter-beam interference

| Carrier | CQI |  | 1 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modulation |  | QPSK | QPSK | QPSK | QPSK | 16QAM | 64QAM |
|  | Code Rate x 1024 |  | 78 | 120 | 308 | 602 | 490 | 466 |
|  | Channel Bandwidth | MHz | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 | 18.72 |
|  | Bit Efficiency | bps/Hz | 0.1523 | 0.23 | 0.60 | 1.18 | 1.91 | 2.73 |
|  | Throughput | Mbps | 2.85 | 4.39 | 11.26 | 22.01 | 35.83 | 51.11 |
| Uplink | Uplink EIRP | dBW | 46.20 | 46.20 | 46.20 | 46.20 | 48.00 | 46.20 |
|  | Uplink path loss | dB | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 | -207.09 |
|  | Satellite G/T | dB/K | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 | 11.00 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Uplink C/N | dB | 5.99 | 5.99 | 5.99 | 5.99 | 7.79 | 5.99 |
| Downlink | Satellite EIRP | dBW | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 | 58.00 |
|  | Output back-off | dB | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 | 10.78 |
|  | Downlink path loss | dB | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 | -205.93 |
|  | Rx Antenna G/T | dB/K | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 | 33.30 |
|  | Bandwidth | dBHz | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 | -72.72 |
|  | Boltzmann Constant | dBW/Hz･K | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 | -228.60 |
|  | Downlink C/N | dB | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 | 30.47 |
| Total | Total C/N | dB | 5.97 | 5.97 | 5.97 | 5.97 | 7.76 | 5.97 |
|  | C/I: inter-beam Interference | dB | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 | 10.00 |
|  | C/(N+I) | dB | 4.53 | 4.53 | 4.53 | 4.53 | 5.73 | 4.53 |
|  | Required C/N | dB | -9.48 | -6.66 | -1.80 | 2.42 | 6.37 | 10.27 |
|  | C/N Margin | dB | 14.00 | 11.18 | 6.32 | 2.10 | -0.64 | -5.74 |

Similar to the maximum EIRP requirement values for the Ka-band, the requirement values for the Ku-band are also defined using a 13 RBs allocation with a 120 kHz SCS. Based on this premise, the evaluation is conducted using the bit efficiency achievable within the allocated bandwidth of 18.72 MHz.

Considering inter-beam interference, the bit efficiency achievable with a certain C/N margin for SDS satellites is approximately 1 bps/Hz when the EIRP is 76.2 dBm, while it reaches around 2 bps/Hz when the EIRP is 88.2 dBm.

Since a bit efficiency of 2 bps/Hz or higher is desirable for GSO services, this link budget analysis indicates that an EIRP of 88.2 dBm is necessary.

Observation 3: In the link budget analysis assuming SDS satellites, when considering inter-beam interference, the bit efficiency was approximately 1 bps/Hz at an EIRP of 76.2 dBm and around 2 bps/Hz at an EIRP of 88.2 dBm.

At RAN#106, it was agreed that the standardization scope for Ku-band includes NTN VSAT Type 4 (Mobile VSAT communicating with GSO using a mechanically steered antenna) and Type 5 (Mobile VSAT communicating with GSO and NGSO using an electronically steered antenna). Since NTN VSAT Type 4 is intended for GSO only, a maximum EIRP requirement of 88.2 dBm is appropriate.

Proposal 3: The maximum EIRP requirement is appropriate to set at 88.2 dBm for NTN VSAT Type 4 in the Ku-band.

On the other hand, NTN VSAT Type 5 covers both GSO and NGSO. Based on the above link budget analysis, a bit efficiency of 2 bps/Hz or higher is desirable for GSO services, regardless of whether a parabolic antenna or an electronically steered antenna is used. Therefore, an EIRP of 88.2 dBm is also necessary for NTN VSAT Type 5.

Following the previous meeting’s WF agreement, it is appropriate for NTN VSAT Type 5 to adopt "an absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separate consideration for non-GSO scenarios."

Proposal 4: For NTN VSAT Type 5 in the Ku-band, an absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separate consideration for non-GSO scenarios is proposed.

- Conclusions

Observation 1: EN 301 428 (14.0-14.5 GHz, for VSAT communicating with GEO), EN 302 340 (14.0-14.5 GHz, for ESV communicating with GEO), and EN 302 977 (14.0-14.5 GHz, for VMES communicating with GEO) contain relaxation conditions for angles greater than 70 degrees.

Proposal 1: If the off-axis EIRP density limit is based on the specifications of EN 301 428, EN 302 340, and EN 302 977, we propose the following values for the off-axis EIRP density limits in any 40 kHz band, taking the above conditions into account.

33 - 25 logθ - 10logN		dBW		for		2.5° ≦ θ ≦ 7.0°

+12 - 10logN 					dBW		for		7.0° < θ ≦ 9.2°

36 - 25 logθ - 10logN		dBW		for		9.2° < θ ≦ 48°

-6 - 10logN					dBW 		for		48° < θ ≦ 70°

4 - 10logN					dBW 		for		70° < θ

Observation 2: While spurious emissions of ETSI standards are specified based on radiated power (EIRP), Japan's regulations for unwanted emissions, which is based on ITU Radio Regulations Appendix 3, are specified based on conducted power.

Proposal 2: When deciding whether to align the UE emission requirements of TS 38.101-5 with the values of the ETSI standard, we propose considering the regulations of other countries.

Observation 3: In the link budget analysis assuming SDS satellites, when considering inter-beam interference, the bit efficiency was approximately 1 bps/Hz at an EIRP of 76.2 dBm and around 2 bps/Hz at an EIRP of 88.2 dBm.

Proposal 3: The maximum EIRP requirement is appropriate to set at 88.2 dBm for NTN VSAT Type 4 in the Ku-band.

Proposal 4: For NTN VSAT Type 5 in the Ku-band, an absolute maximum upper limit on 88.2 dBm EIRP fulfilling GSO scenarios, while allowing separate consideration for non-GSO scenarios is proposed.

- References

- R4-2419790, Way Forward for [113][314] NR_NTN_Ku_Band_UE_SAN_RF; Moderator (CHTTL), RAN4#113

- R4-2419558, Ku-band Maximum TRP and EIRP Analysis; SKY Perfect JSAT, RAN#113