# 3GPP TSG-RAN WG4 Meeting #114	R4-2501696

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.1

Source: 		Qualcomm Incorporated

Title: 		Views on general aspects for SBFD

Document for: 		Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#113, a WF was agreed in [2] entailing several aspects regarding SBFD general system design and deployment. In this contribution we share our views on the general aspects for Rel-19 SBFD WI within RAN4.

# General aspects

## Specification implementation of operating bands support for SBFD

In [2], the following was agreed regarding how to indicate which bands could support SBFD deployment

The following TP is adopted for clause 5.2 in TS 38.104 - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96, n101 and n102, and NR operating bands in FR2-1 which are defined in Table 5.2-2. FFS a note in the band table 5.2-1 to indicate the NR TDD bands not applicable to SBFD operation.

In order to ensure clarity and provide future robustness for new bands to be defined in 3GPP, another alternative is to add a note in Table 5.2-1 in TS 38.104 for any TDD band that is not applicable to SBFD operation. By doing so, it will be easier for the readier to follow which bands are excluded for the SBFD feature and any new standardized band within RAN$ could be easily integrated into this framework.

Proposal 1: RAN4 to agree on the below framework regarding specification implementation of operating bands support for SBFD: Add the following statement in Section 5.2, TS 38.104:

  - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 and FR2-1, which are defined in Table 5.2-1 and Table 5.2-2, respectively. Bands with Note X presents exempted bands from supporting SBFD.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD (NOTE X) |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n46 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | TDD (NOTE 3, (NOTE X) |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD (NOTE X) |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD (NOTE X) |
| n54 | 1670 MHz – 1675 MHz | 1670 MHz – 1675 MHz | TDD (NOTE X) |
| n96 (NOTE 4) | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | TDD (NOTE 3, NOTE X) |
| n975 | 2300 MHz – 2400 MHz | N/A | SUL |
| n985 | 1880 MHz – 1920 MHz | N/A | SUL |
| n996 | 1626.5 MHz -1660.5 MHz | N/A | SUL |
| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD (NOTE X) |
| n1024 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD (NOTE3, NOTE X) |
| n1048 | 6425 MHz – 7125 MHz | 6425 MHz – 7125 MHz | TDD |
| n105 | 663 MHz – 703 MHz | 612 MHz – 652 MHz | FDD |
| NOTE 1: This band is applicable in China only. NOTE 2: Variable duplex operation does not enable dynamic variable duplex configuration by the network, and is used such that DL and UL frequency ranges are supported independently in any valid frequency range for the band. NOTE 3: This band is restricted to operation with shared spectrum channel access as defined in TS 37.213 [20]. NOTE 4: This band is applicable only in countries/regions designating this band for shared-spectrum access use subject to country-specific conditions. NOTE 5: The requirements for this band are applicable only where no other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area. For scenarios where other NR or E-UTRA TDD operating band(s) are used within the frequency range of this band in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 6: UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. NOTE 7: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order 20-51 [24] NOTE 8: [This band is applicable only in countries/regions designating this band for IMT licensed operation in accordance with RCC Recommendation 1/21.] NOTE X: SBFD operation is not supported for this band. |  |  |  |

## Declaration and flexibility for DL/UL subband configurations support for SBFD

In [2], the following was agreed regarding how to indicate which bands could support SBFD deployment

The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications

In our view, it is not possible to impose such a restriction on field operation of a given SBFD configuration. Typically, an infra-vendor will declare which SBFD configuration its product support and the testing spec will provide a number of configurations to be tested. However, from operation in the field, it is not common that 3GPP specs would enforce that operation of other-than-declared configurations shall not be operated. Such an enforcement comes typically from the regulatory bodies and the regulatory specs in different regions (e.g., Harmonized standards in Europe).

Observation 1: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

With focus on the SBFD DL/UL subband configuration, RAN4 had extensive discussions on specifying certain configurations, specifically for wide area BSs in Rel-19 work item. To progress on this, we understand the rational behind such proposals for wide area BSs. However, for medium range and local area BSs, we believe that such restriction is not needed. To recall from the study item phase, TR 38.858 Section 9.3.4 summarizes the conclusion of the feasibility of medium range BS as follows:

Based on RAN4 feasibility study on FR1 medium range BS, specifically the analysis on self-interference, co-site inter-sector co-channel inter-subband interference and inter-site inter-subband interference, RAN4 concluded that: - For self-interference analysis, the implementation feasibility of controlling the residual interference to meet the 1dB receiver desensitization target depends on the implementation aspects mentioned in clause 9.3.1. Based on the different assumptions and/or technique adoption for the above-mentioned implementation aspects, based on 5 companies’ technical inputs, companies have come to the following conclusions: - 1dB receiver desensitization target is achievable according to 4 companies. - 1dB receiver desensitization target is not achievable for a base station that strictly meets the RAN4 minimum requirements and/or operates with the maximum BS transmit power according to 2 companies. - Note: Among 5 companies, 1 company shows different feasibility results based on different assumptions. - Sectorized deployments of medium range BS are by no means universal. Dependent on the deployment needs, single sector MR BS are often deployed at least in some locations. In case a three sector deployment is used, for co-site inter-sector co-channel inter-subband interference, it can be observed that the implementation feasibility of controlling the co-site inter-sector co-channel inter-subband interference to meet the target (i.e., being less than certain level of receiver desensitization) depends on the implementation aspects mentioned in clause 9.3.2. Based on the different assumptions and/or technique adoption for the above-mentioned implementations aspects, and based on 3 companies’ technical inputs companies concluded that: - The total interference caused by two co-site sectors would be above the noise floor, causing a desensitization of the receiver, according to 2 companies. - The residual co-site inter-sector interference can be controlled, causing reasonable residual self-interference, causing up to 1 dB desensitization of the receiver, according to 1 company.

As we can observe, majority of companies during the study item showed the feasibility of SBFD in terms of self-interference handling to meet the 1dB Rx desense. In addition, RAN4 requirements represent minimum requirements, so, actual performance is expected to be better that the minimum requirements. One of the challenges of not being able to converge on a one view in RAN4 regarding the medium range feasibility during the study item is the dependency on the underlying implementation aspects. It is important to recall that requirements should be developed in a neutral manner without having a single/ specific architecture/ implementation solution in mind. Thus, the proposed WF that was captured in RAN4#113 that each RF front end operational configuration from the supported SBFD configurations shall be tested is not realistic as RAN4 was not able to agree on a benchmark architecture during the study item phase, due to the implementation differences.

For local area BSs, it was concluded in TR 38.858, Section 9.4.3 that from implementation point of view, it is feasible to have SBFD-capable local area BS. Similar conclusion was reached for FR2 BSs, for wide area, medium range, and local area classes. Accordingly, the restriction that the SBFD capable BS product shall not operate any SBFD configuration other than those that have been declared and tested should not be applicable to medium or local area base station classes.

Observation 2: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations.

Observation 3: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS.

Proposal 2: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested.

Regarding the definition of the UL/DL subband configurations for RF requirements, it is viable to consider a configuration-agnostic approach, such as the RF requirements are defined independent of the subband configuration. The supported configuration will impact the conformance work, which is one of the main driving factors of the above discussion. On one extreme, leaving the DL subband, UL subband, and guardband fully flexible might impose additional testing challenges, as conveyed by some companies within RAN4. On the other hand, specifying only one configuration will limit the feature deployment as operators will be restricted to deploy such a configuration.  Regarding the two alternatives above on top of the agreed option 2b from RAN4#112, it is important to ensure within RAN4 that the above discussion should be restricted to WA FR-1 SBFD-capable gNB. For other base station classes and FR2-1 gNB, from such a restriction might not be needed, since there might be more flexibility in term of switching between different SBFD configurations as different (and more flexible) interference cancellation techniques are feasible, in contrast, to less flexible cancellation for WA BS.

Proposal 3: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

Moving to the alternative one vs alternative two, our initial preference was alternative two as it is more aligned with how a SBFD capable BS is expected to be tested and deployed. Restricting the SBFD configuration only to one single configuration imposes unneeded restriction on the feature adoption without clear motivation why is that needed. In order to progress the discussion within RAN4, it is a viable to consider a fixed number of configurations to be supported by FR1 WA BS. It was discussed last meeting that this number can be two. Accordingly, RAN4 can further discuss what would be a reasonable number of configurations supported by the FR1 WA BS.

Proposal 4: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed.

## Dl/UL subband configuration support for SBFD

In [2], the following was agreed.

- For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

Similar to the discussion on the necessity to have additional restriction on the deployment of SBFD-capable BS with configurations that have only been declared and tested, in our view, restricting the number of UL subbands should be applied only to wide area BSs, given the background that was shared in Section 2.2. Restricting the SBFD configuration to only one DL subband size that is allowed to be declared and tested was agreed for wide area BS class. For medium range, we do not see the necessity to have such restrictions, and our preference is to keep it to the vendor to declare which DL subband it supports and only limit the number of supported UL subband sizes. In addition, no restrictions should be applied to local area base station class.

Proposal 5: RAN4 to agree that no restriction to be applied on the maximum number of supported UL subband sizes for medium range and local area base stations.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Proposal 1: RAN4 to agree on the below framework regarding specification implementation of operating bands support for SBFD: Add the following statement in Section 5.2, TS 38.104:

  - Subband full duplex (SBFD) is designed to operate in NR TDD operating bands in FR1 and FR2-1, which are defined in Table 5.2-1 and Table 5.2-2, respectively. Bands with Note X presents exempted bands from supporting SBFD.

Observation 1: Restrictions on in-field operation of certain configurations is not within the scope of RAN4 specs, but could be captured via regional regulations (e.g., Harmonized Standards in Europe).

Observation 2: It is challenging for RAN4 to agree on a reference RF solution for interference handling in SBFD-capable BS to be used to test SBFD configurations for medium range base stations.

Observation 3: In TR 38.858, no challenges were reported from implementation point of view on the feasibility of SBFD-capable local area BS.

Proposal 2: If it is feasible to capture RAN4 specs, RAN4 to consider the below only for wide area base stations

- The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared and tested.

Proposal 3: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

Proposal 4: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed.

Proposal 5: RAN4 to agree that no restriction to be applied on the maximum number of supported UL subband sizes for medium range and local area base stations.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, RAN4#113, Samsung.