# 3GPP TSG-RAN WG4 Meeting #114		R4-2501698

# Athens, GR, 17th – 21st February, 2025

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	7.18.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed regarding modelling of high power UEs as well as simulation parameters, scenarios and configurations that are the basis for the demodulation requirements to be defined in this work item. In this contribution, we provide our views on the open items captured in [2].

# 2 	Discussion

## 2.1	Number of interferers

In [2], below options were agreed:

- Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

During RAN4#113 meeting, several companies have submitted simulation results showing gains of MMSE-IRC over MMSE receiver for both 1 and 2 interferers. Such results are expected, since it was already captured in [3], § 8.1, that for the 2Rx cases, that both 1 and 2 interferers provide gains, however, considering 2 interferers did not show additional gain when compared to the single interferer case. Thus, it was agreed to only consider a single interferer case for the 2Rx scenario. Similar observations were captured in other RAN4#113 initial simulation results (e.g., R4-2417782).

Observation 1: Since the interference profile follows what was considered in TR 36.884, it is expected that gains trends between 1 and 2 interferers for 2Rx will be applicable.

In our views, having 2Rx requirements (whether based on 1 or 2 interferers) could be motivated to provide consistency with legacy PUSCH requirements, however, it is not clear if it reflects practical deployments. Accordingly, we propose to deprioritize 2Rx test cases when deciding on the simulation parameters and configurations needed to derive the final demodulation requirements.

Proposal 1: Since the consideration of 2Rx requirements in practical deployments might not be needed, it is proposal to down prioritize 2Rx test cases when finalizing the simulation parameters and configurations needed to derive the final demodulation requirements.

## 2.2	Test parameters and simulation assumptions

### 2.2.1	Channel bandwidth

In [2], below options were agreed:

- Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

In our view, the trend of having SINR gains between legacy MMSE and MMSE-IRC receiver is not expected to vary across different channel bandwidth configurations. In addition, minimum channel bandwidth assumption has been typically considered in RAN4 demodulation work. In this regard, prioritizing the smallest and largest channel bandwidth for 15KHz and 30KHz SCS (i.e., option 4) can be adopted in this work item as it will showcase the gains anticipated and will provide robust testability at the UE side, independent of the declared supported BW. Moreover, it would be desirable to reuse existing FRCs in TS 38.104 when deriving the requirements for MMSE-IRC receiver. In this regard, 5/20MHz BW for 15MHz and 10/100 MHz for 30KHz should be prioritized.

Proposal 2: RAN4 to consider option 4 (i.e., 5/50MHz CHWBW for 15KHz SCS and 10/100MHz BW with 30KHz SCS.

### 2.2.2	Waveform

In [2], below options were agreed:

- Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

It is recommended from our side to consider option 1 and not exclude DFT-s-OFDM at this stage. The rationale that with DFT-s-OFDM, the SINR requirements could be higher due to higher PAPR, thus making sure that we capture them throughout this work item is important. In addition, if simulation results show that in terms of requirements, no difference is observed between the two waveforms, then RAN4 could consider to capture only CP-OFDM in the specs.

Proposal 3: RAN4 to consider both CP-OFDM and DFT-s-OFDM waveforms. Further down scoping can be carried out after the simulations campaign.

### 2.2.4	Test requirement definition

In [2], below options were agreed:

- Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

SINR based requirement was used for DIP-based modelling for LTE PUSCH and PDSCH MMSE-IRC demodulation requirement in TS 36.104 and 36.101. On the other hand, PDSCH demod requirements were specified considering SNR based requirement definition. Generally, SINR-based requirement and SNR-based requirement could be regarded as equivalent. Given that RAN4 has agreed to use INR-based modelling for defining Rel-19 PUSH MMSE-IRC requirements, it is recommended to consider both and carry out the down scoping after the finalization of the parameters/ configurations needed to derive the requirements.

Proposal 4: RAN4 to select between SNR and SINR definition for the requirement after finalizing configuration and parameters needed to decide the demodulation requirements.

## 2.3	Manufacturer declarations and applicability rules

### 2.3.1 Applicability rule for different test scenarios

In [2], below options were agreed:

- Proposals on applicability for Homnet and Hetnet scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS - Proposals on additional applicability rule: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested

In [3], the demodulation requirements for HomNet scenario was not appliable for Local Area BS and Home BS, as such deployment was envisioned for macro urban deployments and not small cell or indoor deployments. On the other hand, the HetNet entail mixture of wide area and local area deployments. In this regard, it is viable to follow the same approach within this work item and consider only HomNet for wide area and medium range deployments (i.e., proposal 1 above).

Proposal 5: RAN4 to define HomNet requirements only for wide area and medium range type base stations.

# Conclusion

In this contribution we provide our views on the remaining issues of BS demodulation aspects for MMSE-IRC receiver. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: Since the interference profile follows what was considered in TR 36.884, it is expected that gains trends between 1 and 2 interferers for 2Rx will be applicable.

Proposal 1: Since the consideration of 2Rx requirements in practical deployments might not be needed, it is proposal to down prioritize 2Rx test cases when finalizing the simulation parameters and configurations needed to derive the final demodulation requirements.

Proposal 2: RAN4 to consider option 4 (i.e., 5/50MHz CHWBW for 15KHz SCS and 10/100MHz BW with 30KHz SCS.

Proposal 3: RAN4 to consider both CP-OFDM and DFT-s-OFDM waveforms. Further down scoping can be carried out after the simulations campaign.

Proposal 4: RAN4 to select between SNR and SINR definition for the requirement after finalizing configuration and parameters needed to decide the demodulation requirements.

Proposal 5: RAN4 to define HomNet requirements only for wide area and medium range type base stations.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2419780, “Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS”, RAN4#113, China Telecom, MediaTek.

[3]	TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS” 3GPP V13.1.0