3GPP TSG RAN WG4 Meeting #114	R4-2501703

Athens, Greece 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.12.3

Source: 	Ericsson

Title:	Discussion on Rel-19 ATG CA RRM Core Enhancement Related Remaining Issues

Document for:	Discussion

# Introduction

In the last meetings, we discussed and agreed on several general issues regarding the enhancement of the ATG CA. The approved WF of RAN4#113 can be found in [1]. This contribution discusses the open issues related to ATG CA enhancement.

# Discussion

In this section, we discuss the important open issues for this meeting.

## General Issues

## UE antenna type for inter-band CA

During the RF session of RAN4#113, it was agreed to discuss the possibility of case 4- ‘ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed’ [2]. Hence, the RAN4 RRM team should wait for the decision of the RF session on case 4.

RAN4 should wait for the decision of RAN4 RF on case 4.

## UE Antenna Type Signaling

The following open issues were discussed in the RAN4#113 meeting but not concluded. As the discussion regarding UE antenna types is pertinent to this work, we believe it is best to wait for the RAN4 RF session's decision on case 4 before moving forward with our discussion. If RAN4 does not preclude case 4, we should then send LS to RAN2 to define the capability to differentiate case 4 from case 5.

Issue 1-1-2: UE Antenna Type signaling FFS - Proposal 1: If ATG UE can perform measurements on different serving cells using different types of antennas, it is necessary to provide assistance information to the network regarding which antenna panel combination is used on the UE side. - Proposal 2: To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed with per BC type for the bands in which UE supporting phase antenna array.

RAN4 should wait for the RF session’s decision on Case 4 and send LS to RAN2 to define the capability to differentiate Case 4, and Case 5.

## Maximum Receive Time Difference (MRTD)

The following open issues were discussed in the last meeting but not concluded.

Issue 1-2-1: MRTD for ATG inter-band CA FFS - Option 1: - For the MRTD except Case#4: 33µs. Further discuss for case 4. - Option 2: - For the MRTD except Case#4: 33µs. - If case 4 is not excluded based on RF discussion: 3µs. - Option 3: - For the MRTD for all cases: 33µs.

In previous meetings, companies expressed diverse opinions on MRTD regarding inter-band CA. The requirement for the NR UE in FR1 MRTD is set at 33 µs, which takes into account TAE, dispersion, and propagation delay differences. Currently, there is no new UE type defined for ATG. We see no implementation issue in maintaining the same MRTD value for inter-band CA, as it accounts for all RTD scenarios, including case 4-‘ATG UE with an antenna array on the bands of PCC and SCC and one common Rx beam assumed.’ If we plan to specify a new MRTD for Case 4, we must first wait for the outcome of the RF session’s discussion. Moreover, it is essential to note that we have limited meetings remaining before finalizing our work. We believe an in-depth study is necessary before agreeing on any new MRTD value. Therefore, we support Option 3.

RAN4 should reuse the legacy MRTD requirement 33µs for ATG inter-band CA for all cases.

## Signaling Characteristics

## SCell Activation and Deactivation Delay

Issue 1-3-2: SCell Activation and Deactivation Delay FFS - Proposal 1: The SCell activation delay requirements will not be impacted by ATG UE antenna type. - Proposal 2: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type. - Proposal 3: SCell activation requirements shall be further investigated for potential impact, considering the UE antenna types.

The above open issues were discussed in the RAN4#113 meeting but not concluded. As per our understanding, the current SCell activation for FR1 does not capture the activation delay time for the array antenna in FR1. Hence, in sec 8.3.2, RAN 4 needs to capture the Activation delay requirements for the array antenna in FR1.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in milliseconds. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5 ms, if the measurement period of the SCell being activated is equal to or smaller than 2400 ms. - TFirstSSB_MAX + Trs + 5 ms, if the measurement period of the SCell being activated is larger than 2400 ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5 ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5 ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5 ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260 ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6 dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5 ms, otherwise. Otherwise, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6 ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3 ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2 ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3 ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5 ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260 ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6 dB; If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) or SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6 dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference cell and A-TRS/P-TRS of SSB-less SCell, and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured. Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for SCell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17, when the the EPRE difference is smaller than or equal to 12 dB - Tfirst_TRS + 2*TTRS +5 ms, when the EPRE difference is larger than 12 dB but smaller than or equal to [30] dB For a UE supporting scellWithoutSSB-InterBandCA-r18, when UE receive SCell activation command for more than one SSB-less SCell, the SCell activation delay for each of the to-be-activated SSB-less SCell in FR1 is the same as the single SSB-less SCell activation delay, when - SSB-less SCells being activated are on different bands, or - all SCells being activated are SSB-less SCells, the SCells are contiguous on the same band, and all to-be-activated SCells have the same QCL-typeC QCL source cell. If the SCell being activated belongs to FR1 array and if there is at least one active serving cell on that FR1 array, then Tactivation_time is TFirstSSB+ 5ms provided: - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and - SSB is in the same half-frame on the SCell and the contiguous FR1 active serving cell array. If the SCell being activated belongs to FR1 array and if there is no active serving cell on that FR1 array provided that PCell or PSCell is in FR1 or in FR1 array and If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is: - 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time. If the SCell being activated belongs to FR1 array and if there is no active serving cell on that FR1 array provided that PCell or PSCell is in FR1 or in FR1 array and If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is: - max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time. If the PCell/PSCell and the target SCell are configured as FR1-FR1array-1 CA or if the PCell/PSCell and the target SCell are in a FR1array-1 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), or - 6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported. If the PCell/PSCell and the target SCell are configured as FR1-FR1 array-FR1 array CA or if the PCell/PSCell and the target SCell are in a FR1 array-FR1 array band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR1 array-1 CA or if the PCell/PSCell and the target SCell are in a FR1 array-1 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, or - 3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)} if UE supports reduceForCellDetection and/or reduceForSSB-L1-RSRP-Meas and/or shortMeasInterval-r18 capabilities, and when SCell activation triggered L3 report is not configured or SCell activation triggered L3 report is configured but not reported. If the PCell/PSCell and the target SCell are configured as FR1-FR1 array-FR1 array CA or if the PCell/PSCell and the target SCell are in a FR1 array-FR1 array band pair with independent beam management(CBM), and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}

The example of the FR1 array needs to be accounted in section 8.3.2. Additionally, if cases 4 and 5 are agreed upon, we need to capture the SCell delay resulting from those cases.

RAN4 needs to capture the Activation delay requirements for the array antenna in FR1.

- Furthermore, the impact of ATG CA and the UE antenna types on the SCell Deactivation delay requirement is not foreseen.

No foreseen impact of ATG CA and the UE antenna types For SCell Deactivation delay requirement.

## SCell Addition/Activation Mechanism

During the RAN4#112-bis meeting, a company proposed exploring the concept of distance-triggered Secondary Cell (SCell) addition and activation[3]. Another proposed approach is to establish enhanced requirements with specific conditions that would allow measurements for SCell addition and scheduling restrictions on neighboring cell measurements for the candidate SCell to be omitted [4]. The application of these requirements would depend on the UE. However, section 38.133 already outlines multiple mechanisms for SCell activation and addition. Therefore, we do not see a strong justification for introducing an additional mechanism. Furthermore, implementing a distance-triggered SCell addition mechanism may require collaboration with RAN2, but this work item scope is currently limited to RAN4 only. Both proposals, as we understand them, are beneficial for ATG UE power saving and in CBM scenarios (case 4). Power saving is not within the scope of ATG UE performance enhancement metrics. Thus, before proceeding further with this discussion, we should wait for the decision of the RF session regarding case 4.

RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion.

## Interruption Requirements

The following open issues on interruption requirements were discussed in the RAN4#113 meeting and should be discussed in this meeting.

Issue 1-3-1: Interruption requirements - FFS whether following requirements need to be introduced - Interruptions at NR SRS antenna port switching - Further discuss the potential impact from UE antenna types - Further discuss how to reflect the interruption requirement for R19 ATG

As specified in the specification, during the NR SRS antenna Port Switching UL/DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx. Hence, RAN4 Should define Interruptions at NR SRS antenna port switching.

RAN4 Should reuse Interruptions at NR SRS antenna port switching for NR ATG CA.

Furthermore, among the agreed requirements below requirements should further study for possible impact from UE antenna type, especially the FR1 band with the array should be captured.

  - Interruptions at SCell addition/release

  - Interruptions at SCell activation/deactivation

  - Interruptions at fast SCell activation

  - Interruptions during measurements on deactivated SCC

  - Interruptions due to SCell dormancy

  - Interruptions due to UE-specific CBW change

  - Interruptions due to Active BWP switching Requirement

RAN4 Should discuss and capture FR1 antenna array impact on the following Interruptions requirements.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions at direct SCell activation

- Interruptions at fast SCell activation

- Interruptions during measurements on deactivated SCC

- Interruptions due to SCell dormancy

- Interruptions due to UE-specific CBW change

- Interruptions due to Active BWP switching Requirement

Moreover, the interruption requirement for Rel-19 ATG CA should be reflected in a separate section specific to ATG.

RAN4 Should Discuss the possibility to reflect the interruption requirement for R19 ATG in a separate section.

## Measurement Procedure

    - Scheduling Availability/Restrictions

The extension of scheduling restrictions to intra-band and inter-band CA is closely associated with the assumptions made about the UE antenna type. For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for RLM, BFD, CBD, intra-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inta-band carrier aggregation:

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

For Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for BFD, inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement following restrictions should be considered for inter-band carrier aggregation:

- For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in bands other than those in which PCell is configured.

- For ATG UE with the antenna array on PCell and one or more omnidirectional antennas on SCell, there are no scheduling restrictions on FR1 SCel configured in bands other than the ones in which PCell is configured.

- For ATG UE with the omnidirectional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in bands other than those in which PCell is configured.

- For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

- For ATG UE with the antenna array on both PCC and SCC, two Rx beams can be formed simultaneously. There are no scheduling restrictions on FR1 SCell configured in bands other than the bands in which PCell is configured.

RAN4 should discuss and further study the following Scheduling restriction options:

- Option1: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

- When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

- Option 2: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement. When inter-band carrier aggregation in FR1 is configured,

  - For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the antenna array on PCell and omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

  - For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

  - For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

## Sharing factor between L1 measurement and RRM measurement

In the RAN4#112-bis meeting, some companies proposed relaxing the sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements. However, to the best of our understanding, there is no feasible impact to address that is specific to ATG UE CA case. Hence, we propose using the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements.

RAN4 to reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations and proposals.

Observation 1	No foreseen impact of ATG CA and the UE antenna types For SCell Deactivation delay requirement.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 should wait for the decision of RAN4 RF on case 4.

Proposal 2	RAN4 to wait for the RF session’s decision on Case 4 and send LS to RAN2 to define the capability to differentiate Case 4 and Case 5.

Proposal 3	RAN4 should reuse the legacy MRTD requirement 33µs for ATG inter-band CA for all cases.

Proposal 4	RAN4 needs to capture the Activation delay requirements for the array antenna in FR1.

Furthermore, the impact of ATG CA and the UE antenna types on the SCell Deactivation delay requirement is not foreseen.

Proposal 5	RAN4 should wait for the decision of RF session on case 4 before proceed with the Scell addition/activation mechanism discussion.

Proposal 6	RAN4 Should reuse Interruptions at NR SRS antenna port switching for NR ATG CA.

Proposal 7	RAN4 Should discuss and capture FR1 antenna array impact on the following Interruptions requirements.

 Interruptions at SCell addition/release

 Interruptions at SCell activation/deactivation

 Interruptions at direct SCell activation

 Interruptions at fast SCell activation

 Interruptions during measurements on deactivated SCC

 Interruptions due to SCell dormancy

 Interruptions due to UE-specific CBW change

 Interruptions due to Active BWP switching Requirement

Proposal 8	RAN4 Should Discuss the possibility to reflect the interruption requirement for R19 ATG in a separate section.

Proposal 9	RAN4 should discuss and further study the following Scheduling restriction options:

Option1: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement.

      - When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions on FR1 serving PCell should apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

Option 2: For scheduling availability of UE performing measurement with a different subcarrier spacing than PDSCH/PDCCH for BFD, intra/inter-frequency measurement without MG, L1-RSRP measurement, and L1-SINR measurement. When inter-band carrier aggregation in FR1 is configured,

o For ATG UE with omnidirectional antennas on the bands of PCC and SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the antenna array on PCell and omni-directional antenna on SCell, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the omni-directional antenna on PCC and antenna array on SCC, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

o For ATG UE with the antenna array on both PCC and SCC, and only one Rx beam can be formed, the scheduling restrictions due to a given serving cell should also apply to another serving cell in a different band on the symbols that fully or partially overla with the aforementioned restricted symbols.

o For ATG UE with the antenna array on both PCC and SCC, and two Rx beams can be formed simultaneously, there are no scheduling restrictions on FR1 SCell configured in other bands than the bands in which PCell is configured.

Proposal 10	RAN4 to reuse the legacy sharing factor Klayer1_measurement and Psharing factor between L1 and RRM measurements.

# References

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC

- R4-2415002, (NR_ATG_enh-Core) Discussion on RRM core requirements for R19 ATG, CMCC

- R4-2417716, Discussion on enhancements for Rel-19 ATG RRM requirements, CATT