3GPP TSG RAN WG4 Meeting #114	R4-2501704

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.4.4

Source: 	Ericsson

Title:	NR CA/DC Performance Requirements for Less Than 5 MHz

Document for:	Discussion

# Introduction

The RRM core requirements related to the Rel-19 NR CA/DC for less than 5 MHz core requirement work item have been completed and will introduced in Rel-19 for less than 5 MHz based on [1].

This paper lists test cases to verify the RRM requirements for CA/DC. The test cases are proposed to be introduced in Rel-19. It also proposes a time plan for completing this work.

# NR less than 5 MHz CA/DC Test Case Scenario List

Tables 2.1 and 2.2 below list the proposed test cases to verify the RRM core requirements for NR CA/DC.

## Table 2.1: NR CA RRM Test Cases less than 5MHz

| No | Feature/Core Requirements | Linked Section in 38.133 | Type of Test | Basic Test Configuration | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Inter-Frequency CA Measurement in Idle Mode | 4.4.2.2, 10.1.4B, 10.1.9B | UE performs inter-frequency CA measurements in idle mode | Number of Cells: 3 PCell: FR1, 10 MHz BW SCells: Inter-frequency, 3 MHz BW MeasCycleSCell: 320 ms | Measurement accuracy for SS-RSRP, SS-RSRQ, for at least [7] inter-frequency carriers. |
| 2 | SCell Activation Delay in CA | 8.3.2 | UE activates an SCell with <5 MHz BW in an NR-CA scenario | PCell: FR1, 10 MHz BW SCell: FR1, 3 MHz BW Tactivation_time limits verified | Ensure SCell activation delay complies with CA timing requirements. |

## Table 2.2: NR DC RRM Test Cases for less than 5MHz

| No | Feature/Core Requirements | Linked Section in 38.133 | Type of Test | Basic Test Configuration | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | PSCell Addition and Change Delay in NR-DC | 6.1.6.1.2, 8.9.2 | UE undergoes PSCell addition or change in NR-DC with <5 MHz BW | PCell: FR1, 10 MHz BW PSCell: FR1, 3 MHz BW Measurement Conditions: TCHO_execution | Measure PSCell handover delay compliance in NR-DC. |
| 2 | SCG Activation Delay in NR-DC | 8.17.2 | UE transitions from deactivated to activated SCG state in NR-DC | MN: FR1, 10 MHz BW SCG: FR1, 3 MHz BW Timing Metrics: TSCG_activation_time | Validate SCG activation delay in NR-DC scenarios. |
| 3 | Conditional PSCell Change delay in NR-DC | 8.11B.2 | UE performs conditional PSCell change under NR-DC | PCell: FR1, 10 MHz BW PSCell: FR1, 3 MHz BW Performance metric: Activation within defined time thresholds | Assess PSCell addition delay under conditional NR-DC conditions. |

RAN4 should Discuss the test cases in tables 2.1 and 2.2 to verify the core requirements for the newly introduced Rel-19 CA RRM requirements for less than 5 MHz operation.

# Test Configurations for NR CA/DC for less than 5 MHzRRM Tests

Additional Test configurations for less than 5 MHz have already been introduced in Rel-18. RAN4 has introduced new PDCH RMC, CORESET for RMSI scheduling, and CORESET for RMC scheduling, all of which can be reused for Rel-19. We do not anticipate any significant changes needed for the test configuration aspect. RAN4 can reuse the basic configurations defined in Rel-18.

RAN4 can reuse the PDCH RMC, CORESET for RMSI scheduling, and CORESET for RMC scheduling specified for less than 5 MHz in Rel-18.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz test cases for the introduced core requirements in Rel-19, and we propose the following:

Proposal 1	RAN4 should Discuss the test cases in tables 2.1 and 2.2 to verify the core requirements for the newly introduced Rel-19 CA RRM requirements for less than 5 MHz operation.

Proposal 2	RAN4 can reuse the PDCH RMC, CORESET for RMSI scheduling, and CORESET for RMC scheduling specified for less than 5 MHz in Rel-18.

# References

- R4-2420241, Draft Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation.

- R4-2410396,Draft BigCr Introducing agreed test cases and common parameters for NR_FR1_lessthan_5MHz_BW, Nokia