3GPP TSG-RAN WG4 Meeting #114						R4-2501733

17th Feb, 2025 – 21st Feb, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx UE demodulation and CSI requirements with inter-cell and intra-cell inter-user interference

Agenda item:	 7.18.2

Document for:	Discussion

# 1.Introduction

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices:

  - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Reuse the existing interference model for 2/4Rx UE as a baseline

In Rel-18 topic NR_ENDC_ RF_FR1_enh2 WI, 8Rx performance requirements for SU-MIMO (single user) were discussed. In Rel-19, 8Rx performance requirements with inter-cell, and intra-cell inter-user interference to be defined for below objectives [1].

### In [2], RAN4 agreed on some of the issues, and encouraged companies to run simulations with R17 interference settings to further discuss considered scenarios. In this paper, we share some of our observations based on simulation results and share our proposals

# 2. PDSCH Demodulation Requirements inter-cell interference (ICI)

## 2.1 Rank choice

### Initial simulation results for inter-cell interference scenarios are shared in Table 2.1, and Table 2.2 for a single interfering cell, and two interfering cells respectively with following test parameters.

Simulation settings:

- FDD, 15KHz SCS, 10MHz BW

- TDA30-10, ULA Low or TDLC 300-100, ULA Low

- Rel-17 interference settings with rank1 or rank2

- Antenna configuration: Rank1: 2 x 8; Rank2: 2 x 8 ; Rank4: 4 x 8

- SNR values @ 70% of maximum throughput

- Interference power levels are 7.58 dB for 1 interferer; 7.77 dB and 2.28 dB for 2 interferers

Table 2.1 Simulation results with 1 interfering cell

| Serving cell | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Rank 1 | TDLA30-10 | 17 | 4.7 | 4.7 | 0 |
| Rank 2 | TDLA 30-10 | 17 | 8.6 | 9.2 | 0.4 |
| Rank 4 | TDLA 30-10 | 13 | 9.6 | 12.9 | 3.1 |

Table 2.2 Simulation results with 2 interfering cells

| Serving cell | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| Rank 1 | TDLC300-100 | 17 | 7.1 | 7.9 | 0.6 |
| Rank 2 | TDLA 30-10 | 17 | 10.0 | 11.3 | 1.2 |
| Rank 4 | TDLA 30-10 | 13 | 11.0 | 15.1 | 3.9 |

### Based on above tables, following are our observations:

Observation 1: Performance gap between the baseline and simplified receivers widens as rank increases as expected due to better interference rejection capability in baseline receiver.

Observation 2: For Rank1, SNR required for both receivers is significantly below interference power levels.

Observation 3: For Rank2 case, SNR gap between baseline and simplified receivers is comparable to the gap in Rank1 case. So, only a single requirement between Rank1 and Rank2 is sufficient in our view.

Observation 4: For Rank4 with single interfering cell, SNR required for both receivers is comparable to interference power level (7.5 dB) and is within Rel-17 SNR range (10 - 15 dB).

Observation 5: All RE have DMRS in Rank4 case unlike in Rank1 and Rank2 cases. So, the interference is present across all tones in DMRS symbols.

Observation 6: Rank4 with MCS13 can support higher throughput than Rank2 with MCS17 at this SNR.

Observation 7: Rank4 is appropriate to clearly distinguish between baseline and simplified receivers due to performance gap of more than 3 dB.

Requirements with Rank1

To ensure limited new requirements to UE, Rank1 requirement can be considered.

Requirements with Rank2

Rank2 requirements are not specified with 4Rx and only Rank1 requirements are considered in Rel-17. From simulation results, Rank2 does not substantially give more insights into performance of  interference + noise estimation or differentiate baseline and simplified receivers. To limit the overall new requirements, Rank2 need not be considered for PDSCH demodulation requirements. We are open to discuss further between Rank1 and Rank2 and choose one of them.

Requirements with Rank4

To demonstrate the true 8Rx (baseline) utility and performance gap with a simplified receiver, support higher spectral efficiency, and verify noise + interreference covariance estimation capability across all RE, Rank4 for serving cell, and up to Rank2 for interference cell (with Rel-17 interference model) is a suitable scenario. From simulation results, MCS13 is a good fit to show higher performance gap between the two kinds of receivers.

Field scenarios

Although concerns were earlier expressed on the probability of Rank4 in cell-edge, the Rank4 residency data with 8Rx will be higher than that with 4Rx since only two beams are required to support 4 layers. Moreover, with rich scattering Rank4 with 8Rx has a higher probability than full rank (Rank4) with 4Rx. Moreover, Rank4 MCS13 can achieve higher throughput than with 2 layers and MCS17 at this SINR regime.

With above reasoning, we propose the following:

Proposal1: RAN4 to have Rank1 requirements with MCS 17, and TDLA 30-10 ULA channel and TDLC300-100 for inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 shall not have Rank2 requirements for inter-cell interference demodulation requirements.

Proposal 3: RAN4 shall have Rank4 requirements with MCS 13 and TDLA 30-10 ULA and a  single interfering cell for inter-cell interference demodulation requirements.

Proposal 4: RAN4 shall re-use Rel-17 interference models for all demodulation requirements.

Proposal 5: RAN4 to have applicability rules to limit the overall number of test cases for both TDD and FDD.

# 3. PDSCH Demodulation Requirements intra-cell inter-user interference

In [2], RAN4 agreed to have Rank2 requirements, and Rank4 case was still under discussion.

## 3.1 MMSE-IRC (Interference rejection)

The MMSE-IRC fundamentally stays the same for both inter-cell interference and intra-cell inter-user interference cases. The key aspect of interference + noise estimation and then whitening is common for both scenarios. Therefore, our view is that scenarios can be similar in defining requirements for intra-cell inter-user (MU-MIMO) too.

3.2 Rank choice

Rank4

Rank4 for both serving and interfering cell requires TD-OCC DMRS (MAXLEN = 2) support which is an optional feature. This implies all the UEs which don’t support this capability are not going to be verified with the requirement. Moreover, such DMRS scheme has a higher DMRS overhead and is not a preferred option for network operators in our view. Furthermore, such DMRS scheme is not considered in RAN4 scenarios other than 8L, 8Rx requirement. R18-enhanced DMRS is a more suitable option to further consider Rank4 case to balance DMRS overhead. Considering above two aspects, we are of the view to not define Rank4 requirements.

Observation 7: Rank4 for both target and co-scheduled UE is not a feasible choice for simplified receiver since total number of layers is higher than the maximum rank supported by the simplified receiver.

Proposal 6: RAN4 shall not define MU-MIMO requirements for Rank4.

3.3 Simulation results for Rank2, and Rank4 MU-MIMO

The following table has the initial simulation results with rank2 for both target and co-scheduled UE.

Table 3.1 Simulation results with Rank2

| Rank of target and co-scheduled UE | Channel | MCS | Baseline receiver SNR (dB) | Simplified receiver SNR (dB) | Gap = Simplified – Baseline (dB) |
| --- | --- | --- | --- | --- | --- |
| 2 | TDLA 30-10 | 19 | 10.1 | 13.8 | 3.7 |
| 4 | TDLA 30-10 | 13 | 15.9 | NA | NA |

Proposal 7: RAN4 to consider Rank2 with MCS 19 and TDLA 30-10 for MU-MIMO demodulation requirements.

# 4. CQI requirements

For CQI requirements, two aspects need to be considered based on specs. defined so far. In Rel-17, CQI requirements for inter-cell interference case were defined with Rank1 and TDLA30-10 channel. While in Rel-18, 8Rx CQI requirements with Rank4 are specified only for static channel.

With static channel, and AWGN propagation from interferer, MMSE is sufficient and MMSE-IRC  performance shall not be better than MMSE. Therefore TDLA30-10 ULA channel and other Rel-17 settings can be extended to 8Rx inter-cell interference CQI requirements.

Proposal 8: RAN4 to use Rank1 with Rel-17 settings for 8Rx CQI requirements.

# 6. Applicability rules

We propose the following applicability rules to keep limited overall number of test cases to be passed by UE.

Proposal 9: RAN4 shall have the following applicability rule: UE that passes Rank1 ICI test case with TDLA30-10 in FDD (or TDD), need not be verified with TDLC300-100 in FDD (or TDD).

# 7. Release independence

Since interference requirements are being discussed in Rel-19 feature, release independence shall be considered from Rel-18 for

Proposal 10: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# 6. Conclusions

Following is the summary of the proposals.

Proposal1: RAN4 to have Rank1 requirements with MCS 17, and TDLA 30-10 ULA channel and TDLC300-100 for inter-cell interference PDSCH demodulation performance requirements.

Proposal 2: RAN4 shall not have Rank2 requirements for inter-cell interference demodulation requirements.

Proposal 3: RAN4 shall have Rank4 requirements with MCS 13 and TDLA 30-10 ULA and a  single interfering cell for inter-cell interference demodulation requirements.

Proposal 4: RAN4 shall re-use Rel-17 interference models for all demodulation requirements.

Proposal 5: RAN4 to have applicability rules to limit the overall number of test cases for both TDD and FDD.

Proposal 6: RAN4 shall not define MU-MIMO requirements for Rank4.

Proposal 7: RAN4 to consider Rank2 with MCS 19 and TDLA 30-10 for MU-MIMO demodulation requirements.

Proposal 8: RAN4 to use Rank1 with Rel-17 settings for 8Rx CQI requirements.

Proposal 9: RAN4 shall have the following applicability rule: UE that passes Rank1 ICI test case with TDLA30-10 in FDD (or TDD), need not be verified with TDLC300-100 in FDD (or TDD).

Proposal 10: Rel-19 8Rx performance requirements shall be independent from Rel-18.

# References

[1] RP-241297, “Revised WID on NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #104, June 17– June 20, 2024, Shanghai, China.

[2] R4-2419784, “Way Forward for [113][322] NR demodulation performance: Phase 5”, 3GPP TSG RAN meeting, #113, Nov18– Nov 22, 2024, Orlando, USA.