3GPP TSG-RAN4 Meeting #114	R4-2501734

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | xxxx | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to add new requirements for EESS protection from n257 and n258 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mmWave_protect-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to WRC-19 decision, a limit of −29 dBW unwanted emission power in any 200 MHz (i.e., equivalent to +1 dBm/200 MHz) of the EESS should be met for IMT mobile stations (UE), and the limit is set to −35 dBW/200 MHz (i.e., equivalent to -5 dBm/200 MHz) for use after 1 September 2027. Therefore, it is necessary to update the requirements related to EESS protection in accordance with the regulations of each region. Based on discussions in RAN4, the requirements related to NS_202/CA_NS_202 will be updated and new requirements related to NS_205/CA_NS_205 will be introduced. These are mandatory requirements for new UEs, and the corresponding modifiedMPRbehavior bit shall be indicated from Rel-15. The table for definitions of the bits in the field modifiedMPRbehavior will be updated from Rel-15 TS38.101-2 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements will be changed: - Clause 6.2.3.1: The mapping for NS_205 and related requirements will be added. - Clause 6.2.3.3: A-MPR requirements for NS_202 will be updated. - Clause 6.2.3.5: A-MPR requirements for NS_205 will be introduced. - Clause 6.2A.3.1: The mapping for CA_NS_205 and related requirements will be added. - Clause 6.2A.3.3: A-MPR requirements for CA_NS_202 will be updated. - Clause 6.2A.3.5: A-MPR requirements for CA_NS_205 will be introduced. - Clause 6.5.3.2.3: Additional spurious emission requirements for NS_202 will be updated. - Clause 6.5.3.2.5: Additional spurious emission requirements for NS_205 will be introduced. - Clause 6.5A.3.2.5: Additional spurious emission requirements for CA_NS_205 will be introduced. - Annex H.1: Definitions of the bits in the field modifiedMPRbehavior for n257 and n258 will be updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for EESS protection in accordance with the regulations of each region will not be introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1, 6.2.3.3, 6.2.3.5 (new), 6.2A.3.1, 6.2A.3.3, 6.2A.3.5 (new), 6.5.3.2.3, 6.5.3.2.6 (new), 6.5A.3.2.5 (new), Annex H.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements.

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_201 (NOTE 1) | 6.5.3.2.2 | n258 |  |  | 6.2.3.2 |
| NS_202 | 6.5.3.2.3 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.3 |
| NS_203 | 6.5.3.2.4 | n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.4 |
| NS_205 | 6.5.3.2.6 | n257, n258 | 50, 100, 200, 400 | Table 5.3.2-1 | 6.2.3.5 |
| NOTE 1: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2.3.1-2: Mapping of Network Signalling labe

| NR Band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_202 | NS_205 |  |  |  |  |  |
| n258 | NS_200 | NS_201 (NOTE 2) | NS_202 | NS_203 | NS_205 |  |  |  |
| n259 | NS_200 |  |  |  |  |  |  |  |
| n260 | NS_200 |  |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| n263 | NS_200 | NS_204 |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

6.2.3.3	A-MPR for NS_202

6.2.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

6.2.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 2.0 dB.

6.2.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.5	A-MPR for NS_202 for power class 5

For power class 5, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.6	A-MPR for NS_202 for power class 6

For power class 6, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

6.2.3.3.7	A-MPR for NS_202 for power class 7

For power class 7, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

<Unchanged sections are omitted>

6.2.3.5	A-MPR for NS_205

6.2.3.5.1	A-MPR for NS_205 for power class 1

For power class 1, A-MPR for NS_205 shall be 7.0 dB if Offset frequency < BWchannel, 6.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel bandwidth.

6.2.3.5.2	A-MPR for NS_205 for power class 2

For power class 2, A-MPR for NS_205 specified in subclause 6.2.3.5.3 applies.

6.2.3.5.3	A-MPR for NS_205 for power class 3

For power class 3, A-MPR for NS_205 shall be 2.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel bandwidth.

6.2.3.5.4	A-MPR for NS_205 for power class 4

For power class 4, A-MPR for NS_205 specified in subclause 6.2.3.5.3 applies.

6.2.3.5.5	A-MPR for NS_205 for power class 5

For power class 5, A-MPR for NS_205 specified in subclause 6.2.3.5.3 applies.

6.2.3.5.6	A-MPR for NS_205 for power class 6

For power class 6, A-MPR for NS_205 specified in subclause 6.2.3.5.3 applies.

6.2.3.5.7	A-MPR for NS_205 for power class 7

For power class 7, A-MPR for NS_205 specified in subclause 6.2.3.5.3 applies.

<Unchanged sections are omitted>

6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_201 | 6.5A.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_202 | 6.5A.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_203 | 6.5A.3.2.4 | n258 |  |  | 6.2A.3.4 |
| CA_NS_205 | 6.5A.3.2.5 | n257, n258 |  |  | 6.2A.3.5 |
| NOTE: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_202 | CA_NS_205 |  |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_201 | CA_NS_202 | CA_NS_203 | CA_NS_205 |  |  |  |
| n259 | CA_NS_200 |  |  |  |  |  |  |  |
| n260 | CA_NS_200 |  |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. NOTE 2: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

<Unchanged sections are omitted>

6.2A.3.3	A-MPR for CA_NS_202

6.2A.3.3.1	A-MPR for CA_NS_202 for power class 1

For intra-band contiguous CA with single contiguous RB allocation, A-MPR for CA_NS_202 shall be 11.0 dB.
For intra-band contiguous CA with non-contiguous RB allocation or intra-band non-contiguous CA, A-MPR for CA_NS_202 shall be [13.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 11.0 dB otherwise.
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.3.2	A-MPR for CA_NS_202 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

6.2A.3.3.3	A-MPR for CA_NS_202 for power class 3

For intra-band contiguous CA with single contiguous RB allocation, A-MPR for CA_NS_202 shall be [5.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 2.0 dB otherwise.
For intra-band contiguous CA with non-contiguous RB allocation or intra-band non-contiguous CA, A-MPR for CA_NS_202 shall be [8.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 2.0 dB otherwise.
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.3.4	A-MPR for CA_NS_202 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

6.2A.3.3.5	A-MPR for CA_NS_202 for power class 5

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

6.2A.3.3.6	A-MPR for CA_NS_202 for power class 6

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

<Unchanged sections are omitted>

6.2A.3.5	A-MPR for CA_NS_205

6.2A.3.5.1	A-MPR for CA_NS_205 for power class 1

For intra-band contiguous CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be [9.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 6.0 dB otherwise.
For intra-band contiguous CA with non-contiguous RB allocation or intra-band non-contiguous CA, A-MPR for CA_NS_205 shall be [13.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 6.0 dB otherwise.
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.5.2	A-MPR for CA_NS_205 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.5.3 applies.

6.2A.3.5.3	A-MPR for CA_NS_205 for power class 3

For intra-band contiguous CA with single contiguous RB allocation, A-MPR for CA_NS_205 shall be [5.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB otherwise.
For intra-band contiguous CA with non-contiguous RB allocation or intra-band non-contiguous CA, A-MPR for CA_NS_205 shall be [8.0] dB if Offset frequency < BWChannel_CA of the UL CA configuration, 0.0 dB otherwise.
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC among the configured UL CA.

6.2A.3.5.4	A-MPR for CA_NS_205 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.5.3 applies.

6.2A.3.5.5	A-MPR for CA_NS_205 for power class 5

For intra-band contiguous CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.5.3 applies.

6.2A.3.5.6	A-MPR for CA_NS_205 for power class 6

For intra-band contiguous CA, A-MPR for CA_NS_205 specified in sub-clause 6.2A.3.5.3 applies.

<Unchanged sections are omitted>

6.5.3.2.3	Additional spurious emission requirements for NS_202

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | -5 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

<Unchanged sections are omitted>

##### 6.5.3.2.6	Additional spurious emission requirements for NS_205

When "NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.6-1: Additional requirements (NS_205)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | -5 | 200 MHz |

<Unchanged sections are omitted>

6.5A.3.2.5	Additional spurious emission requirements for CA_NS_205

When "CA_NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

<Unchanged sections are omitted>

H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behaviour indicated per supported NR band in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. Moreover, the bits in the field can explicitly indicate NS value(s) supported by a UE.

NOTE 1:	In the present release, the modifiedMPR-Behaviour is indicated [13] by an 8-bit bitmap per supported NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n257 |
|  | 1 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 of 38.101-2 [v19.x.x] | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
|  | 2 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.5 of 38.101-2 [v19.x.x] | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |
| n258 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n258 |
|  | 1 | Void |  |
|  | 2 | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 3 | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 of 38.101-2 [v19.x.x] | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
|  | 4 | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.5 of 38.101-2 [v19.x.x] | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |
| n260 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 | - This bit shall be set to 1 by a UE supporting n261 |

<Unchanged sections are omitted>