3GPP TSG-RAN4 Meeting #114	R4-2501739

# Athens, Greece 17th – 21st February, 2025

Title:	A-MPR values for NS_36 and NS_26

Source:	Qualcomm Incorporated

Agenda item:	6.17.1

Document for:	Information

# 1.	Introduction

This contribution provides NS_26 and NS_36 AMPR specifications for the n68 band based on simulations and lab measurements. The simulations and measurements assume the same filter assumptions as previously utilized in the Rel-17 band 68 A-MPR specifications [1].

# 2. 	Discussion

## 2.1	Simulation and measurement filter assumptions

In our simulations and measurements, we utilize the filter assumption stated in [1]. The filter assumptions are based on 15MHz passband filter under extreme conditions, as highlighted in yellow in Table 1 below.

Table 1: Filter characteristics

| Frequency (MHz) | Vendor B SAW 15MHz passband |  |
| --- | --- | --- |
|  | Relative rejection (dB) |  |
| MHz | Extreme | Normal |
| 693.5 | 0.0 | 10.0 |
| 692.5 | 4.0 | 16.0 |
| 691.5 | 10.0 | 20.0 |
| 690.5 | 16.0 | 26.0 |
| 689.5 | 20.0 | 30.0 |
| 688.5 | 26.0 | 30.0 |
| 687.5 | 30.0 | 30.0 |
| 686.5 | 30.0 | 30.0 |

## 2.2 A-MPR results and proposed A-MPR

We analysed A-MPR for 5, 10 and 15 MHz channel bandwidths located at the lowest frequencies of n68 band to fulfil -25dBm/8MHz (NS_26) and -42dBm/8MHz (NS_36) below 694MHz, considering extreme filter assumptions given in section 2.1. We also used the following impairment levels in our simulations and measurements:

- Carrier feedthrough = -28dBc

- Image = -28dBc

- CIM3 = -60dBc

### 2.2.1 Proposed regions and corresponding A-MPR specification for NS_26

In Table 2, we show the proposed A-MPR regions for 10 and 15 MHz channel bandwidths for NS_26 requirement, and their corresponding A-MPR values. The regions were calculated based on our simulations and lab measurements.

| Channel Bandwidth (MHz) | Region A |  | Region B |  |
| --- | --- | --- | --- | --- |
|  | RBstart | LCRB | RBstart | LCRB |
| 10 | ≤ 1.8MHz/12/SCS | ≤ 3.96MHz/12/SCS | ≤ 2.16MHz/12/SCS | > 3.96MHz/12/SCS |
| 15 | ≤ 3.06MHz/12/SCS | ≤ 4.68MHz/12/SCS | ≤ 4.68MHz/12/SCS | > 4.68MHz/12/SCS |

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A | Region B | Region A | Region B |
| 10 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |
| 15 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |

Table 2: Proposed A-MPR regions and values for NS_26

Proposal 1: Adopt the following A-MPR region and values for NS_26.

| Channel Bandwidth (MHz) | Region A |  | Region B |  |
| --- | --- | --- | --- | --- |
|  | RBstart | LCRB | RBstart | LCRB |
| 10 | ≤ 1.8MHz/12/SCS | ≤ 3.96MHz/12/SCS | ≤ 2.16MHz/12/SCS | > 3.96MHz/12/SCS |
| 15 | ≤ 3.06MHz/12/SCS | ≤ 4.68MHz/12/SCS | ≤ 4.68MHz/12/SCS | > 4.68MHz/12/SCS |

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A | Region B | Region A | Region B |
| 10 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |
| 15 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |

### 2.2.2 Proposed regions and corresponding A-MPR specification for NS_36

In Figure 1, we show the graphical representation of the proposed A-MPR regions for 5, 10 and 15 MHz channel bandwidths for NS_36 requirement. The regions were calculated based on our simulations and lab measurements.

Figure 1. Graphical representation of proposed NS_36 A-MPR regions for 5, 10, and 15 MHz channel bandwidths

In the tabular Figure 2 below, we show the proposed A-MPR region definitions for 5, 10 and 15 MHz channel bandwidths for NS_36 requirement.

Figure 2. Proposed NS_36 A-MPR region definitions for 5, 10, and 15 MHz channel bandwidths

Table 3 below proposes the corresponding required A-MPR for each region defined above in Figure 2 for NS_36 requirement, across applicable channel bandwidths, modulations and waveforms.

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A1 | Region A2 | Region A1 | Region A2 |
| 5 | QPSK | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 16QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 64QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 256QAM | 7.0 |  | 7.0 |  |
| 10 | QPSK | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 16QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 64QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 256QAM | 10.0 |  | 10.0 |  |
| 15 | QPSK | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 16QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 64QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 256QAM | 10.0 | 7.0 | 10.0 | 7.0 |

Table 3: Proposed A-MPR values for NS_36

Proposal 2: Adopt the following A-MPR region and values for NS_36.

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A1 | Region A2 | Region A1 | Region A2 |
| 5 | QPSK | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 16QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 64QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 256QAM | 7.0 |  | 7.0 |  |
| 10 | QPSK | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 16QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 64QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 256QAM | 10.0 |  | 10.0 |  |
| 15 | QPSK | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 16QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 64QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 256QAM | 10.0 | 7.0 | 10.0 | 7.0 |

# Conclusion

A-MPR specification proposals for n68 band NS_26 and NS_36 were presented as information to RAN4, based on lab measurements and simulations. The following observations were made:

Proposal 1: Adopt the following A-MPR region and values for NS_26.

| Channel Bandwidth (MHz) | Region A |  | Region B |  |
| --- | --- | --- | --- | --- |
|  | RBstart | LCRB | RBstart | LCRB |
| 10 | ≤ 1.8MHz/12/SCS | ≤ 3.96MHz/12/SCS | ≤ 2.16MHz/12/SCS | > 3.96MHz/12/SCS |
| 15 | ≤ 3.06MHz/12/SCS | ≤ 4.68MHz/12/SCS | ≤ 4.68MHz/12/SCS | > 4.68MHz/12/SCS |

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A | Region B | Region A | Region B |
| 10 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |
| 15 | QPSK | 3.5 | 5.0 | 2.0 | 3.5 |
|  | 16QAM | 3.5 | 5.0 | 2.5 | 4.0 |
|  | 64QAM |  | 5.0 | 3.0 | 4.5 |
|  | 256QAM |  |  |  |  |

Proposal 2: Adopt the following A-MPR region and values for NS_36.

| Channel Bandwidth (MHz) | Modulation order | CP-OFDM |  | DFT-s-OFDM |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Region A1 | Region A2 | Region A1 | Region A2 |
| 5 | QPSK | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 16QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 64QAM | 7.0 | 4.0 | 7.0 | 4.0 |
|  | 256QAM | 7.0 |  | 7.0 |  |
| 10 | QPSK | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 16QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 64QAM | 10.0 | 6.0 | 10.0 | 6.0 |
|  | 256QAM | 10.0 |  | 10.0 |  |
| 15 | QPSK | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 16QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 64QAM | 10.0 | 7.0 | 10.0 | 7.0 |
|  | 256QAM | 10.0 | 7.0 | 10.0 | 7.0 |

# Reference

[1]	R4-2417080, “WF on A-MPR for NR band n68”, Ericsson, RAN4#112bis