3GPP TSG-RAN WG4 Meeting #114 	    R4-2501761

Athens, Greece, 17  – 21 February 2025

Agenda Item:	7.21.3

Source: 	Ericsson

Title:	On testability issues related to AI/ML for mobility

Document for:	Approval

# Introduction

RAN4#112bis discussed the testability aspects related to AI/ML for mobility. Issues discussed and the agreements reached are listed in the WF document [1].

# Discussion

## Testing setup

In the last meeting, setup for AI/ML based mobility testing was discussed and the following agreement was reached.

Agreement from RAN4#113 [1]

- RAN4 to further discuss:

  - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2

For AI/ML based mobility, two different types of scenarios are being considered. One of the scenarios refer to intra-cell scenario where the AI/ML model at UE takes the measurement from a cell to predict the RRM measurement for the same cell. For example, scenarios 2 and 4 agreed in RAN4#112bis meeting. The other type of scenario is inter-cell scenario, where the AI/ML model at UE takes measurement from one cell to predict RRM measurements for another cell. For example, scenario 3 agreed in RAN4#112bis meeting as one of the prioritized scenarios for RAN4 discussions.

Agreement from RAN4#112bis [2]

RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

### Testing for intra-cell scenarios

Intra-cell scenarios characterize deployment of AI/ML model at UE to predict the RRM measurements in temporal domain, where the measurements performed within observation window (OW) are used by the UE to perdict measurements valid for time instances inside the prediction window (PW) for the same cell as shown in Figure 1. The UE may report the measurements performed inside the OW and/or the measurements predicted to be valid for the instances inside the PW depending on the measurement prediction condition/configuration received from the network. The aim of testing for intra-cell scenarios should therefore be to validate that the measurements reported by the UE for a cell meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

From testing setup point of view, two-cell setup used to validate UE capability to meet requirements for the legacy L3-based mobility can be used to validate whether requirements for intra-cell predictions can be achieved when AI/ML model is deployed at the UE for the prediction of RRM measurements in temporal domain for mobility. The two-cell setup is needed to also consider the impact of interference coming from the cell other than the measured cell on temporal predictions during testing.

Intra-cell temporal prediction case-A.

Intra-cell temporal prediction case-B.

Figure 1. Measurement timeline for intra-cell temporal prediction scenarios.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

### Testing for inter-cell scenario

Inter-cell scenario characterizes deployment of AI/ML model to predict RRM measurement of a cell in one carrier frequency by exploiting the measurements performed by UE on cell in another carrier frequency in FR1. The aim of testing should therefore be to validate that the measurements reported by the UE

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

This scenario is similar to the inter-frequency measurement scenario supported for non-AI/ML mobility, where the UE is configured to perform and report measurement for two different cells that are in two different carrier frequencies. Test setup defined to validate requirements for inter-frequency measurements can therefore be re-used for testing AI/ML based mobility for inter-frequency scenario. While testing for AI/ML based mobility, cell 1 in the test setup can be considered as the cell for measurement and cell 2 in the test setup can be considered as the target cell for which the UE predicts the measurement for.

Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

## Accuracy definition for testing

For AI/ML based mobility testing, the following agreement has been reached for the definition of absolute accuracy of the predicted L3 RSRP measurement in the last meeting. The definition of ground truth of L3-RSRP for FR1 and FR2 is yet to be concluded.

Agreement from RAN4#113 [1]

- For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP

  - FFS Ground truth definition of L3-RSRP for FR1 and FR2

In RAN4#112bis meeting, baseline option for testing in FR1 was discussed. It was then agreed to consider conducted testing for testing AI/ML based mobility in FR1. In conducted testing, true value of the L3-RSRP measurement can be calculated that can be used as ground truth value to determine the accuracy of the predicted L3 RSRP value reported by the UE in the test. For testing in FR2, before discussing or agreeing on the definition of the accuracy of absolute L3 predicted value, baseline assumption for testing in FR2 needs to be agreed. In our view, for testing AI/ML based mobility in FR2 OTA testing should be considered. This approach is similar to what has been considered for BM use case in RAN4. The ground truth value of L3-RSRP measurement can then be a measurement performed by the UE under a sufficiently high SINR condition under the assumption that the resulting value is close to the true value and therefore is used testing the absoluted accuracy of the predicted L3 RSRP value in FR2 in the test.

Agreement from RAN4#112bis [2]

- RAN4 to consider Option 2 as a baseline option for testing in FR1.

  - Option 2: Conducted testing.

Proposal 5: For testing in FR1, ground truth of the predicted L3-RSRP value is calculated for the conducted testing setup in place for testing.

Proposal 6: Consider OTA testing as baseline option for testing in FR2.

Proposal 7: Ground truth value of L3-RSRP measurement in FR2 is the measurement performed by the UE under a sufficiently high side condition under the assumption that the resulting value is close to the true value. RAN4 to further discuss side condition to derive ground truth value for predicted L3-RSRP.

# Summary

In this contribution Ericsson’s view on testability issues related to AI/ML for mobility is presented. The proposals below summarize the discussion presented in this contribution.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup, similar to the test setup for legacy L3-based mobility.

Proposal 2: The aim of testing for intra-cell prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 3: The aim of testing for inter-cell scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 4: Existing test setup defined to validate requirements for inter-frequency measurements is re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered as the cell for measurement.

- Cell 2 in the test can be considered as the target cell for measurement prediction.

Proposal 5: For testing in FR1, ground truth of the predicted L3-RSRP value is calculated for the conducted testing setup in place for testing.

Proposal 6: Consider OTA testing as baseline option for testing in FR2.

Proposal 7: Ground truth value of L3-RSRP measurement in FR2 is the measurement performed by the UE under a sufficiently high side condition under the assumption that the resulting value is close to the true value. RAN4 to further discuss side condition to derive ground truth value for predicted L3-RSRP.

# References

- R4-2420095, WF on AI/ML Mobility, Moderator (Nokia).

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).