3GPP TSG-RAN WG4 Meeting #114	R4-2501788

# Athens, Greece, February 17th – 21st 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 SCM for demodulation

Agenda item:	7.16.2

Document for:	Information

# Introduction

In RAN4#113 meeting, companies agreed comparison simulation parameters for SU-MIMO PDSCH, SU-MIMO PMI and MU-MIMO PDSCH [1]. The impact of different antenna configurations and corresponding CSI-port mapping methods are also discussed.

In this contribution, we deliver some simulation results for SU-MIMO cases based on CDL approach to compare the performance curve under different configurations.

# Simulation

The results for SU-MIMO PDSCH and SU-MIMO PMI cases are delivered.

## 2.1	Parameters

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7DS2U |  |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |  |
| UE receiver type |  | IRC Baseline | IRC Baseline |
| Number of Tx Ports |  | 8,4 | 8 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |
| Number of layers |  | 8,4 | 4,2 |
| PMI |  | Random, Fixed PMI | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |
| Channel Models |  | The purpose of the study item. |  |
| Testing Metric |  | Full Throughput Curves |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | (+45, -45) |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |
|  | UE Antenna Polarisation | (0, 90) |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

### Following BS antenna configurations are used for comparison.

#### Antenna Array Virtualisation (AAV) using subarray configuration (SV)

One CSI-port is mapping to one polarization port per subarray.

- (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (128 AE)

- (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (128 AE)

- (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (64 AE)

- (M,N,P,Ms,Ns)  = (8,4,2,8,2) for 4Tx CSI-RS Ports (64 AE)

- New option for comparison: (M,N,P,Ms,Ns)  = (4,8,2,4,1) for 8Tx CSI-RS Ports (64 AE HorConf)

Note: All beams pointing to same direction.

#### Full connected (FC)

- (M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports (128 AE)

- (M,N,P) = (8,4,2) for 8Tx and 4Tx CSI-RS Ports (64 AE)

- New options: (M,N,P) = (4,8,2) for 8Tx and 4Tx CSI-RS Ports (64 AE)

#### No AAV no subarray:

- (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

- (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

#### Full connected with AAV with subarray (FC + AAV)

- (M,N,P,Ms,Ns) = (8,8,2,2,1) for 8Tx CSI-RS ports (128 AE 64TR Massive-MIMO)

### Following transmission methods are simulated for comparison:

Strongest beam directions for CDL model (based on FR1 Uma CDLC FR1 profile in 38.827):

- 1st strongest beam: AoD = -7.27o, ZoD = 100o.

- 2nd strongest beam: AoD = -21.82o, ZoD = 100o.

- 3rd strongest beam: AoD = -35.98o, ZoD = 100o.

- 4th strongest beam: AoD = -50.97o, ZoD = 100o.

Transmit different layers by different beams:

- 8 layers by all 4 strongest beams above

- 4 layers by 1st and 2nd strongest beams above

Transmit 8 layers or 4 layers by the 1st strongest beam direction above.

### PMI hypothesis for PMI statistic histogram

Precoding matrix index i=0, 1, 2, …, 15,  where i1,1=modi, 8,i1,2=0 and i2=floor(i/8).

| PMID | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i1,1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| i1,2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| i2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Note: The histogram is taken from the high normalized throughput range, i.e., >=70%. The PMI with highest probability in histogram will be used for “fixed PMI” simulations.

## 2.2	Simulation results for comparison

### 2.2.1	SU-MIMO PDSCH

#### 2.2.1.1  8Tx8Rx with 8 layers

BS antenna 128AE, FC vs SV

- Combined average throughput                                                  (b)  Per CW throughput

Figure 2.2.1.1-1 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, 128 AE full connection vs. subarray virtualization, random PMI (sub-band) vs. fixed PMI (wideband)

Figure 2.2.1.1-2 Histograms of PMI hypothesis of 8Tx8Rx with 8 layers, 128 AE full connection vs. subarray virtualization, FR1 Uma CDLC vs. TDLC300-100

Figure 2.2.1.1-3 Normalized PDSCH total throughput of 8Tx8Rx with 8 layers, 128 AE full connection, random PMI vs. fixed PMI vs. follow PMI (all with wideband precoder)

BS antenna 64AE, FC vs SV

- Combined average throughput                                                (b)  Per CW throughput

Figure 2.2.1.1-3 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, 64 AE full connection vs. subarray virtualization, random PMI vs. fixed PMI

Figure 2.2.1.1-4 Histograms of PMI hypothesis of 8Tx8Rx with 8 layers, 64 AE full connection vs. subarray virtualization, FR1 Uma CDLC

BS antenna 8AE, no SV

- Combined average throughput                                             (b) Per CW throughput

Figure 2.2.1.1-5 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, 8 AE, random PMI vs. fixed PMI

Figure 2.2.1.1-6 Histograms of PMI hypothesis of 8Tx8Rx with 8 layers, 8AE, FR1 Uma CDLC vs. TDLC300-100

Table 2.2.1.1-1 Results summary of preferred PMI

| BS antenna Config. | Port mapping | Channel model | Preferred top 4 PMI |
| --- | --- | --- | --- |
| 128AE | FC | FR1 Uma CDLC | 2, 6, 10, 14 |
|  |  | TDLC300-100 | 3, 7, 11, 15 |
|  | SV | FR1 Uma CDLC | 2, 6, 10, 14 |
|  |  | TDLC300-100 | 8, 9, 11, 12 |
| 64AE | FC | FR1 Uma CDLC | 0, 3, 11, 15 |
|  | SV | FR1 Uma CDLC | 1, 3, 9, 11 |
| 8AE | 1 to 1 | FR1 Uma CDLC | 1, 3, 9, 11 |
|  |  | TDLC300-100 | 9, 10, 11, 13 |

Table 2.2.1.1-2 Results summary of SNR at 70% normalized throughput

| BS antenna Config. | Port mapping | Channel model | PMI configuration | SNR at 70% NThp. [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | CW0 | CW1 | Combined |
| 128AE | FC | FR1 Uma CDLC | Random | 20.5 | 15 | 17.5 |
|  |  |  | Fixed | 27 | 7.5 | 25.1 |
|  | SV |  | Random | 14 | 22.5 | 20.7 |
|  |  |  | Fixed | 0.5 | 28.3 | 26.5 |
| 64AE | FC | FR1 Uma CDLC | Random | NA | 17 | NA |
|  |  |  | Fixed | NA | 1 | NA |
|  | SV |  | Random | 27 | 27.5 | 27.2 |
|  |  |  | Fixed | 17.3 | 31.7 | 30.5 |
| 8AE | 1 to 1 | FR1 Uma CDLC | Random | 33.2 | 33.8 | 33.5 |
|  |  |  | Fixed | 25 | 37.5 | 36.5 |

#### 2.2.1.2	4Tx4Rx with 4 layers

BS antenna 128AE, FC vs SV

Figure 2.2.1.2-1 Normalized PDSCH throughput of 4Tx4Rx with 4 layers, 128 AE full connection vs. subarray virtualization, random PMI vs. fixed PMI, FR1 Uma CDLC

Figure 2.2.1.2-2 Histograms of PMI hypothesis of 4Tx4Rx with 4 layers, 128 AE full connection vs. subarray virtualization, FR1 Uma CDLC

BS antenna 64AE, FC vs SV

Figure 2.2.1.2-3 Normalized PDSCH throughput of 4Tx4Rx with 4 layers, 64 AE full connection vs. subarray virtualization, random PMI vs. fixed PMI, FR1 Uma CDLC

Figure 2.2.1.2-4 Histograms of PMI hypothesis of 4Tx4Rx with 4 layers, 64 AE full connection vs. subarray virtualization, FR1 Uma CDLC

BS antenna 4AE, no SV

Figure 2.2.1.2-5 Normalized PDSCH throughput of 4Tx4Rx with 4 layers, 4 AE, random PMI vs. fixed PMI, FR1 Uma CDLC

Figure 2.2.1.2-6 Histograms of PMI hypothesis of 4Tx4Rx with 4 layers, 8 AE, FR1 Uma CDLC vs. TDLC300-100

Table 2.2.1.2-1 Results summary of preferred PMI

| BS antenna Config. | Port mapping | Channel model | Preferred top 4 PMI |
| --- | --- | --- | --- |
| 128AE | FC | FR1 Uma CDLC | 0, 1, 2, 3 |
|  | SV |  | 1, 2, 9, 10 |
| 64AE | FC | FR1 Uma CDLC | 3, 7, 11, 15 |
|  | SV |  | 1, 2, 9, 10 |
| 4AE | 1 to 1 | FR1 Uma CDLC | 1, 5, 9, 13 |
|  |  | TDLC300-100 | 0, 1, 3, 8 |

Table 2.2.1.2-2 Results summary of SNR at 70% normalized throughput

| BS antenna Config. | Port mapping | Channel model | PMI configuration | SNR at 70% NThp. [dB] |
| --- | --- | --- | --- | --- |
| 128AE | FC | FR1 Uma CDLC | Random | -1.2 |
|  |  |  | Fixed | -2 |
|  | SV |  | Random | 1.7 |
|  |  |  | Fixed | -0.1 |
| 64AE | FC | FR1 Uma CDLC | Random | 6.5 |
|  |  |  | Fixed | 3.2 |
|  | SV |  | Random | 3.9 |
|  |  |  | Fixed | 2.5 |
| 4AE | 1 to 1 | FR1 Uma CDLC | Random | 18.3 |
|  |  |  | Fixed | 16.8 |

### 2.2.2 	SU-MIMO PMI

#### 2.2.2.1	Release 15 Type-I codebook

BS antenna 128AE, FC vs SV, 4 layers and 2 layers

- 4 layers                                                                                             (b)  2 layers

Figure 2.2.2 -1 Normalized PDSCH throughput of 8Tx4Rx, 128 AE full connection vs. subarray virtualization, random PMI vs. follow PMI, FR1 Uma CDLC

BS antenna 64AE, FC vs SV, 4 layers and 2 layers

(a) 4 layers                                                                                               (b) 2 layers

Figure 2.2.2-2 Normalized PDSCH throughput of 8Tx4Rx, 64 AE full connection vs. subarray virtualization, random PMI vs. follow PMI, FR1 Uma CDLC

BS antenna 8AE, no SV, 4 layers and 2 layers

(a)   4 layers FR1 Uma CDLC                                                                            (b)  4 layers TDLC300-100

(c)  2 layers FR1 Uma CDLC                                                                    (d) 2 layers TDLC300-100

Figure 2.2.2-3 Normalized PDSCH throughput of 8Tx4Rx, 8 AE, random PMI vs. follow PMI

Table 2.2.2-1 Results summary of PMI results for TypeI codebook

| BS antenna Config. | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp. [dB] | NThp. of Random PMI | Gama |
| --- | --- | --- | --- | --- | --- | --- |
| 128AE | 4 | FC | FR1 Uma CDLC | 5 | 77% | 1.16 |
|  |  | SV |  | 1.8 | 52% | 1.73 |
|  | 2 | FC |  | -4.5 | 78% | 1.15 |
|  |  | SV |  | -5.5 | 47% | 1.91 |
| 64AE | 4 | FC | FR1 Uma CDLC | 7.5 | 62% | 1.45 |
|  |  | SV |  | 6.2 | 50% | 1.8 |
|  | 2 | FC |  | -3.9 | 62% | 1.45 |
|  |  | SV |  | -5.6 | 35% | 2.57 |
| 8AE | 4 | 1 to 1 | FR1 Uma CDLC | 14 | 48% | 1.88 |
|  |  |  | TDLC300-100 | 18 | 54% | 1.67 |
|  | 2 |  | FR1 Uma CDLC | 2.5 | 34% | 2.65 |
|  |  |  | TDLC300-100 | 2.5 | 34% | 2.65 |

#### 2.2.2.2	Comparison between Type-I and enhanced Type-II (eType-II) codebooks with wideband PMI precoder granularity

BS antenna 128AE, 4 layers and 2 layers: FC vs SV

- 128 AE FC                                                                                             (b)  128 AE SV

Figure 2.2.2 -1 Normalized PDSCH throughput of 8Tx4Rx, 128 AE full connection (a) vs. subarray virtualization (b), with wideband random Type-I PMI and follow PMI Type I and eType-II codebooks, using an FR1 Uma CDLC model

BS antenna 64AE, 4 layers and 2 layers: FC vs SV

(a) 64 AE FC                                                                                               (b) 64 AE SV

Figure 2.2.2-2 Normalized PDSCH throughput of 8Tx4Rx, 64 AE full connection (a) vs. subarray virtualization (b), with wideband random Type-I PMI and follow PMI Type I and eType-II codebooks, using an FR1 Uma CDLC model

BS antenna 8AE, 4 layers and 2 layers: CDLC vs TDLC

(a)   FR1 Uma CDLC                                                                            (b)  FR1 Uma TDLC300-100

Figure 2.2.2-3 Normalized PDSCH throughput of 8Tx4Rx, 8 AE, wideband random Type-I PMI and follow PMI Type I and eType-II codebooks

Table 2.2.2.2-1 Results summary of PMI results using wideband eType-II codebook.

| BS antenna Config. |  | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp | NThp. of Random PMI | Gamma |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128AE | 4 |  | FC | FR1 Uma CDLC | 4.99 | 78% | 1.15 |
|  |  |  | SV |  | 2.84 | 57% | 1.57 |
|  | 2 |  | FC |  | -8.58 | 47% | 1.91 |
|  |  |  | SV |  | -8.99 | 36% | 2.5 |
| 64AE | 4 |  | FC | FR1 Uma CDLC | 6.59 | 59% | 1.52 |
|  |  |  | SV |  | 6.59 | 52% | 1.73 |
|  | 2 |  | FC |  | -6.07 | 50% | 1.8 |
|  |  |  | SV |  | -6.48 | 31% | 2.9 |
| 8AE | 4 |  | 1 to 1 | FR1 Uma CDLC | 14.98 | 52% | 1.73 |
|  |  |  |  | TDLC300-100 | 19.18 | 56% | 1.6 |
|  | 2 |  |  | FR1 Uma CDLC | 2.77 | 36% | 2.5 |
|  |  |  |  | TDLC300-100 | 2.33 | 34% | 2.64 |

#### 2.2.2.3	Comparison between Type-I and enhanced Type-II (eType-II) codebooks with sub-band PMI precoder granularity

BS antenna 128AE, 4 layers and 2 layers: FC vs SV

- 128 AE FC                                                                                             (b)  128 AE SV

Figure 2.2.2 -1 Normalized PDSCH throughput of 8Tx4Rx, 128 AE full connection (a) vs. subarray virtualization (b), with sub-band random Type-I PMI and follow PMI Type I and eType-II codebooks, using an FR1 Uma CDLC model

BS antenna 64AE, 4 layers and 2 layers: FC vs SV

(a) 64 AE FC                                                                                               (b) 64 AE SV

Figure 2.2.2-2 Normalized PDSCH throughput of 8Tx4Rx, 64 AE full connection (a) vs. subarray virtualization (b), with sub-band random Type-I PMI and follow PMI Type I and eType-II codebooks, using an FR1 Uma CDLC model

BS antenna 8AE, 4 layers and 2 layers: CDLC vs TDLC

(a)   FR1 Uma CDLC                                                                            (b)  FR1 Uma TDLC300-100

Figure 2.2.2-3 Normalized PDSCH throughput of 8Tx4Rx, 8 AE, sub-band random Type-I PMI and follow PMI Type I and eType-II codebooks

Table 2.2.2.3-1 Results summary of PMI results using sub-band eType-II codebook.

| BS antenna Config. | Rank | CSI Port mapping | Channel model | SNR of Follow PMI at 90% NThp | NThp. of Random PMI | Gamma |
| --- | --- | --- | --- | --- | --- | --- |
| 128AE | 4 | FC | FR1 Uma CDLC | 1.92 | 59% | 1.52 |
|  |  | SV |  | 0.59 | 48% | 1.875 |
|  | 2 | FC |  | -9.33 | 43% | 2.09 |
|  |  | SV |  | -9.41 | 33% | 2.72 |
| 64AE | 4 | FC | FR1 Uma CDLC | 5.01 | 53% | 1.69 |
|  |  | SV |  | 4.56 | 46% | 1.95 |
|  | 2 | FC |  | -6.26 | 49% | 1.83 |
|  |  | SV |  | -6.74 | 29% | 3.1 |
| 8AE | 4 | 1 to 1 | FR1 Uma CDLC | 12.87 | 46% | 1.95 |
|  |  |  | TDLC300-100 | 16.54 | 52% | 1.73 |
|  | 2 |  | FR1 Uma CDLC | 1.56 | 30% | 3 |
|  |  |  | TDLC300-100 | 2.33 | 34% | 2.64 |

## 2.3	Simulation results for further investigation

### 2.3.1	BS antenna array impact

(a) 128 AE FC                         (b) 64AE FC                   (c) 64AE HC FC        	      (d) 128AE 64SV FC

Figure 2.3.1-1 BS antenna array configuration

- Combined average throughput                                                              (b) per CW throughput

Figure 2.3.1-2 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, full connection

### 2.3.2	Doppler impact

(a) Total throughput                                                         (b) Per CW throughput

Figure 2.3.2-1 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, 128 AE full connection, random PMI, UE speed 30km/h vs. 3km/h

### 2.3.3	Tx beam direction impact

(a) Total throughput                                                         (b) Per CW throughput

Figure 2.3.3-1 Normalized PDSCH throughput of 8Tx8Rx with 8 layers, 128 AE [Me, Ne, P] = [8, 8, 2], random PMI, full connection with different directions per layer vs. subarray virtualization ([Ms, Ns] = [8, 1]) with same direction for all layers vs. full connection with same direction for all layers

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2419782, Way Forward for [113][320] FS_NR_demod_SCM, Nokia

[2]