3GPP TSG-RAN WG4 Meeting # 114							R4-2501798

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.5.2.1

Source: 	OPPO

Title: 	Non-col R4-25xxxxx UE RF requirements for type 4

Document for:	Approval

- Introduction

During the RAN4#113 meeting, how to capture the UE RF requirement have been extensively discussed. The specific topic on whether to capture the UE capability type definition for EN-DC and NR-CA has been fully discussed. In this paper we try to give further discussion on this topic.

- Discussion

The WF agreement from [1] is captured as below.

<Issue 1-1:  UE Capability Type definition for EN-DC and NR-CA>

Way forward:

- Continue further discussion on following options and conclude it in the next meeting, so that RAN4 can begin Rel-19 38.101-1/101-3 CR work in April meeting.

- Proponents of Option 2 need to propose specific examples in Feb. meeting.

- Option 1:

Add the table into TS 38.101-3: 7.10B	Power imbalance for EN-DC and 38.101-1: 7.10A	Power imbalance for NR-CA.

- Option 2:

Not adding the table in 38.101-1/3 and refer to 38.331 and its sections.

<Issue 2-1-1:  How to capture the power imbalance and REFSENS requirements for Big CR>

Way forward:

- In the previous meeting RAN4 agreed no rush to make decision on Rel-19 CR. Discuss it as Rel-19 big CR.

- The CR leader take companies’ inputs of R4#112-bis meeting into account when drafting actually.

<Issue 2-1-2:  Whether to introduce term “Type 2” for pre Rel-19 specifications>

Way forward:

- Wait for the conclusion of Issue 1-1: UE Capability Type definition for EN-DC and NR-CA.

For issue 1-1, we have explained our opinion during the last meeting online discussion. From our opinion, basically we don’t want to introduce such UE capability type definition into the UE RF requirement specification. The reason is quite simple, we have other similar situations when referring the different UE capability which is defined in TS 38.306 or used in TS 38.331. Once we agree to introduce the UE type under this WID, the other parts will also be influenced, let alone same principle tries to be also applied to RRM specification. So, we really need to be very careful on the decision. However, if this is the main group understanding, we don’t have technical concern on this issue and can follow the group decision.

Observation 1: The agreement on introducing UE types will also impact other part of UE RF specification and also the RRM specification.

If we have to agree on the introduction of the UE type definition, we have concern on the exact wording of the definition. During the last meeting discussion, following wording is introduced by some of the company as:

Table 1: EN-DC power imbalance UE capability types

| [UE Capability Type] | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in caluse 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | TBD |

Add the following to TS 38-101-1:

Table 2: NR CA power imbalance UE capability types

| [UE capability types] | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | TBD |

The parts we have concern has been marked as yellow.

Firstly, it is hardly to say a UE type “supports intra-band RF requirements”. Usually we say a UE fulfil the specific UE RF requirement.

Secondly, it is kind of confusing what is the meaning of intra-band RF requirement. Maybe wording like “UE RF requirements for intra-band CA is more appropriate”.

Last but not least, the 6dB power imbalance is not a UE RF requirement but a pre-requisite for intra-band non-contiguous CA deployment, so using a “()” to introduce the power imbalance after the UE RF requirement will make the confusion that this is a specific UE RF requirement which for sure is not the intention. During the last meeting online discussion, we have proposed that to use a note as similar to the LTE specification to introduce this 6dB power imbalance and this could be more acceptable for us.

So even if we can accept to add such UE type definition, we think still we might need to be more careful about the exact wording of the type definition.

Proposal: If the group agrees on introducing the UE types into specification, the wording needs more refinement.

3	Conclusions

In this paper, we give discussion on the UE type definition, the observations and proposals are as below:

Observation 1: The agreement on introducing UE types will also impact other part of UE RF specification and also the RRM specification.

Proposal: If the group agrees on introducing the UE types into specification, the wording needs more refinement.

4 References

[1] R4-2420399	WF on non-collocated intra-band EN-DC and NR-CA, KDDI