3GPP TSG-RAN WG4 Meeting # 114							R4-2501800

Athens, GR, 17th – 21st February, 2025

Agenda Item:	10

Source: 	OPPO

Title: 	Discussion on non-zero switching time for SUL and NUL

Document for:	Approval

- Introduction

The zero-switching time for SUL and NUL has been introduced since Rel-15 when the first SUL combination SUL_n78_n80 was introduced at that time. However, during the whole 5G development, the 2/4 RX are deemed mandatory for different bands, the number of TX chains has not been decided and left for UE implementation. In such case, we would like to discuss the possibility of introducing the non-zero switching time for SUL and NUL.

- Discussion

During the Rel-15 discussion, when the SUL_n78_n80 was introduced, different implementation has been discussed as shown below which is captured from [1]:

Two different example UE architectures are described as below and illustrated in Figure 1.

- The two bands have separate transceivers, PLLs, PAs and antennas

- The two bands have separate transceivers, PLLs, PAs but share antennas. Duplxer/Triplxer can be used to combine the two PAs.

Figure 1 Simplified UE architectures for different implementations

For Implementation 1, the switching time between UL and SUL can be zero because the UE can prepare the RF chain in advance separately. Considering some implementation may still require some small switching time such as to response the control signalling for the hardware components, almost 0us is proposed.

- ~0us

For Implementation 2a, the RF switching time is the same as Implementation 1 because there is no extra time required by the hardware.

- ~0us

Based on analysis above, the switching time between SUL and UL is almost 0us.

Although there is two implementations included in the discussion paper, the only difference is whether shared antenna or separate antenna is used. Two TX chains including different LO, mixer and PA are well prepared simultaneously and hence the 0us switching time between SUL and NUL has been proposed and finally agreed.

Observation 1: For SUL band combination, the 0us switching time between SUL and NUL has been agreed based on the two different TX chain assumption.

When it comes to the DC combination which includes SUL combination, the switching time is set up to 140us as captured in TS 38.101-3 which is captured as below:

This has been discussed also in early Rel-15 and the corresponding discussion is captured as below for information from [2].

However, except some two bands combinations which can share the same agreement of 0us switching time, some three bands combinations were also introduced, including:

- DC_3_SUL_n78-n82

- DC_20_SUL_n78-n83

For three bands combinations, some implementations only have two PLLs, which means NR non-SUL and SUL should share one PLL, as shown in Figure 2. Therefore, switching between non-SUL and SUL needs to spend ~130us for PLL retuning for this kind of implementation which is 2 symbols for 15kHz SCS and 4 symbols for 30kHz SCS.

Figure 2 Simplified UE architectures for three bands combination

The PLL retuning time has been considered when there is three bands with three different Pas while only 2 LOs are implemented.

Observation 2: For three bands DC combinations with SUL band combinations included, 140us switching is considered between SUL and NUL.

With the above observation 1 and 2, at least we see that non-zero switching time due to implementation has been introduced. At this development stage of 5G, different implementation compared to the year of 2018 at the early stage has been investigated. From specification perspective, there is no such requirement that for a UE to support SUL band combination that it has to support two simultaneously workable RF chain. However, current 0 us switching time has limited the UE implementation for UE to support such feature.

Observation 3: Current 0us switching time for SUL band combination has limited the UE implementation.

Also, to fulfil the 0 us switching time, even when there is only on TX of both SUL and NUL is transmitting, the other one needs to transmit at the same time with full power. Otherwise some switching time is needed. This is quite power consuming for such PA working at its highest power mode but transmitting nothing.

Observation 4: The 0 us switching between SUL and NUL needs one chain to be fully standby when the other chain is transmitting, which is quite power consuming.

Based on the above observation 3 and 4, it is proposed that the switching time between SUL and NUL to be investigate in RAN4 so that the two TX chain can be switched with some time and more flexible UE implementation and power saving can be reached.

Proposal: To study the switching time between SUL and NUL in SUL band combination in RAN4.

3	Conclusions

In this paper, we give discussion on the 0us switching time for SUL combination and non-zero switching time for DC combination with SUL combination, the observations and proposals are as below:

Observation 1: For SUL band combination, the 0us switching time between SUL and NUL has been agreed based on the two different TX chain assumption.

Observation 2: For three bands DC combinations with SUL band combinations included, 140us switching is considered between SUL and NUL.

Observation 3: Current 0us switching time for SUL band combination has limited the UE implementation.

Observation 4: The 0 us switching between SUL and NUL needs one chain to be fully standby when the other chain is transmitting, which is quite power consuming.

Proposal: To study the switching time between SUL and NUL in SUL band combination in RAN4.

4 References

[1] R4-1800475	On switching time between UL and SUL, Huawei, HiSilicon

[2] R4-1803892	Switching time for SUL and non-SUL for DC with three bands combinations, Huawei, HiSilicon