3GPP TSG-RAN WG4 Meeting #114                                      R4-2501802

Athens, Greece, February 17th – February 21st, 2025

Source: 	Huawei, HiSilicon

Title: 	On signalling aspect of fragmented carriers

Agenda Item:	7.6.2

Document for:	Discussion

Introduction

The SI of NR FR1 DL Fragmented Carriers was approved at RAN#104. During previous RAN4 meetings, the discussion of signalling aspect was deferred until better understanding of the RF performance is achieved. For this meeting, we provided the initial performance evaluation in [6]. In this document, we would like to discuss the signalling aspect accordingly.

Discussion

Triggering condition

At RAN4#113 meeting, the triggering condition to enable “One RF Rx chain” was discussed. The absolute power level of in-gap interferer and/or PSD difference between interferer and target CC are identified as potential triggering conditions. No conclusion was made since better understanding of the RF performance is needed before deciding the triggering condition.

In this meeting we provided some RF performance evaluation results in [6]. Based on the evaluation results, we have the observation that the relaxation required for “One RF Rx chain” is <10dB for REFSENS and <2dB for ACS/IBB. This amount of relaxation is at the same level of many MSD exceptions of normal CA configurations. We think it’s possible that RAN4 handles the “One RF Rx chain” in the same way of normal CA configurations, i.e. no special triggering condition is needed.

Proposal 1: The “One RF Rx chain” is handled in the same way of normal CA configurations. No special triggering condition is needed.

Fallback scheme

The fallback scheme was also discussed in previous meetings. Some potential options were listed in the topic summary [3] as below.

From CA configuration point of view, the two fallback options are:

- CA with Single Rx RF Chain --> Non CA. E.g, CA_nX(2A)-nYA --> CA_nXA-nYA

- CA with Single Rx RF Chain --> CA with Separate Rx RF Chain. E.g, CA_nX(2A)-nYA --> CA_nX(2A)

The option a) is a simple solution from both UE aspect and Network aspect. A UE supporting CA with Single Rx RF chain will undoubtedly support the non-CA configuration on the same band. From Network perspective, if the performance of a component carrier is not meeting the expectation, it’s also natural choice to deactivate/release this component carrier.

The option b), on the other hand will require proper coordination between the UE and the Network. The switching from Single Rx RF Chain to Separate Rx RF Chain requires additional RF resource, which is dependent on UE’s hardware ability. For example, the initial CA configuration is CA_nX(2A)-nY(2A) with Single Rx RF Chain on both bands (Conf. 1 in Table 2.2-1). The UE might be able to fall back to Separate Rx RF Chain on only one band due to the hardware limitation, which means the possible fallback configurations are Conf. 2 or Conf. 3 in the same table, but the Conf. 4 couldn’t be supported by the UE. This needs to be clearly communicated to the Network.

Table 2.2-1 The possible fallback configurations of CA_nX(2A)-nY(2A)

|  | nX(2A) | nY(2A) | Support status |
| --- | --- | --- | --- |
| Conf. 1 | Single | Single | √ |
| Conf. 2 | Single | Separate | √ |
| Conf. 3 | Separate | Single | √ |
| Conf. 4 | Separate | Separate | X |

From Network perspective, the option b) also requires more complex implementation and the signalling adaptation. The Network needs to first decide the better configuration between the Non-CA and CA with Separate Rx RF Chain. To enable the switching between Single Rx RF Chain and Separate Rx RF Chain, a new signalling scheme is also required to explicitly indicating the RF architecture.

Observation 1: CA with Single Rx RF Chain falling back to Non-CA is a simple solution from both UE aspect and Network aspect.

Observation 2: CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain requires UE fallback capability reporting, new CA configuration signalling with indication of RF architecture and Network implementation update.

Proposal 2: Prioritizing the option of CA with Single Rx RF Chain falling back to Non-CA.

UE capability reporting

Based on the RF performance evaluation, the UE’s Rx requirements are different for Single Rx RF Chain architecture and Separate Rx RF Chain architecture. Considering some higher order CA configurations could only be supported with Single Rx RF Chain, the conformance testing needs to follow the updated RF requirements accordingly. A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

Proposal 3: A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

Another UE capability signalling for the fallback scheme also needs to be considered. As illustrated in section 2.2, the capability of switching between Single Rx RF Chain and Separate Rx RF Chain needs to be explicitly reported by the UE. The clearest way would be to report the CA configuration with different RF architectures separately.

Proposal 4: In UE capability signalling, the same CA configuration with Single Rx RF Chain and with Separate Rx RF Chain are reported separately.

To easily distinguish the different RF architecture, a new notation for CA configuration as suggested by [4] and [5] could be also considered. The letter ‘F’ is suggested in both contributions, but we would like to suggest using a different letter since ‘F’ is being used in TS 38.101-2 and TS 36.101 to represent a contiguous CA bandwidth class. Taking the FR2 bandwidth classes ‘R2~R12’ as a reference, maybe we could use a similar way to represent the CA with Single Rx RF Chain, such as ‘Z2~Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

Proposal 5: Using a new notation for CA configurations with Single Rx RF Chain, such as ‘Z2’~’Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

Potential measurement and report

In previous meetings, the measurement of in-gap interference was discussed. From our point of view, the measurement of interference is not necessary since we didn’t identify any critical performance issue [6].

Proposal 6: Measurement of in-gap interference is not introduced.

If the in-gap interference measurement is identified as needed based on more performance evaluations, we prefer not introducing too complex measurement process. As a starting point, the existing inter-frequency measurement process could be reused.

Proposal 7: Consider reusing the existing inter-frequency measurement process if the in-gap interference measurement is identified as needed.

Conclusion

In this contribution we provide some consideration on the signalling aspect of fragmented carriers. According to the analysis, we have the following proposals:

Observation 1: CA with Single Rx RF Chain falling back to Non-CA is a simple solution from both UE aspect and Network aspect.

Observation 2: CA with Single Rx RF Chain falling back to CA with Separate Rx RF Chain requires UE fallback capability reporting, new CA configuration signalling with indication of RF architecture and Network implementation update.

Proposal 2: Prioritizing the option of CA with Single Rx RF Chain falling back to Non-CA.

Proposal 3: A new UE capability signalling is needed to indicate whether Single Rx RF Chain is supported for a specific CA configuration at least for the testing purpose.

Proposal 4: In UE capability signalling, the same CA configuration with Single Rx RF Chain and with Separate Rx RF Chain are reported separately.

Proposal 5: Using a new note for CA configurations with Single Rx RF Chain, such as ‘Z2~Z6’, where ‘Z2’ means 2 non-contiguous CCs received by a single Rx RF Chain.

Proposal 6: Measurement of in-gap interference is not introduced.

Proposal 7: Consider reusing the existing inter-frequency measurement process if the in-gap interference measurement is identified as needed.

- References

- RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”, MediaTek Inc, RAN#104 Meeting, June 2024

- R4-2420519, “WF on NR FR1 DL fragmented carriers study”, MediaTek Inc, RAN#113 Meeting, November 2024

- R4-2418146, “Topic summary for [113][124] FS_NR_FR1_DL_Frag_Carrier”, MediaTek, RAN4#113 Meeting, November 2024

- R4-2418531, “On methods for reducing the number of UE Rx chains for fragmented carriers”, Apple, RAN4#113 meeting, November 2024

- R4-2419454, “Discussion on Fragmented Carrier reduction of Rx RF chains”, Nokia, RAN4#113 meeting, November 2024

- R4-2501803, “On RF requirements of fragmented carriers”, Huawei, HiSilicon, RAN4#114 meeting, February 2025