3GPP TSG-RAN4 Meeting #114	R4-2501807

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on coexistence results and conclusion

Source:	Samsung

Agenda item:	7.10.2.1

Document for:	Approval

1	Introduction

In the RAN4#113 meeting, there were 8 companies provided their co-existence results which were captured in R4-2420406 [1]. Based on the agreed principle of results processing and discussion, the initial findings were agreed and captured in R4-2420403 [2].

In this paper, we will further discuss the co-existence results, and propose to finalize the co-existence discussion with conclusion agreed in this meeting.

2	Results discussion and summary

Based on the results collected in [1], the following processes were made to derive the average results:

For NR-NTN PC2:

| NR-NTN PC2 |  | Required ACIR (by interpolate) | Average |
| --- | --- | --- | --- |
| With Isolation distance 1.5km | Cell average | All companies indicate <20 (Samsung, vivo, Ericsson, ZTE, MTK ,Thales, Huawei) | Samsung, vivo, Ericsson, MTK, Thales: <20 ZTE: 30.6 (ACLR) Huawei: 23.7 (ACLR) Average: < 22 |
|  | Cell edge | Samsung, vivo, Ericsson, MTK, Thales: <20 ZTE: 30.6 (ACLR) Huawei: 23.7 (ACLR) |  |
| Without Isolation distance, i.e. 0 km | Cell average | Samsung, vivo < 20 | Average: 26.5 |
|  | Cell edge | Samsung: 25.9 --> 25.9 (ACLR) Vivo: 27.1 (ACLR) |  |

For NR-NTN PC1.5:

| NR-NTN PC1.5 |  | Required ACIR (by interpolate) | Average |
| --- | --- | --- | --- |
| With Isolation distance 1.5km | Cell average | Samsung, vivo, Ericsson Thales, Huawei: <20 ZTE: 21.4 (ACLR) MTK: 22.9 （ACLR） | Samsung, vivo, Ericsson, Thales: <20 ZTE: 31.4 (ACLR) MTK: 22.9 (ACLR) Huawei: 26.2 (ACLR) Average: <22.9 |
|  | Cell edge | Samsung, vivo, Ericsson, Thales: <20 ZTE: 31.4 (ACLR) MTK: 22.9 (ACLR) Huawei: 26.2 (ACLR) |  |
| Without Isolation distance, i.e. 0 km | Cell average | Samsung, vivo < 20 | Average: 30.2 |
|  | Cell edge | Samsung: 29.2 Vivo: 31.2 (ACLR) |  |

For NR-NTN PC 1:

| NR-NTN PC1 |  | Required ACIR (by interpolate) | Average |
| --- | --- | --- | --- |
| With Isolation distance 1.5km | Cell average | Samsung, vivo, Ericsson Thales, Huawei: <20 ZTE: 23.5 (ACLR) MTK: 24.8 （ACLR） | Samsung, Ericsson, Thales: <20 Vivo: 20.5 (ACLR) ZTE: 33.7 (ACLR) MTK: 24.8（ACLR） Huawei: 27.2 (ACLR) Average: < 23.7 |
|  | Cell edge | Samsung, Ericsson, Thales: <20 Vivo: 20.5 (ACLR) ZTE: 33.7 (ACLR) MTK: 22.9 (ACLR) Huawei: 27.2 (ACLR) |  |
| Without Isolation distance, i.e. 0 km | Cell average | Samsung, vivo, Qualcomm < 20 | Average: 28.3 (w. QC) 32.7 (w/o. QC) |
|  | Cell edge | Samsung: 30.8 Vivo: 34.5 (ACLR) Qualcomm: 19.6 |  |

For IoT-NTN PC2:

| IoT-NTN PC2 |  | Required ACIR (by interpolate) | Average |
| --- | --- | --- | --- |
| With Isolation distance 1.5km | Cell average | Samsung, vivo(ACLR), Ericsson: <20 ZTE: 24.5 (ACLR) | Samsung, vivo(ACLR), Ericsson: <20 ZTE: 24.5 (ACLR) Average: < 21.1 |
|  | Cell edge | Samsung, vivo(ACLR), Ericsson: <20 ZTE: 39.2 (ACLR) |  |
| Without Isolation distance, i.e. 0 km | Cell average | Samsung: 20.5 Vivo: <20 (ACLR) | Average: <20.3 |
|  | Cell edge | Samsung: 34.9 Vivo: <20 (ACLR) |  |

For IoT-NTN PC1:

| IoT-NTN PC1.5 |  | Required ACIR (by interpolate) | Average |
| --- | --- | --- | --- |
| With Isolation distance 1.5km | Cell average | Samsung, vivo, Ericsson: <20 ZTE: 27.4 (ACLR) | Samsung, vivo(ACLR), Ericsson: <20 ZTE: 27.4 (ACLR) Average: < 21.9 |
|  | Cell edge | Samsung, vivo, Ericsson: <20 ZTE: 41.8 (ACLR) |  |
| Without Isolation distance, i.e. 0 km | Cell average | Vivo, Qualcomm: <20 Samsung: 24.2 | Samsung: 24.2 vivo: <20 (ACLR) Qualcomm: 28.4 Average: <24.2 |
|  | Cell edge | Samsung: 38.9 vivo: <20 (ACLR) Qualcomm: 28.4 |  |

And based on the summary above, the WF[2] had captured agreed initial conclusions as follows:

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective: For NR-NTN PC2: - Less than 22 dB with 1.5km isolation distance assumed (baseline assumption) - [26.5 dB] with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1.5: - Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption) - [30.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1: - Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption) - [28.3/32.7] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC2: - Less than 21.1 dB with 1.5km isolation distance assumed; - Less than [20.3] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC1: - Less than 21.9 dB with 1.5km isolation distance assumed; - Less than [24.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case

With above, we have the following observations for NR-NTN HPUE:

Observation 1: With 1.5km isolation distances assumed, the NR-NTN HPUE co-existence results indicated a much more relaxed required ACLR for PC2, PC1.5 and PC1, which is less than 24dB, compared to the existing NR-NTN PC3 ACLR requirement, 30dBc.

Observation 2: The existing NR-NTN PC3 ACLR requirement is inherited from NR TN PC3 legacy ACLR requirement, which was not directly determined by Rel-17 NR-NTN co-existence results.

Observation 3: Even with 0km isolation distance assumed, the existing NR-NTN PC3 ACLR (30dB) can be sufficient for at least both PC2 and PC1.5 NR-NTN HPUE.

And given no updated results were received during the offline between two meetings, we propose to confirm the following findings, from co-existence study perspective:

Proposal 1: Based on Observation 1~3, we propose to conclude from co-existence study perspective, the PC3 ACLR (30dB) is sufficient for NR-NTN PC2 and PC1.5 HPUE (regardless of isolation distance assumptions).

Proposal 2: To conclude if PC3 ACLR (30dB) is sufficient for NR-NTN PC1, by taking into account any results updates and further discussions in this meeting, and also different isolation distance assumptions.

Moreover, the results for IoT NTN HPUE seems more stable, and we propose to conclude the PC3 ACLR requirements can be sufficient for IoT NTN HPUE for up to PC1.

Proposal 3: To conclude from coexistence study perspective, PC3 ACLR is sufficient for IoT NTN HPUE with power classes up to PC1.

3	Conclusion

For the NTN HPUE co-existence study, we propose the following:

Observation 1: With 1.5km isolation distances assumed, the NR-NTN HPUE co-existence results indicated a much more relaxed required ACLR for PC2, PC1.5 and PC1, which is less than 24dB, compared to the existing NR-NTN PC3 ACLR requirement, 30dBc.

Observation 2: The existing NR-NTN PC3 ACLR requirement is inherited from NR TN PC3 legacy ACLR requirement, which was not directly determined by Rel-17 NR-NTN co-existence results.

Observation 3: Even with 0km isolation distance assumed, the existing NR-NTN PC3 ACLR (30dB) can be sufficient for at least both PC2 and PC1.5 NR-NTN HPUE.

Proposal 1: Based on Observation 1~3, we propose to conclude from co-existence study perspective, the PC3 ACLR (30dB) is sufficient for NR-NTN PC2 and PC1.5 HPUE (regardless of isolation distance assumptions).

Proposal 2: To conclude if PC3 ACLR (30dB) is sufficient for NR-NTN PC1, by taking into account any results updates and further discussions in this meeting, and also different isolation distance assumptions.

Proposal 3: To conclude from coexistence study perspective, PC3 ACLR is sufficient for IoT NTN HPUE with power classes up to PC1.

4	Reference

[1] R4-242046, Collected NTN HPUE co-existence results, Samsung

[2] R4-2420403, WF on NR NTN HPUE, Samsung