3GPP TSG-RAN4 Meeting #114	R4-2501808

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on NR-NTN HPUE Tx requirements

Source:	Samsung

Agenda item:	7.10.2.2.1

Document for:	Approval

1	Introduction

In previous several RAN4 meetings, we and many other companies have expressed anlaysis and views over the feasibility of NR-NTN HPUE architecture for different power classes. And in last #113 meeting, the WF[1] had captured the summarized views from different companies over different power classes.

In this paper, we will discuss each power class, and propose to warp-up the discussion and propose to focus and progress the work for this item.

2	Power class 2

From the last meeting WF [1], it can be concluded, from co-ex study point of view, that the ACLR for PC2 can be relaxed from the PC3 ACLR, which is 30 dBc.

Observation 1: The coexistence study indicate a relaxed ACLR can be sufficient for NTN PC2 compared to the NTN PC3.

Considering the follwing:

- above observation 1;

- discusions of MPR, A-MPR are pending for ACLR decisions, and

- the potential benefit of higher output power, and also the findings from co-ex study

We propose the meetig to consider the ACLR for NR-NTN PC2 HPUE as 30dB same as NTN PC3, but at the same time to recognize the co-ex study suggests a lower ACLR for PC2 for NR-NTN. With this compromise, we could further progress the work for NR-NTN PC2.

Proposal 1: Propose to re-use NR-NTN PC3 ACLR 30dB as ALCR requirement for NR-NTN PC2 HPUE, and recognizing a less stringent ACLR is suggested from co-ex study.

Once the ACLR for PC2 can be agreed, regardless of whether it ends up with our above proposal 1 or not, with previously agreed SEM, the normative work to determine the MPR and A-MPR should be carried out.

Observation 2: The SEM was agreed re-use the PC3 as starting point. And after several meeting discussion, our observation is that common view is not to change SEM requirement as it is mostly from existing regulatory requirements.

If we follow the above Proposal 1, with existing PC3 SEM requirements as in Observation 2, the MPR could potentially re-use the requirement from TN spec, at least for handheld PC2 UE with 1Tx architecture.

Proposal 2: At lesat for 1Tx PC2 NR-NTN HPUE, the MPR could take the TN 38.101-1 PC2 MPR as a starting point.

And regarding A-MPR, per previous regulatory survey, for n256, the A-MPR requirements for n65 could be as starting point. From TS 38.101-1, since the maximum carrier centre frequency of NS_05 and NS_05U is lower than 1960 MHz and the maximum carrier centre frequency of NS_51 is lower than 1980 MHz, it is proposed NS_05, NS_05U and NS_51 don’t need to be considered for n256. The requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65 can be reused for n256.

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection) (Power Class 3 and Power Class 2) Modulation/Waveform; Outer (dB). DFT-s-OFDM; Pi/2 BPSK; ≤ 2. QPSK; ≤ 2. 16 QAM; ≤ 2.5. 64 QAM; ≤ 3. 256 QAM; ≤ 4.5. CP-OFDM; QPSK; ≤ 4. 16 QAM; ≤ 4. 64 QAM; ≤ 4. 256 QAM; ≤ 6.5. NOTE 1: Void NOTE 2: Void. Table 6.5.2.4.2-1: UTRA ACLR requirement Power class 2; Power class 3. UTRAACLR1; 33 dB; 33 dB. UTRAACLR2; 36 dB; 36 dB. Table 6.2.3.15-1: A-MPR for NS_24 Channel Bandwidth, MHz; Carrier Centre Frequency, Fc, MHz; Region A; Region B; Region C. RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR; RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR; RBend*12*SCS MHz; LCRB*12*SCS MHz; A-MPR. 5MHz; 1987.5 < Fc ≤ 1992.5; >3.24; A7. 5MHz; 1992.5 < Fc ≤ 1997.5; >3.24; A4. 5MHz; 1997.5 < Fc ≤ 2002.5; >1.98; A1; >3.6; >1.08 ≤1.98; A2; ≤3.6; ≤1.98; A3. ≤1.08; A6. 10MHz; 1975 < Fc ≤ 1985; >5.4; A4. 10MHz; 1985 < Fc ≤ 1995; >4.32; A1; ≥7.2; >1.08 ≤4.32; A2; <7.2; ≤4.32; A3. ≤1.08; A6. 10MHz; 1995 < Fc ≤ 2000; ≥5.76; A5; <3.06; A5; ≥3.06 <5.76; >1.44; A6. 15MHz; 1972.5 < Fc ≤ 1987.5; >6.84; A1; ≥10.8; >1.08 ≤6.84; A2; <10.8; ≤6.84; A3. ≤1.08; A6. 15MHz; 1987.5 < Fc ≤ 1997.5; ≥8.64; A5; <3.78; A5; ≥3.78 <8.64; >1.44; A6. 20MHz; 1970 < Fc ≤ 1990; ≥12.96; A5; <4.68; A5; ≥4.68 <12.96; >2.16; A6. 20MHz; 1990 < Fc ≤ 1995; ≥11.52; A5; <5.58; A5; ≥5.58 <11.52; >1.44; A6. NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies. Table 6.2.3.15-2: A-MPR for modulation and waveform type Modulation/Waveform; A1; A2; A3; A4; A5; A6; A7. Outer/Inner; Outer/Inner; Outer/Inner; Outer; Outer/Inner; Outer/Inner; Outer. DFT-s-OFDM PI/2 BPSK; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM QPSK; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM 16 QAM; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 18; ≤ 10; ≤ 3.5. DFT-s-OFDM 64 QAM; ≤ 11; ≤ 5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 10; ≤ 3.5. DFT-s-OFDM 256 QAM; ≤ 11; ≤ 5; ≤ 8.5; ≤ 20; ≤ 10. CP-OFDM QPSK; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 16 QAM; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 64 QAM; ≤ 13; ≤ 6.5; ≤ 4; ≤ 8.5; ≤ 19; ≤ 12; ≤ 5.5. CP-OFDM 256 QAM; ≤ 13; ≤ 6.5; ≤ 8.5; ≤ 20; ≤ 12. NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2. Table 6.5.3.3.13-1: Additional requirements for "NS_24" Frequency range (MHz); Channel bandwidth (MHz) / Spectrum emission limit (dBm); Measurement bandwidth. 5 MHz, 10 MHz, 15 MHz, 20 MHz. 2010 ≤ f ≤ 2025; -50; 1 MHz. NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band..

From existing NS_24 and NS_100, it can be observed that the A-MPR of NS_100 already cover to both PC3 and PC2, while the NS_24 may require further work to makes ure the emission limit is met for power class 2.

Proposal 3: For A-MPR of n256, take the requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65 as starting point, further check if updates for NS_24 for PC2 is needed.

3	Power class 1.5

The views over PC1.5 is summarized in [1] as follows:

Issue 2-6: NR-NTN PC1.5 UE Agreement in #112bis meeting: Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not. - Proposals PC1.5 UE; 1Tx; 2Tx. Handheld; To specify: Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015, P1/R4-2419521 FFS:; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532 Not to specify: P1b/R4-2417747, P2/r4-2419072, P1/R4-2419015 FFS:. Non-handheld; To specify: Not to specify: P1/R4-2419668, P3/R4-2419072 FFS: P5/R4-2418200, P1/R4-2419521; To specify: P1/R4-2417814, P5/R4-2418200, P1/R4-2419521, P1/R4-2419532, P2/R4-2419521 Not to specify: P1/R4-2419668, P3/R4-2419072, P1b/R4-2417747 FFS:.

In short, many companies have shared their analysis over the feasibility, and most companies agreed the PC1.5 is feasible for both handheld and non-handheld devices, and the concern is over whether 1Tx or 2Tx, and whether PC1.5 for non-handheld is needed if PC1 non-handheld is to be specified.

Observation 3: Most companies confirmed feasbility of PC1.5 for both handheld and non-handheld device type, the concern is over the necessity and priority of specific device type.

In addition, for handheld UE, when compared to the 1Tx PC2, the 2Tx PC1.5 requires additional phase control per Tx chain for satellite. And the potential benefits seems marginal considering the real gain compared to 1Tx PC2. And considering the implementaion margin may not be large enough for massive production for 1Tx PC1.5 for handheld device, the general interests over handheld device in PC1.5 could be deprioritized.

For non-handheld device, the concern was from the whether the PC1.5 is necessary if PC1 non-handheld device is to be defined. We somehow share this concern.

Proposal 4: Propose to de-prioritize the PC1.5 work in Rel-19 while recognizing the NR-NTN PC1.5 can be feasible for both handheld and non-handheld device type.

And from coexistence study outcomes in [1], we suggest the PC1.5 ACLR can be suggested same as PC3 as 30dB.

Proposal 5: Propose to re-use NR-NTN PC3 ACLR of 30dB for NR-NTN PC1.5.

4	Power class 1

For NR-NTN in PC1, [1] had summarized the views from different companies in #113 as follows:

Issue 2-7: NR-NTN PC1 UE Agreement in #112bis meeting: Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE.. - Proposals PC1; 1Tx; 2Tx; 3Tx; 4Tx. Handheld; To specify: Not to specify: P1/R4-2419521 FFS:; To specify: Not to specify: P5/R4-2418200, P1/R4-2419521 FFS:; To specify: P1/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: Not to specify: FFS:. Non-handheld Only non-handheld: P2/R4-2419015, P1/R4-2419072; To specify: P1a/R4-2417747, P2/R4-2419521 Not to specify: FFS:; To specify: Not to specify: P5/R4-2418200 FFS:; To specify: P2/R4-2419521 Not to specify: P2/R4-2419668 FFS: P5/R4-2418200; To specify: P4/R4-2419072 Not to specify: FFS:.

Observation 4: Based on the feasibility discussion, it potentially can be feasible for handheld UE if adopt 3Tx and with existing components. And the non-handheld UE type has no issue even with 1Tx.

Given the continued discussion over the feasibility of PC1, especially on handheld devices, we are of the view that there are contributions expressing the handheld device could reach PC1 for NR-NTN with some design, which proved it could be feasible, but the beneficial of such design requires more analysis over the cost and in terms of the real throughput gain. With this, we believe it could be better to focus on the non-handheld UE device in normative phase of Rel-19, while continue discussing the potential possibilities of enabling PC1.5 and PC1 for handheld devices in parallel or in a de-prioritized manner.

Proposal 6: Propose to focus on non-handheld device for PC1 in Rel-19 for specification work.

For PC1 ACLR, from [1], the required ACLR for NR-NTN PC1 is around 30 dB considering different scenarios. It indiciate the PC3 ACLR can be sufficient if we follow the same isolation distance as Rel-17.

Observation 5: If we follow the same logic of isolation distance as in Rel-17, the PC3 ACLR of 30dB can be sufficient for NR-NTN PC1 ACLR requirement.

However, if we follow the Proposal 6 above, and to focus on non-handheld device type as current TN PC1 vehicle-based device type, then it’s also reasonble to follow the Rel-17 logic of re-using the TN spec to utilize the existing devices to support NR-NTN. With this consideration, the ACLR for NR-NTN PC1 can re-use the TN PC1 of 37dBc.

Observation 6: For non-handheld device, if we follow same logic of re-use existing device and component as Rel-17, then the TN PC1 ACLR of 37 dBc can be reused for NR-NTN PC1 non-handheld devices.

Considering above, we think it’s worth to note the required ACLR for PC1 is actually much lower than the TN PC1 ACLR of 37dBc, and if we limit the requirement to a non-handheld devices, then it can be beneficial to re-use the TN PC1 ACLR requirement, but it should be clear it does not apply to handheld devices, if it will be specified later.

Proposal 7: For PC1 non-handheld devices, to re-use the TN PC1 ACLR of 37dBc, while recognizing a less stringent ACLR is suggested from co-ex study, and it does not apply to handheld device if specified later.

Similarly, if the ACLR of TN PC1 is adopted for NR-NTN PC1, and same SEM will be re-used, the MPR requirement from TS 38.101-1 Table 6.2.2-4b could be re-used.

Proposal 8: For MPR for NR-NTN PC1 non-handheld device, the TS 38.101-1 Table 6.2.2-4b could be re-used.

5	Conclusion

For NR-NTN HPUE Tx requirements, we propose the following:

Power class 2:

Observation 1: The coexistence study indicate a relaxed ACLR can be sufficient for NTN PC2 compared to the NTN PC3.

Proposal 1: Propose to re-use NR-NTN PC3 ACLR 30dB as ALCR requirement for NR-NTN PC2 HPUE, and recognizing a less stringent ACLR is suggested from co-ex study.

Observation 2: The SEM was agreed re-use the PC3 as starting point. And after several meeting discussion, our observation is that common view is not to change SEM requirement as it is mostly from existing regulatory requirements.

Proposal 2: At lesat for 1Tx PC2 NR-NTN HPUE, the MPR could take the TN 38.101-1 PC2 MPR as a starting point.

Proposal 3: For A-MPR of n256, take the requirements of NS_01, NS_24 and NS_100 specified in Table 6.2.3.1-1 in TS 38.101-1 for band n65 as starting point, further check if updates for NS_24 for PC2 is needed.

Power class 1.5:

Observation 3: Most companies confirmed feasbility of PC1.5 for both handheld and non-handheld device type, the concern is over the necessity and priority of specific device type.

Proposal 4: Propose to de-prioritize the PC1.5 work in Rel-19 while recognizing the NR-NTN PC1.5 can be feasible for both handheld and non-handheld device type.

Proposal 5: Propose to re-use NR-NTN PC3 ACLR of 30dB for NR-NTN PC1.5.

Power class 1:

Observation 4: Based on the feasibility discussion, it potentially can be feasible for handheld UE if adopt 3Tx and with existing components. And the non-handheld UE type has no issue even with 1Tx.

Proposal 6: Propose to focus on non-handheld device for PC1 in Rel-19 for specification work.

Observation 5: If we follow the same logic of isolation distance as in Rel-17, the PC3 ACLR of 30dB can be sufficient for NR-NTN PC1 ACLR requirement.

Observation 6: For non-handheld device, if we follow same logic of re-use existing device and component as Rel-17, then the TN PC1 ACLR of 37 dBc can be reused for NR-NTN PC1 non-handheld devices.

Proposal 7: For PC1 non-handheld devices, to re-use the TN PC1 ACLR of 37dBc, while recognizing a less stringent ACLR is suggested from co-ex study, and it does not apply to handheld device if specified later.

Proposal 8: For MPR for NR-NTN PC1 non-handheld device, the TS 38.101-1 Table 6.2.2-4b could be re-used.

6	Reference

[1] R4-2420403, WF on NR NTN HPUE, Samsung