3GPP TSG-RAN4 Meeting #114	R4-2501809

# Athens, GR, 17th – 21st February, 2025

Title:	Discussion on NR-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	7.10.2.2.2

Document for:	Approval

1	Introduction

In previous several RAN4 meetings, the Rx requirements, especially the RSD requirements for different power classes were discussed and analyzed.

In this paper, we continue to discuss the remaining issues and to progress the work.

2	Rx requirements

In [1], the PC2 RSD for n256 seems stable, and the only requested follow-up work is to further check if any updates from duplex isolation and PA interference assumptions.

Issue 3-1-4: RSD requirements for PC2 on n256 Way forward: For PC2 on n256: [0] dB RSD for PC2 NR-NTN for 1Tx and 2Tx as starting point - To be confirmed by further discuss the following assumptions: - Option 1: Duplex isolation as 50~52; PA interference as -117.8 dBm/Hz - Option 2: Duplex isolation as 51; PA interference as -124 dBm/Hz

We would like to repeat our previous proposals which is to suggest 0 dB for n256 in PC2. In addition, in last meeting, several companies shared the similar view that the n256 would generally re-use n1 component, and also to re-use the assumptions from n1 to derive the RSD requirements for n256. And given the n256 has wider duplexing distance than n1, so the 0dB specified for n1 for PC2 for UE with and without TxD can be re-used.

Proposal 1: To confirm the 0dB RSD for n256 PC2 compared to PC3 for both 1Tx and 2Tx.

And also, for PC1, we had a seaprate document carrying our proposal to focus on non-handheld UE type in Rel-19. With that proposal, we believe the RSD can be set to 0 dB if specified.

Issue 3-3: RSD requirements for PC1 - Proposals - Option 1: For PC1 1Tx for NR-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB (P4a/R4-2418201) - Option 1-1: FFS for PC1 with 2 or 3 Tx cases (P4b/R4-2418201) - Option 2 no RSD is needed for non-handheld UE including PC 1 and PC 1.5. (P3/R4-2419016)

Observation 1: We had proposed in a separate document that to focus on non-handheld UE type for NR-NTN PC1 in Rel-19.

Proposal 2: For RSD for NR-NTN PC1 non-handheld device, to confirm 0 dB RSD.

3	Conclusion

For NR-NTN HPUE Rx requirements, we propose the following:

Proposal 1: To confirm the 0dB RSD for n256 PC2 compared to PC3 for both 1Tx and 2Tx.

Observation 1: We had proposed in a separate document that to focus on non-handheld UE type for NR-NTN PC1 in Rel-19.

Proposal 2: For RSD for NR-NTN PC1 non-handheld device, to confirm 0 dB RSD.

4	Reference

[1] R4-2420403, WF on NR NTN HPUE, Samsung