3GPP TSG-RAN WG4 Meeting #114	R4-2501822

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	NR NTN system parameters for Ku band

Agenda item:	7.11.3

Document for:	Approval

Introduction

In RAN4#113 and RAN#106 meeting, it was agreed to define two sets of bands for both FR1 and FR2 numerology [1][2][3] for the same frequency range. The WID was revised in [4].

Sub-topic 1-1: Defining bands by regulatory authority Agree the following band definition Table X: Band definitions for Ku band Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. nX(Note 1); 14000 MHz - 14500 MHz; 10700 MHz - 12750 MHz; FDD. nY(Note 2); 14000 MHz - 14500 MHz; 10700 MHz - 12700 MHz; FDD. NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25.. Sub-topic 3-9: FR1 FR2 down selection Overall observation is that both FR1 numerology and FR2 numerology are feasible. There are use cases that favour either FR1 numerology or FR2 numerology Some of the disadvantages of numerology combinations can be mitigated with implementation choices WF on numerology: Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

In this contribution, we discuss operating band definition and system parameters of the Ku band. There is also earlier agreement on the system parameters from RAN4#112-bis [5] in the following.

Sub-topic 3-12: SCS channel raster SCS-based channel raster is assumed. Sub-topic 3-13: Flexible TX Rx spacing Flexible TX Rx spacing is assumed. Sub-topic 3-14: Sync raster The existing sync raster design is reused for Ku band(s).

Discussion

Operating band

For FR1 numerology, the following band definitions are proposed. The unused band numbers, [n249] and [n248], are tentatively assumed for FR1-NTN Ku bands.

Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n249] | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n248] | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |

For FR2 numerology, the following band definitions are proposed. The unused band numbers, [n509] and [n508], are assumed for FR2-NTN Ku bands.

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| [n509] | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| [n508] | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |

Channel bandwidth and Subcarrier spacing

For FR1-NTN Ku bands, 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing are supported as defined in WID [4].

For FR2-NTN, 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing are supported.

The following channel bandwidth table should be introduced for FR1-NTN.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 50 | 70 | 100 |
| [n249] | 30 |  | 10 |  | 20 | 25 | 30 | 35 | 50 | 70 | 100 |
| [n248] | 30 |  | 10 |  | 20 | 25 | 30 | 35 | 50 | 70 | 100 |

The following channel bandwidth table should be introduced for FR2-NTN.

Table 5.3.5-2: Channel bandwidths for each NTN satellite band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| [n509] | 120 | 50 | 100 | 200 | 400 |
| [n508] | 120 | 50 | 100 | 200 | 400 |

Channel raster

SCS-based raster was agreed in RAN4#112-bis [5], i.e., 30 kHz raster for 30 kHz SCS of FR1-NTN Ku bands and 120 kHz raster for 120 kHz SCS of FR2-NTN Ku bands. Therefore, the following channel raster definitions apply.

For NTN satellite operating bands with 15 kHz channel raster above 3GHz, ΔFRaster = I × ΔFGlobal, where I ϵ {1,2}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <I> for FR1-NTN.

In frequency bands with two or more ΔFRaster, the higher ΔFRaster: For 15 kHz and 30 kHz channel raster applies to channels using only the SCS that is equal to or larger than the higher ΔFRaster and SSB SCS is equal to the higher ∆FRaster.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| [n248] | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1246666 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 | 1333334 – <8> – 1366666 | 1113334 – <8> – 1250000 |
| [n508] | 120 | 1333334 – <8> – 1366666 | 1113334 – <8> – 1246666 |

Tx-Rx Separation

Since we agreed flexible duplex in RAN4#112-bis [5], no specific Tx-Rx separation is necessary in clause 5.4.4 of TS 38.101-5.

Sync raster

In the WF agreed in RAN4#112-bis [5], the existing sync raster design was assumed for Ku bands, meaning that the GSCN parameters in Table 5.4.3.1-1 of TS 38.101-5.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS Block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 MHz | N * 1200kHz + M * 50 kHz, N=1:2499, M ϵ {1,3,5}1 | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| NOTE: The default value for operating bands with which only support SCS spaced channel raster(s) is M=3. |  |  |  |

For the FR1-NTN Ku band with 30 kHz SCS, SS block pattern C should be assumed, instead of pattern B which is intended for a specific LTE-NR coexistence scenario. GSCN ranges are derived in the following.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n249] | 30 kHz | Case C | 12850 – <1> – 14266 |
| [n248] | 30 kHz | Case C | 12850 – <1> – 14221 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

For the FR2-NTN Ku band with 120 kHz SCS, SS block pattern both D and E should be assumed to allow both 120 kHz and 240 kHz SS block SCS (while data SCS is only 120 kHz). GSCN ranges are derived in the following.

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n509] | 120 kHz | Case D | 12858 – <1> – 14258 |
|  | 240 kHz | Case E | 12870 – <2> – 14246 |
| [n508] | 120 kHz | Case D | 12858 – <1> – 14223 |
|  | 240 kHz | Case E | 12870 – <2> – 14212 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

Conclusion

In this contribution, the systema parameters for Ku bands based on both FR1-NTN and FR2-NTN are proposed.

- References

- R4-2419909 Way Forward for [112][313] NR_NTN_Ku_Band_General, RAN4#113

- R4-2419902 LS on Ku band numerology, RAN4#113

- RP-243301 Way forward on Ku band numerology, RAN#106

- RP-243314 Revised WID: Introduction of Ku bands for NR NTN, RAN#106

- R4-2416593 Way Forward for [112bis][313] NR_NTN_Ku_Band_General, RAN4 #112 bis