3GPP TSG-RAN WG4 Meeting #114                                         R4-2501835

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR2 CA

Agenda Item:	7.1.1.2.3

Document for:	Approval

Introduction

During RAN4#113, MPR applicability for FR2 single carrier UL and intra-band UL CA with DL intra-band CA has been discussed. Remaining FFS points can be found in [1].

- Conditions of CC activation based enhancement Agreement in main session and WF: - For the Rel-19 activation based enhancement, FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration where all the activated UL CCs forms a contiguous block. - FFS emission requirements are activated UL CCs BW based or Cumulative aggregated channel bandwidth (CABW) based - Optional UE capability Agreement in main session and WF: - For Rel-19 FR2 UE MPR, define distinct enhancement capabilities for configuration-based enhancements and activation-based enhancement. - FFS if the capabilities should be mutually exclusive Proposals in R4-2418337 (Samsung) - Proposal 1: Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables. - Proposal 2: RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz. - WF - FFS proposal 2 with the identified discrepancy of CA BW class and MPR table - FFS if 800MHz is explicitly referenced as an upper limit

In this contribution, we would like to share our views regarding this topic.

Discussion

In last meeting, it was agreed to introduce configuration based MPR improvement, by which the MPR can be determined by UL aggregated channel bandwidth BWchannel_CA rather than cumulative aggregated channel BW. Regarding the activated-based mechanism, since implementation agnostic is emphasized and reflected in the WF, our understanding is the upcoming discussion for such mechanism should cover all possibilities. When we lay our eyes on the OOB requirements, it is clear that the requirements are based on UL aggregated channel bandwidth as defined in TS 38.101-2, which is configuration based.

##### Aggregated Channel Bandwidth: The RF bandwidth in which a UE is configured to transmit and receive multiple contiguously aggregated carriers. … ##### 6.5A.2.1.1 Spectrum emission mask for intra-band contiguous UL CA For intra-band contiguous UL carrier aggregation, the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the ± edge of the UL aggregated channel bandwidth (Table 5.3A.5-1). For any bandwidth class defined in Table 5.3A.5-1, the UE emission shall not exceed the levels specified in Table 6.5A.2.1-1. …… ##### 6.5A.2.3.1 Adjacent channel leakage ratio for CA intra-band contiguous UL CA In case the CA configuration consists of a single UL CC, the adjacent channel leakage ratio defined in subclause 6.5.2.3 applies. For intra-band contiguous UL carrier aggregation, the carrier aggregation NR adjacent channel leakage power ratio (CA NRACLR) is the ratio of the filtered mean power centred on the UL aggregated channel bandwidth to the filtered mean power centred on an adjacent UL aggregated channel bandwidth at spacing equal to the UL aggregated channel bandwidth. The assigned UL aggregated channel bandwidth power and adjacent UL aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.5A.2.3.1-1. If the measured adjacent channel power is greater than -35 dBm then the CA NRACLR shall be higher than the value specified in Table 6.5A.2.3.1-1.

Observation 1: The OOB requirements for FR2 UL intra-band contiguous CA are based on UL aggregated channel bandwidth, which is configuration based.

Thus we prefer to keep it as it is and leave some room for the UE that intends to achieve the improved MPR e.g. UE incapable of LO retuning.

Proposal 1: For Rel-19 FR2 intra-band contiguous UL CA MPR improvement (both configuration based and activation based), the OOB requirements are still based on UL aggregated channel bandwidth as defined in TS 38.101-2.

Another issue is whether capabilities i.e. configuration based and activation based should be mutually exclusive. For analysis purpose, we think this is important to make the implementation assumption behind each capability clear. In previous discussion, it was actually shared/independent Tx/Rx LO and LO tuning being discussed rather than the signalling coming from the network. Thus if both capabilities are indicated by a UE, it means that the UE is capable of determining MPR according to the bandwidth composed of activated UL CCs since the implementation assumption for this type of UE is independent LO and LO tuning. In conclusion, we think these two capabilities are not mutually exclusive.

Observation 2: As the implementation assumption behind the capabilities on configuration-based and activation-based could be the support of separate Tx/Rx LO and LO tuning respectively, these two capabilities do not need to be mutually exclusive.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR2 CA. We have the following observations and proposal:

Observation 1: The OOB requirements for FR2 UL intra-band contiguous CA are based on UL aggregated channel bandwidth, which is configuration based.

Observation 2: As the implementation assumption behind the capabilities on configuration-based and activation-based could be the support of separate Tx/Rx LO and LO tuning respectively, these two capabilities do not need to be mutually exclusive.

Proposal 1: For Rel-19 FR2 intra-band contiguous UL CA MPR improvement (both configuration based and activation based), the OOB requirements are still based on UL aggregated channel bandwidth as defined in TS 38.101-2.

- References

- R4-2420527, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #113, Orlando, US, November 18-22, 2024.