3GPP TSG-RAN WG4 Meeting #114                                         R4-2501840

Athens, Greece, February 17 - 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	7.23.1

Document for:	Approval

Introduction

During RAN4#113, general aspects (including RAN4 aspects for SBFD system parameters) for SBFD have been discussed. Following discussion points are reproduced from [1].

##### Clarification on declaration and flexibility Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications ##### DL/UL subband configurations support for SBFD Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS ##### Other restriction on DL subband size Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). - FR2 minimum CBW to support SBFD is 100MHz. ##### RF requirement impact for PUSCH with resource muting Way Forward: - FFS the RF requirement impact, if any, due to UL resource muting introduced in RAN1.

In this contribution, we would like to share our views regarding this topic.

Discussion

To be further discussed, one of the hot topics in last meeting is subband configuration along with a sequence of limitations for SBFD. In addition, we would like to share our views on the RF requirement impact for PUSCH with resource muting.

The guard band and subband configuration for SBFD

To specify DL/UL subband configuration in BS RF requirements, it was agreed to further study whether additional restrictions can be introduced to Option 2b.

Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

With great efforts in last meeting, companies have reached a compromise that for a specific channel BW supported for SBFD, the number of maximum UL subband size that are allowed to be declared and selected from Table 5.3.2-1 in TS 38.104 would be decided between 2 and 3.

From real deployment perspective, allowing more UL subband sizes would bring more flexibility to operator especially considering different spectrum holdings among regions. During the Rel-18 SBFD SI, extensive analysis and discussion regarding the interference caused by simultaneous Tx/Rx at gNB side have been conducted. To guarantee the feasibility of SBFD, multiple techniques in RF domain have been proposed. If we consider subband filter as one key technique to support SBFD, to our understanding the implementation difficulties are not in supporting multiple UL subband sizes, because anyway switchable subband filter could be implemented as vendor will not be requested to support one specific UL subband size under this case. In conclusion we think from technical perspective there is no reason to limit the number of maximum subband size to 2 also because the support of SBFD is optional.

Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- Different sizes for specific UL subband can be supported for DUD and DU/UD, respectively.

Regarding whether to apply such restriction to the MR BS, we do not think it is necessary unless proponent can provide more technical reasons to prove e.g. more difficulties need to be overcome for MR BS comparing to WA BS.

Proposal 2: The restrictions on SBFD UL/DL subband configuration do not need to be considered for MR BS.

On the potential RAN4 impact for UL resource muting

According to the WID [2], the potential transmit signal quality/MPR requirement impact due to UL resource muting will be checked. Details on this CLI handling solution are reproduced from WID as follows.

The LS in [3] also provides RAN1 study progress so far, we understand that the most important information to help RAN4 evaluation is a comb-2 pattern will be applied when UL resource muting is enabled on a symbol for PUSCH.

To our understanding, this technique would have no impact to the existing transmit quality/MPR requirement. The reason is comb pattern is nothing new from Rel-15. For instance, DMRS multiplexing is specified but there is no dedicated minimum RF requirement for it so that doesn’t mean a UE can meet the aforementioned RF requirements may fail the very same requirements when transmission in comb pattern is scheduled. Further, if the total transmission power remain the same, which is already supported by 3dB power boosting as specified in the spec, there is no performance loss can be expected, either.

Figure 1.

The simulation assumptions are listed as below:

- Waveform & modulation: DFT-s-OFDM QPSK

- Bandwidth: 100MHz

- LCRB: 51RB (20MHz with 30kHz SCS)

- Power class: PC3

We can observe that there seems no MPR performance degradation for UL resource muting at least for the above configurations.

Observation: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH as comb-2 pattern is already supported since LTE.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following proposals:

Observation: For UL resource muting, there is no transmit quality or MPR requirement impact can be expected for PUSCH as comb-2 pattern is already supported since LTE.

Proposal 1: For a specific channel BW supported for SBFD, maximum 3 specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104.

- Different sizes for specific UL subband can be supported for DUD and DU/UD, respectively.

Proposal 2: The restrictions on SBFD UL/DL subband configuration do not need to be considered for MR BS.

- References

- R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

- R4-2500006, “LS on impact on transmit signal quality/MPR requirement”, RAN1, 3GPP TSG-RAN4 Meeting #113, Orlando, USA, November 18-22, 2024.