3GPP TSG-RAN WG4 Meeting#114                                R4-2501878

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.13.4.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on the OTA testing scope reduction

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. For the OTA testing scope reduction, we would like to share the further views based on the latest agreement in [4].

- OTA testing scope reduction: Identify BS excessive OTA test scope cases and reduce it with the aim to shorten the test duration time for TX IM, RX out of band blocking, for NR FR1. Issue 1-1: test scope reduction for Tx requirement - Recommended WF: - Focus on test scope reduction for Tx IMD requirement - Further discuss the applicable interference freq offset for ACLR, OBUE and spurious emission requirements; - Further discuss the test scope reduction of test configuration. Issue 1-2: Out of band blocking - Recommended WF: - Focus on test scope reduction for Rx OOBB requirement - Focus on the measurement step size for Rx OOBB requirement;

Discussion

- 2.1. Tx requirements

For BS RF requirement, there are two most time consuming requirement e.g. transmitter spurious emission requirement and and receiver OOBB requirement because its applicable frequency range is extremely large compared with other requirements.

For the 3rd party OTA verification chamber, it usually take 300 Euro per hour and maximum 10 working hours per day, therefore to ensure the acceptable test coverage with reasonable testing time is extremely important for vendors from both implementation cost and also development Iteration. As pointed out in [2], it has been found that the test time is prohibitive taking up to 3 times the time to perform the conformance testing compared to conducted conformance testing. Excessive OTA testing time has become a constraint for AAS product design, development and certification .

In the contribution [2], the TX IMD texting is identified to comprise of 96 test cases testing output power, ACLR and out of band emissions (all TRP requirements) with 6 interferer offsets are tested for each (12 for NC operation), we propose to focus on the testing scope reduction for OTA Tx IMD requirement as indicated in the following test cases.

Table 6.8.5.1-1 in 38.141-2: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal | NR single or multi-carrier, or multiple intra-band contiguously or non-contiguously aggregated carriers |
| Interfering signal type | NR signal, the minimum BS channel bandwidth (BWChannel) with 15 kHz SCS of the band defined in clause 5.3.5 of TS 38.104 [2] |
| Interfering signal power level | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from the lower (upper) edge of the wanted signal or edge of sub-block inside a gap | , for n=1, 2 and 3 |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the BS are excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. NOTE 2: In Japan, note 1 is not applied in Band n77, n78, n79. NOTE 3: For BS type 1-O supporting dual polarization, the interfering signal power shall be equally divided between supported polarizations at the CLTA interfaces. |  |

During the last RAN4 meeting, there were some initial discussions on the candidate frequency offset for Tx IMD requirement as worst case.

f1

f2

2f1-f2

BW=b

interfering signal

2f2-f1

BW=a

wanted signal

(a+b)/2

2a+b

2b+a

a + b

Figure 1. the generic model using for the analysis of 3rd intermodulation mechanism

f1

f2

2f1-f2

BW=5

2f2-f1

BW=a

wanted signal

10 MHz

2a+5 (MHz)

a+10 (MHz)

10 MHz

Figure 2. the 3rd intermodulation product for 5 MHz interfering signal in the 1st adjacent channel

For ACLR requirement, based on the initial understanding, it seems that1st interference signal freq offset should be the worst case for compliance testing.

For spurious emission requirement, the 3rd interference signal frequency offset seems to be the worst case for compliance testing.

For OBUE requirement, more evaluation might be needed even though in-band spurious emission requirement is most stringent requirement, however it is not clear whether this would be worst case to be complied with.

Proposals 1: propose to focus on the testing scope reduction for OTA Tx IMD requirement as listed in the following table.

Proposals 2: propose to further evaluate the candidate frequency offset for Tx IMD requirement as worst case for ACLR, OBUE and spurious emission requirement.

Observation 1: ACLR requirement, it seems that1st interference signal freq offset should be the worst case for compliance testing.

Observation 2:spurious emission requirement, the 3rd interference signal frequency offset seems to be the worst case for compliance testing.

- 2.2. RX out of band blocking

It is well known that RX out of blocking requirement is another most time consuming requirement from receiver perspective in addition to transmitter spurious emission requirement. We also have similar discussion for FR2 BS OOBB requirement in terms of step size for CW signal. The corresponding technical justification is captured in TR 38.817-02. For FR1 BS OOBB requirement testing, the same principle could be reused to reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity.

Extracted from TR38.817-02 The base station out-of-band blocking requirement is tested with a CW interferer and the wanted uplink signal. A spurious response occurs when the m-th harmonic of the interferer frequency mixes with the n-th harmonic of an internal signal, like a clock or a local oscillator signal, and this mixing product falls within the receive bandwidth of the wanted signal (or near the receive bandwidth, if the receiver selectivity is insufficient to suppress the spurious mixing product). The frequency of the unwanted mixing product equals ±m·fCW±n·finternal, where fCW is the interferer frequency and finternal is the frequency of the internal signal involved in the mixing product. m can assume the values 1, 2, 3, … and n can assume the values 0, 1, 2, …. Spurious responses for which the harmonic number m of the interferer signal equals 4 or more are usually not significant. Hence, the frequency step size should be chosen such that any spurious response due to a maximum third harmonic of the interferer will be captured within the receive bandwidth of the wanted signal. To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal at frequency above 6 GHz can be set according to the minimum supported BS channel bandwidth as shown in Table 10.6.2-1 in the NR BS type 2-O OTA out-of-band blocking conformance testing. For interfering signal at frequency below 6 GHz, the traditionally measurement step size of 1 MHz can be kept. Table 10.6.2-1: Measurement step size for out-of-band blocking conformance testing Minimum supported BS Channel bandwidth of wanted signal [MHz]; 50; 100; 200; 400. Measurement step size for interfering signal [MHz]; 15; 30; 60; 60.

Proposals 3: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

Conclusions

In this contribution, we shared some initial views on OTA test scope cases in RAN4 and proposals are made as following:

Proposals 1: propose to focus on the testing scope reduction for OTA Tx IMD requirement as listed in the following table.

Proposals 2: propose to further evaluate the candidate frequency offset for Tx IMD requirement as worst case for ACLR, OBUE and spurious emission requirement.

Observation 1: ACLR requirement, it seems that1st interference signal freq offset should be the worst case for compliance testing.

Observation 2:spurious emission requirement, the 3rd interference signal frequency offset seems to be the worst case for compliance testing.

Proposals 3: propose to reuse the principle design for Rel-15 FR2 BS OOBB CW step size e.g. 1/3 of BW as captured in TR 38.817-02.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- RP-240430,Views on BS RF topics for Rel-19, Huawei, approved.

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2419243	Test case reduction for TX IM, Huawei

- ] R4-1709451,Conducted transmit intermodulation for NR BS range 1, ZTE