3GPP TSG-RAN WG4 Meeting#114                                R4-2501881

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.2.3

Source: 	ZTE Corporation

Title: 	Discussion on the remaining issue for 15GHz BS RF requirements

Document for:	Approval

Introduction

In last RAN4 meeting, 15GHz has been declared to be finalized with the corresponding LS sent to WP5D, however there are some typos in the original LS and also some requirements are still missing for MR and LA BS. In this contribution, we would like to share some further views on these remaining issues.o

Issue 4-2-6: BS ACLR Agreement: - BS ACLR 30dB Issue 4-2-3: BS Emissions mask Agreement - Do not include category B OBUE in reply - Mention in TR that regional regulation corresponding to category B is not yet decided Issue 4-2-7: Spurious emission Agreement Reply that SM.329 applies Issue 4-2-5: Sensivitity Previous agreement: For LS: “To be specified” For TR: Define based on NF and antenna gain Agreement : define BS OTA sensitivity requirement based on the NF and the antenna gain similar 7 and 10GHz; EISREFSENS=PkT+10∗log10BW+NF+IM+SNR−G (dBm) Where: - BW is the configured bandwidth of the FRC, - NF is the noise figure, - IM is implementation margin not related to antenna array, - SNR is the required SNR to reach 95% throughput, and - G is the antenna gain including RF losses and 3dB off peak margin. Issue 4-2-8: Blocking response Agreement - ΔfOBUE and ΔfOOB 100MHz - Define requirement as follows, where interferer is defined in radiated way - FFS how to define the requirements for 1-H Frequency range of interfering signal (MHz); Wanted signal mean power (dBm); Interferer RMS field-strength for OTA requirements (voltage/meter); Type of interfering signal. 30 to 12750; EISREFSENS + 6 dB; 0.36; CW. 12750 to FUL,low – 100; EISREFSENS + 6 dB; 0.1; CW. FUL,high + 100 to 2nd harmonic of the upper frequency edge of the operating band; EISREFSENS + 6 dB; 0.1; CW. Issue 4-2-9: BS ACS BS ACS 30dB Issue 4-2-X: In-band blocking Agreement For LS: Mention “To be specified" For TR: FFS for next meeting

Discussion

In the previous RAN4 meeting, we agreed to define the Cat A requirement for 15GHz as following. The following requirement in table 6.1.2-1 indeed is the basic limit for WA BS at 15GHz. For the WP5D reply LS as shown in Table 1A, it is the AAS BS spectral mask which should be scaled up with 9dB compared with basic limit in table 6.1.2-1.

- Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter 3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 50 MHz | 0.05 MHz £ f_offset < 50.05 MHz | −7dBm−750foffsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, Dfmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | 100.5 MHz £ f_offset < f_offsetmax | -13 dBm | 1MHz |

TABLE 1A

AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -14 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -13 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Proposal 1: for WA BS OBUE requirement for 15GHz, propose to define the emission requirement as following:

AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -5 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -4 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

In addition, in the previous RAN4 meeting, there were some coexistence study for Dense urban and local indoor evaluation, however there were no discussions on RF requirement for MR and LA BS. In the following section, we would like to share our further views on these emission requirement. Since for WA BS, RF requirement for 15GHz BS is reused from 6-10GHz Cat A requirement, we could follow the same principle for and LA. In other words, the MR and LA BS requirement as captured in TS38.104 as following could be reused for 15GHz.

For BS type 1-H operating in Band n104, basic limits are specified in Table 6.6.4.2.3-1b and Table 6.6.4.2.3-2b:

Table 6.6.4.2.3-1b. Medium Range BS type 1-H operating band unwanted emission limits for band n104, 31< Prated,x  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | Prated,x−53dB−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | Prated,x - 60dB | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | Min(Prated,x - 60dB, -25dBm) (Note 3) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be Min(Prated,x -60dB, -25dBm)/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

Table 6.6.4.2.3-2b. Medium Range BS type 1-H operating band unwanted emission limits for band 104, Prated,x  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −22dBm−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -29 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -29 dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap. NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

For BS type 1-H operating in Band n104, basic limits are specified in Table 6.6.4.2.4-1b:

Table 6.6.4.2.4-1b. Local Area BS type 1-H operating band unwanted emission limits for band n104

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (Note 1, 2) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | −30dBm−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -37 dBm | 100 kHz |
| 100 MHz f fmax | 100.05 MHz f_offset < f_offsetmax | -37 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the emission limits within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the sub-block gap. Exception is f ≥ 100MHz from both adjacent sub-blocks on each side of the sub-block gap, where the emission limits within sub-block gaps shall be -37dBm/100kHz. NOTE 2: For a multi-band connector with Inter RF Bandwidth gap < 2*ΔfOBUE the emission limits within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap NOTE 3: The requirement is not applicable when fmax < 100 MHz. |  |  |  |

Proposal 2: propose to reuse the MR and LA BS requirement as captured in TS38.104 for 15GHz BS RF requirements.

Regarding other issues,e.g. in-band blocking requirement or OOBB requirement for BS type 1-H, this could be further discussed during the WI phase once this band is introduced in the future.

Proposal 3: propose to further discuss the in-band blocking or OOBB requirement for BS type 1-H in the WI phase.

Conclusions

In this contribution, we want to share our views on the remaining issue and proposals are made as following.

Proposal 1: for WA BS OBUE requirement for 15GHz, propose to define the emission requirement as following:

AAS BS Spectral mask (Operating band unwanted emissions limits) for 14800 - 15350 MHz operation (Category A)

| Frequency offset of measurement filter ‑3dB point from the carrier frequency, Δf | Basic limits | Measurement Bandwidth |
| --- | --- | --- |
| 0 MHz £ Df < 50MHz | 2dBm−750f_offsetMHz−0.05 | 100 kHz |
| 50 MHz £ Df < min(100 MHz, Dfmax) | -5 dBm | 100 kHz |
| 100 MHz £ Df £ Dfmax | -4 dBm | 1 MHz |
| NOTE: Dfmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter, where f_offsetmax is the offset to the frequency ΔfOBUE = 100 MHz outside the downlink operating band. |  |  |

Proposal 2: propose to reuse the MR and LA BS requirement as captured in TS38.104 for 15GHz BS RF requirements.

Proposal 3: propose to further discuss the in-band blocking or OOBB requirement for BS type 1-H in the WI phase.

References

- R4-2420385.

- R4-2420330, WF on 15GHz BS parameters, ZTE Corporation