3GPP TSG-RAN WG4 Meeting#114                                R4-251xxx

Athens, Greece, 17th – 21th Feb, 2025

Agenda item:	7.23.2.2

Source: 	ZTE Corporation

Title: 	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on these remaining open issues.

Discussion

## 2.1. Tx requirements

### 2.1.1. Co-location ACLR requirement

Issue 2-3-1: The necessity of Co-location ACLR requirement - Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. Issue 2: Unwanted emissions - Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

From our understanding, the additional or tightening requirement when two SBFD BS operating at the adjacent frequency, then tightening ACLR requirement or OBUE requirement should be helpful to address the Bs2Bs CLI problem especially for Scenario 1, Case 3. However it should be noted that there are no much ACLR/OBUE requirement to be improved for AAS based SBFD BS due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity. As proposed by the some companies, the front-end RF filter for Wide area SBFD BS and analog receiver sub-band filter should be implemented to achieve better ACLR/OBUE requirement to protect the co-located victim SBFD BS UL .e.g aggressive BS and victim mounted shoulder by shoulder or top/bottom position, however this kind of requirement might be not valid or useful for other deployments e.g. with victim SBFD BS in the boresight direction of aggressive BS. In short, we propose to keep the legacy ACLR requirement for SBFD capable of BS without defining any more tightening requirement for SBFD BS.

Proposal 1: propose to follow the legacy ACLR requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS.

### 2.1.2. OBUE requirement

Issue 2-3-2: OBUE - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency. Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

Similar as Co-located ACLR requirement, during the last RAN4 meeting, there were some discussions on how to define OBUE requirement for SBFD capable BS in order to ensure the coexistence performance for SBFD BSs operating in adjacent channel shown as following. During the Rel-18 SBFD SI phase, the following coexistence scenario was not evaluated, however the CLI interference is quite similar as Case 3 where NR DL interfering SBFD UL. Especially for Scenario 1, Case 3, the severe performance degradation could be observed for SBFD UL due to the strong CLI from NR DL in the adjacent channel. Even with ACIR requirement tightening 8dB, the 5% throughput loss is still up to 43.5% and the mean throughput loss could be minimized down to 4.67%. However as we know that, for AAS based SBFD BS, due to the higher operating frequency and larger channel bandwidth and small transceiver’s PA linearity, there are no much ACLR/OBUE requirement to be further improved. In other words, even though RAN4 is capable to agree more tightening requirement e.g. with 10dB tightening requirements than the legacy requirements, it is still not feasible to be achieved for SBFD capable BS. As mentioned before, the RF filter for wide area SBFD BS also need more time to check its feasibility and it expected filter size to ensure that BS is not too chunky and without too big insertion loss at the end

Based on the above considerations, we propose to follow the legacy OBUE requirement for SBFD BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

Proposal 2: propose to follow the legacy OBUE requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

### 2.1.3. EVM

WF: - FFS on the followings: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

In the previous RAN4 meetings, there were some discussions on how to do the average EVM for SBFD capable BS.  As shown in the following figure 2.1.3-1a, the EVM performance for 1st symbol after the transmitter is turn on and last symbol before transmitter is turn off is supposed to be very bad (e.g. up to 17.5% EVM) which is also observed in UE Tx EVM measurement. That is parts of the reasoning to have 10ms measurement period for EVM evaluation to get the stable EVM measurement.

Figure 2.1.3-1a. EVM performance for normal TDD configuration, DDDSU.

For the following figure, we have two downlink slots configured for SBFD configuration, as shown in the following figure 2.1.3-1b, the ratio of bad EVM performance for normal DL slots or non-SBFD slots will increase, meanwhile the ratio of bad EVM performance for SBFD symbols will be also relatively higher compared with the legacy TDD configuration without SBFD configuration as shown in Figure 2.1.3-1a. In the words, the expected EVM performance would be relatively worse if to measure non-SBFD symbols and SBFD symbols separately due to more symbols with bad EVM performance. In addition, even go with joint average between SBFD symbols and non-SBFD symbols, then EVM performance will be become worse compared with the legacy TDD configuration in theory.

Figure 2.1.3-1b. EVM performance for TDD configuration with SBFD symbols, DDDSU with 2 D slots for SBFD configuration

Based on the above analysis and considerations, even though we think that joint EVM between non-SBFD symbols and SBFD symbols should be better option since the serving UE might be scheduled in both non-SBFD symbols and SBFD symbols. To ensure the BS EVM performance across non-SBFD and SBFD should be more reasonable assumption to ensure the UE DL reception performance. However we are also fine to compromise with separate the EVM testing, some additional modifications on EVM calculation in Annex is needed. The detailed could be found as following:

As extracted from the Annex B.7 in TS 38.104, the number of downlink frame is calculated based on the slots granularity. According to the SBFD configuration as agreed in RAN1, it could be symbols level granularity, then following definition could be updated as following for SBFD capable of BS.

Calculation of averaged EVM is defined in Annex B.7 for FR1 in TS 38.104 [5]:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl_symbol symbols where Ndl_symbol  is the number of dl symbols in a 10 ms measurement interval.

For TDD, let Ndl_symbolsTDD be the number of downlink SBFD symbols or non-SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  Ndl_symbolsTDD symbols of different 10 ms measurement intervals and should have a minimum of Ndl_symbol symbols averaging length where Ndl_symbol is the number of symbols in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1Ndl_symbolTDDNii=1Ndl_symbolTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in symbol i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl_symbol slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×Nsymbol Ndl_symbolsTDD

and Nsymbol=14 for 15 kHz SCS, Nsymbol=28 for 30 kHz SCS and Nsymbol=56 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

Proposal 3: propose to update the EVM calculation in Annex B.7 as above.

Conclusions

In this contribution, we want to share some initial views on RF requirements for SBFD BS and proposals/observations are made as following:

Proposal 1: propose to follow the legacy ACLR requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS.

Proposal 2: propose to follow the legacy OBUE requirement for SBFD capable BS without defining any more tightening requirement for SBFD BS to address the CLI concern in the adjacent channel.

Proposal 3: propose to update the EVM calculation in Annex B.7 as above.

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl_symbol symbols where Ndl_symbol  is the number of dl symbols in a 10 ms measurement interval.

For TDD, let Ndl_symbolsTDD be the number of downlink SBFD symbols or non-SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  Ndl_symbolsTDD symbols of different 10 ms measurement intervals and should have a minimum of Ndl_symbol symbols averaging length where Ndl_symbol is the number of symbols in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1Ndl_symbolTDDNii=1Ndl_symbolTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in symbol i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆tℎ (l and h, low and high; where low is the timing ∆c−W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl_symbol slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×Nsymbol Ndl_symbolsTDD

and Nsymbol=14 for 15 kHz SCS, Nsymbol=28 for 30 kHz SCS and Nsymbol=56 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.