3GPP TSG-RAN WG4 Meeting # 114											R4-2501896

Athens, Greece, Feb. 17th ‒ Feb. 21st, 2025

Agenda item:			7.6.2

Source:	Nokia

Title:	Discussion on methods for reducing the number of UE Rx chains.

Document for:	Discussion

# Introduction

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below relevant for in-gap requirements:

#### Issue 2-4: Signalling related

WF:

Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting.

#### Issue 3-2: Discussion on maximum power level of in-gap interference

Open Issue:

For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS

#### Issue 2-7: Others

Open Issue:

RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s

In this contribution we present aspects that need to be agreed to discuss requirements for the fragmented carrier aggregation operation. Recapturing the objective, we highlight the notable considerations for the study that matter if we discuss methods for reducing the number of UE Rx RF chains.

Discussion

Fragmented Carrier Aggregation and RRM considerations

As specified in the objectives of the SI, the aim is to have the UE to operate in a fragmented carrier aggregation configuration using a minimum number of RX chains.

During the discussion several aspects have been discussed while some RRM related issues haven’t been addressed yet. The understanding is that fragmented carriers is to be used in a CA combination with or without PCell. Additionally, the work currently only addresses a total of 2 CCs as part of the fragmented CA configuration:

- PCell + SCell, or

- SCell + SCell.

From RRM perspective there are some open questions. The main questions to be addressed are:

- Will the network explicitly configure the UE for fragmented carriers or is the UE allowed to autonomously switch RF configurations?

One issue to address is how to operate the SCell being part of a fragmented CA configuration. From network point of view (and UE as well) it is not efficient to apply a try and error approach when it comes to adding the SCell. Hence, it does not seem very attractive that network try to add an SCell on a fragmented carrier just to realize that it fails due to in-gap interference being too high. Hence:

- Is there a need for in-gap measurements to ensure proper network operations?

For example, in Fragmented CA, the UE is receiving both CCs and the in-gap interferer, ideally with one RX chain. Since, single Rx chain is being used, measurements on both CCs and the in-gap interferer can be performed, at least to some extent, in parallel. Therefore, it could be assumed that a UE should be able to measure the in-gap interference once configured with such CA configuration.

- Need for reporting of any in-gap interference to the network?

As mentioned, it will likely not be attractive from system point of view to design a feature in which the CA operations would rely on a try and error approach when it comes to adding an SCell. Considering this discussion, it may be good to discuss when, if any, in-gap interference measurements are to be performed (and reported). Hence:

- When would in-gap measurements be performed?

For example, such measurements could be done before or after SCell addition, before SCell activation or some other point.

Considering above discussion it would also be good to consider:

- Is there a need for any Fragmented CA fallback procedures?

For example, would such fallback be a configured fallback (by network) or a UE autonomous fallback?

For fragmented carriers, it is evident that a fallback from fragmented CA operation may occur if the in-gap interference exceeds the UE tolerable power level, preventing the device from maintaining acceptable performance on one or both CCs. For the case that this limit is surpassed, i.e. the power spectral density of the in-gap interferer is too high, it would be good to discuss and clarify how UE behaves and e.g. fallback solutions. These solutions can either be fully controlled and configured by the network or determined by the UE itself.

From network point of view, SCell should be under network control. In fragmented CA, there are one or two SCells. If the in-gap interference exceeds a tolerable limit, decoding of the wanted carriers will be affected, and it should be discussed which SCell should be chosen for continuing the connection in a fallback procedure.

There is currently no defined fallback procedure for the case when in-gap interference exceeds a UE’s limit.

If UE experiences high in-gap interference, this will likely have negative effect on the demodulation performance of the two non-contiguous intra-band carriers.

High in-gap interference can have negative impact on the demodulation performance of the two non-contiguous intra-band carriers.

For the fallback procedures, we would at least have the following two options:

- Network controlled: Since the network is in control of adding and removing SCells, an in-gap interference reporting for making the decision would be beneficial. The network would need to know the in-gap interference power level through measurement reporting.

- Pre-configured fallback: A pre-configured fallback procedure can be devised so that the UE autonomously decides whether or not to drop a CC.

The fallback from fragmented CA can be both network controlled or preconfigured.

The fragmented carrier pairs could form PCC+SCC or SCC+SCC. In the case of PCC+SCC, maintaining the performance of PCC unaffected by the in-gap interference is essential. Therefore, there should be a  mechanism to protect the PCell.  In case of SCC+SCC there could be more relaxation and the UE can have autonomy in choosing the dropped CC.

If one of the CCs in a fragmented CA configuration is a PCC, maintaining it must have higher priority than the other SCC if fallback is needed. In case both CCs in a fragmented CA configuration are SCCs, the fallback can be any of the SCCs.

Syntax for defining fragmented carriers

We find it necessary to clearly distinguish the radio configuration of the non-contiguous intra-band carrier aggregation between the modes of operation related to using the partially shared RF architecture and the fully shared RF chain at the UE side. It must be agreed that configuring the UE using the fully shared RF chain includes the operation at the UE as identified in this study including a risk of performance degradation.

The currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

The currently undefined NR CA bandwidth class ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two and four (should the concept evolve beyond the 2CC in scope) fragments are shown in the figures below.

Fig.  3: Example of CA_n25(2F)

Fig.  4: Example of CA_n25(4F)

Conclusion

In this contribution we presented aspects that needs to be agreed to discuss requirements for the fragmented carrier aggregation operation. Additionally, we discussed the possible RRM aspects related to introduction of fragmented carrier CA.

We observe and propose following:

- There is currently no defined fallback procedure for the case when in-gap interference exceeds a UEs limit.

- high in-gap interference can have negative impact on the demodulation performance of the two non-contiguous intra-band carriers.

- The fallback from fragmented CA can be both network controlled or preconfigured.

If one of the CCs in a fragmented CA configuration is a PCC, maintaining it must have higher priority that other SCC if fallback is needed. In case both CCs in a fragmented CA configuration are SCCs, the fallback can be any of the SCCs.

The currently undefined NR CA bandwidth class ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

- References

- R4-2420519, WF on NR FR1 DL fragmented carriers study, MediaTek Inc.

- R4-2419454, Discussion on Fragmented Carrier reduction of Rx RF chains, Nokia

- R4-2418146, Topic summary for [113][124] FS_NR_DL_Frag_Carrier, MediaTek Inc.