3GPP TSG-RAN WG4 Meeting #114	R4-2501900

Athens, Greece, 17th February – 21st February 2025

Agenda item:	7.18.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussion on BS demodulation for NR MMSE-IRC.

Document for:	Discussion

- Introduction

In the last meeting RAN4 #113, it was discussed that RAN4 demodulation spec on NR MMSE-IRC. We discussed the factors for simulation assumption [1]. This contribution proposes our views based on our simulation results [2].

- Discussion

According to RAN4#113 WF [1], the status is noted as below,

Number of interferers for 2Rx - Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain. Channel bandwidth - Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz MCS - Consider the following options for performance evaluation and as candidates for further down-selection: o For 2Rx: - Option 1: MCS 2 - Option 2: MCS 16 o For 4Rx: - Option 1: MCS 16 o For 8Rx: - Option 1: MCS 20 - Option 2: MCS 16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration. Waveform – whether to cover DFT-s-OFDM - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

We performed the simulations as described in R4-2501901[2] by choosing parameters for each element so that the SNR would not be extremely poor.

Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

RAN4 has agreed to define the requirements for NR MMSE-IRC with INR + SNR approach. We support option2.

Proposal 1: Use the SNR based requirement definition.

- Conclusion

This contribution states general aspects of NR BS MMSE-IRC. The summary is follows.

Proposal 1: Use the SNR based requirement definition.

References

- R4-2419780, Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek, RAN4#113

- R4-2501901, Simulation results for NR MMSE-IRC BS demodulation, NTT DOCOMO, INC., RAN4#114