3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501939

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.12.1.2

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, and the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the UL MIMO are shown below:

Issue 4-1: whether preclude the single antenna port related requirement

Agreement:

- FFS

Issue 4-2: ULFPTx mode for UL-MIMO

WF:

- FFS

- Companies are courage to analyze the additional specification impact and benefit if ULFPTx mode for ATG is introduced.

There are still many open issues that need to be addressed. The following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

For ATG UE UL-MIMO, in the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors, the uplink full power transmission (ULFPTx) mode is applicable. From Rel-16 onwards, with the support of ULFPTx mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full-power PA in full power mode. This ensures that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power. For ATG UE, the power is based on the declaration, and the declaration is per band, not per layer [3]. Without the ULFPTx support, UE needs a new signaling to inform the network when it is transmitting at full power mode.

An example of ATG UE ULFPTx mode was proposed in [4], which could be adopted into the specification.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

- ULFPTx mode should be supported for ATG UL-MIMO

### Single antenna port related requirement for ATG UE

In the last meeting, one company mentioned that the single antenna port was not useful due to CDD being used. Unfortunately, that is not correct. CDD is a transmit diversity mechanism implemented by applying a different phase delay (cyclic phase delay) for each OFDM subcarrier.

Using CDD is not always better than single antenna operation, and it depends on the specific channel conditions. If the channels between the multiple antennas and receiver are highly correlated, e.g. in line-of-sight (LoS) or near LoS environments. More specifically, the downside of CDD is that it can degrade performance in such environments since the artificial phase shift between the two antennas can create a deep fade in such conditions.

Anyhow, regardless of the conditions of the propagation channel, as stated in the previous session, The ATG UE UL-MIMO is possible to be scheduled for a single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1. Therefore, from the completeness perspective, the single antenna requirement still needs to be specified for ATG UE UL MIMO.

- Apply single antenna port-related requirements for ATG UE.

# Conclusion

In the previous sections, we made the following observations:

Based on the discussion in the previous sections, we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	Apply single antenna port-related requirements for ATG UE.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2420338, WF on Rel-19 ATG UE RF requirements

- TS 38.306 v18.3.0

- R4-2415897, ATG UE RF requirements for UL MIMO, LG Electronics.