# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501953

# Athens, Greece, 17 – 21 February 2025

# Title:	CA_n40A-n41A Second MSD test point

# Agenda Item:	6.12.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document proposes the cross-band isolation second MSD test point requirements for power class 3 (PC3) and PC2 CA_n40A-n41A based on the agreed Way Forward (WF) [1]. This test point is the only allowed exception to the current cross-band isolation rules [2] due to regional frequency allocation considerations.

# Discussion

## Background

We reproduce in Table 1 the agreed 2nd MSD test point for CA_n40A-n41A with non-simultaneous Rx\Tx from [1] and highlight in yellow the “deviations” from the agreed MSD test rules for cross-band isolation [2].

Table 1: PC3 (top) and PC2 (bottom) 2nd test points for CA_n40A_n41A from [1].

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC3 MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [11.8] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [28.6] | ACLR2 |

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC2 MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [14.4] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [31.6] | ACLR2 |

These points do not meet the agreed rules [2] for the following reasons:

- Band n40 is not configured with the highest supported CBW specified in the CA_40A-n41A for BCS4/5 in Table 5.5A.3.1-1i.

- The UL bands are not configured closest to the affected DL bands.

- These exceptions can be seen in the measured power amplifier output spectrum of Figure 1.

At RAN4 meeting #113, proponents clarified that the justifications for these exceptions are related to regional frequency band allocation considerations specific to Band n40 and Band n41 deployments. Such exceptions are allowed for the DL band CBW, but not currently agreed for the UL band CBW configuration. We believe a it is necessary to add a footnote to these 2nd MSD test points to clarify that RAN4 is not introducing an operator-specific requirement, but rather an exception due to regional frequency band allocations.,

Proposal 1: To clarify that the exceptional deviation from the agreed MSD test point guidelines is not due to operator specific spectrum holdings, but rather due to considerations on regional spectrum allocation, attach the following new footnote to the PC3 and PC2 2nd MSD test points for CA_n40A-n41 cross-band isolation interference.

“Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations.”

In the following section, we present power amplifier measurement results for PC3 based on previous assumptions for PA calibration assumptions and RF Front-End parameters [3,4,5]. The PC2 MSD requirements are simplified by adopting the 1UL CC HPUE MSD simplification look-up table concept of [6].

## PC3 MSD measurement results

The Band n41/Band n40 PA output spectrum emissions are shown in Figure 1 left/right respectively.

Figure 1: PA PC3 output spectrum, left: Band n41 100MHz Lcrb=270(RBstart=0) Fc=2565MHz, right: Band n40 50MHz Lcrb=128(RBstart=5) Fc=2345MHz.

The corresponding PC3 Band n40/band n41 MSD requirements are summarized in Table 2.

Table 2: PC3 2nd test points cross-band isolation MSD requirements for CA_n40A_n41A.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | PC3 MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [5.7] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [18.4] | ACLR2 |

## PC2 MSD

The PC2 MSD requirements are evaluated using the 1UL CC HPUE MSD simplification look-up table (LUT) reproduced in Table 3 from [6].

Table 3: 1UL PC21Tx , PC22Tx , PC1.5 LUT [6].

| PC3 MSD (dB) | PC21Tx (dB) | PC22Tx (dB) | PC1.5 (dB) |
| --- | --- | --- | --- |
| <1 | 0.6 | 0.9 | 2.0 |
| 1 | 0.8 | 1.2 | 2.7 |
| 2 | 1.2 | 2.1 | 4.1 |
| 3 | 1.6 | 2.7 | 5.1 |
| 4 | 1.8 | 3.3 | 5.9 |
| 5≤ <7 | 2.0 | 4.0 | 7.0 |
| 7≤ ≤9 | 2.5 | 5.0 | 8.0 |
| ≥10 | 3.0 | 6.0 | 9.0 |

For CA_n40A-n41A, and according to proposal 1 [6], we propose only the MSD requirements for PC2 achieved via 1Tx:

  - Band n40 PC2 1Tx MSD = PC3 MSD + PC21Tx = 18.4 + 3 = 21.4dB

  - Band n41 PC2 1Tx MSD = PC3 MSD + PC21Tx = 5.7 + 2 = 7.7dB

Proposal 2: Consider adopting the following PC3 and PC2 Band n40/n41 cross-band isolation MSD requirements for CA_n40A-n41A with simultaneous Rx\Tx. PC2 MSD requirements assume that PC2 is achieved via 1Tx.

Table 4 PC3 MSD for simultaneous Rx\Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41X | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [5.7] | >ACLR2 |
| n41 | n40X | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [18.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

Table 5 PC2 1Tx MSD for simultaneous Rx\Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [7.7] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [21.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution summarizes our views on the agreed cross-band isolation 2nd MSD test points for CA_n40A-n41A with simultaneous Rx\Tx.

Proposal 1: To clarify that the exceptional deviation from the agreed MSD test point guidelines is not due to operator specific spectrum holdings, but rather due to considerations on regional spectrum allocation, attach the following new footnote to the PC3 and PC2 2nd MSD test points for CA_n40A-n41 cross-band isolation interference.

Proposal 2: Consider adopting the following PC3 and PC2 Band n40/n41 cross-band isolation MSD requirements for CA_n40A-n41A with simultaneous Rx\Tx. PC2 MSD requirements assume that PC2 is achieved via 1Tx.

Table 4 PC3 MSD for simultaneous Rx\Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41X | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [5.7] | >ACLR2 |
| n41 | n40X | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [18.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

Table 5 PC2 1Tx MSD for simultaneous Rx\Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [7.7] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [21.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

# References

- R4-2420351 WF on simultaneous Rx-Tx for CA_n40A-n41A, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, Huawei.

- R4-2501963 Input to RAN4 PRD on rules for 1UL CC MSD requirements, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc.

- R4-2300916 CA_n40-n41 non-simultaneous RxTx PC3 PC2 MSD, 3GPP TSG-RAN WG4 Meeting#106, Athens, Greece, Skyworks Solutions, Inc.

- R4-2305741 SUL_n41_n97 and CA_n40-n41 simultaneous RxTx MSD, 3GPP TSG-RAN WG4 Meeting#106be, Electronic meeting, Skyworks Solutions, Inc.

- R4-2405955 CA_n40A-n41C MSD, 3GPP TSG-RAN WG4 Meeting#106be, Changsha, China, Skyworks Solutions, Inc.

- R4-2501955 1UL MSD simplifications rules for Rel-19, , 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Nokia.