3GPP TSG-RAN WG4 Meeting #114	R4-2501958

Athens, Greece, 17th – 21st February 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 2

Agenda item:	7.20.1

Document for:	Discussion

Introduction

In RAN4#106, a new SI ‎[1] “New SID for NR_AIML_air_Phase2” was agreed to continue the study on the remaining issues of AI/ML air interface phase1. In this new SI, following scope was agreed for RAN4:

- Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In this paper, we share our views on the following topics:

- Generalization, number of tests and non-stationary conditions

- Post-deployment validation

LCM related aspects are treated in our accompanying paper ‎[2] for RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface ‎[3].

Discussion

On Generalization, number of tests and non-stationary conditions.

At the RAN4#112 meeting, it was agreed to introduce the new definition of the testing goals that is defined in the feature/functionality level ‎[4]:

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Based on the recent agreements in the RAN WG1 and WG2, we propose the following possible illustration of interplay in between AI/ML-enabled Feature functionalities (to be interpreted here as indicating the set of parameters and their values as supported for a particular Feature configuration based on the UE capabilities), Associated ID and applicability of the functionality (as discussed in RAN2), shown in Figure 1. The logical ML models are shown for illustration purposes only, as it was agreed in RAN4 that the requirements will be defined on the level of UE capabilities and functionalities, even though they can be provisioned by one or several UE-side ML models, and this is left up to UE implementation.

Figure 1: Illustration of simplified relationship in between AI/ML-enabled Feature (NW-UE), functionalities (UE), Associated IDs (NW) and logical ML models (UE) based on recent RAN1 and RAN2 agreements.

The point of the diagram in Figure 1 is to understand how the requirement/test parameters/configurations will need to be defined in RAN4 so that the UE-side AI/ML-enabled features functionalities can be (adequately) tested. Following the diagram, the test configurations parameters for test requirements that needs to be specified can be split at least into the following groups:

- Parameters that are defined by the Feature functionality configurations based on the UE capabilities

- Parameters that are defined by the Associated IDs (i.e., NW-related parameters including NW side additional conditions) that help the UE to determine/check the functionality applicability (RAN2) and enable consistency between training and inference conditions (RAN1) for the UE-side AI/ML-enabled Features functionalities.

- Other/additional parameters that cannot be defined neither by the Feature functionality/configuration nor include in the Associated ID e.g., propagation conditions/channel model, UE-side configurations/additional conditions, etc.

Correspondingly, it can be expected that every specified AI/ML-enabled Feature, with functionalities/configurations from group (1) specific to it, shall be tested i.e., at least one requirement/test per functionality shall be defined. However, the total number of requirements per UE Feature configuration will be dependent on whether an explicit list of Associated ID will be specified or not. If yes, then it will be necessary to define requirements per each of those because which of the Associated IDs are applicable to a UE Feature configuration will be UE-specific. For example, some UEs might support best beam prediction for certain gNB beam codebooks, but others - not. Then, the explicit list of Associated IDs will not be specified, and the definition of requirements becomes more challenging because potentially it could be UEs without any applicable Associated IDs leaving the functionality without test. This should not be the case. Therefore, at least a minimum set of representative Associated ID(s), NW and UE vendor agnostic, shall be considered as mandatory/default and, be applicable and testable for the ML-enabled Feature configurations.

The way how test configurations/parameters and requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized in RAN1 and RAN2.

Each specified AI/ML-enabled Feature shall have RAN4 requirements and UE needs to pass at least one test for specific sub-set of applicable Feature configurations (matching functionalities) as part of conformance testing.

A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

The change of the Associated ID (parameter group (2) in the list above) may be indicated in a RRC reconfiguration message because UE will need to re/check the applicability/compatibility of the Associated ID to its AI/ML-enabled Feature. Thus, the UE should have sufficient time to check the new conditions and adapt if needed e.g., to switch its underlining ML models if needed or to report that such configuration is not applicable at all. Therefore, we do not see a need to define requirements that will cover a possible of change Associate ID during the test run/sequence.

A considerable change of NW-side conditions that require the change of Associated ID may require RRC reconfiguration for the UE.

RAN4 does not need to define performance requirements that imply the change of NW configuration/Associated ID during the test.

Lastly, we have a group of parameters/configurations that are not explicitly defined neither by the feature/functionality nor by the Associated ID (parameters in group (3) in the list above). For example, the capability of the UE to maintain the minimal level of performance in different radio propagation conditions is the core question of generalization (and valid problem regardless of the use/non-use of ML algorithms) and should be ensured by the sufficient test/requirement coverage. We envision two open issues related to such parameters:

- What should be the number of additional test requirements per given UE AI/ML-enabled Feature configuration and selected Associated ID(s) combination?

  - This will depend, for example, whether the deployment, UE configurations or channel model will be specified by those or not.

  - On top of that, it will be necessary to study how the performance of the AI/ML-enabled Feature is sensitive to variations in those test parameters in group (3), i.e., how well the functionality generalizes to changes in those.

- Are there any potential degradations in the performance when the change of test parameters/configurations happens dynamically, i.e., in non-static way?

  - We cannot expect that RRC reconfiguration happens when additional conditions / parameters / configurations (e.g. channel conditions) change. UE should use its internal mechanisms/implementation to ensure good performance of the functionality, e.g., by changing to the proper underlying model within functionality to match the conditions.

Regarding the first issue above, we think that more clarity from other WGs and a study of generalization UE capabilities to different scenarios will be needed in RAN4. It will still take some time.

The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in per use-case manner.

RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Regarding the second issue above, when additional conditions might change without the change of the AI/ML-enabled Feature configuration and Associated ID, it brings us to the question of non-static test conditions. At the previous RAN4 meetings, we have already discussed such tests and definition of non-static conditions:

Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

The current RAN4 (and to large degree also RAN1) assumptions for UE feature requirements, testing and evaluations are based on static scenarios and configurations, which correspond to stationary radio channel conditions. In our understanding, for the ML-enabled features to be introduced in NR Release 19, these assumptions are still valid and can be used as baseline. Additional assumptions and conditions for testing e,g, in the form of non-static scenarios (for non-stationary radio channel conditions) can be introduced on a use case basis.

The static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID shall be assumed as baseline for RAN4 requirements purposes.

We still believe that it is necessary to emphasize the difference in the discussion of generalization aspects and non-static conditions:

Non-static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID should be discussed separately from the study of AI/ML-enabled Feature generalization to different but static conditions (legacy way of testing).

Post deployment testing

The following way forward on post-deployment testing was agreed at RAN4#112bis [4]:

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

Discussion on Option 1

The first FFS under Option 1 is addressing where, and implicitly the test conditions under which, the post-deployment testing would be carried out. As a reminder, the original target for the potential introduction of the new, post-deployment testing procedure was to avoid expensive, extensive, and repeated conformance testing in laboratory conditions, of any new UE implementations, due simply to an update in the underlying ML models. We believe this in the lab testing is not practical solution for the intended purposes. The usual RAN4/5 based, in the lab, conformance testing is mandatory, regardless of the UE implements or not ML-enabled Features.

In case if Option 1 is agreed, the post-deployment testing procedures should be designed to allow testing in the field, in live radio networks, without the need for any additional lab testing.

For the second FFS under Option 1, it is likely that some testing (e.g. limited to certain AI/ML-enabled Features configurations) can be done at the UE device, given the increase in compute power and evolution of the hardware software platforms used. However, whether this testing can be performed on-device along with inference for another active configurations/functionality(ies) or not, strongly depends on at least the time-scale at which these AI/ML-enabled Features operate, therefore it is use case dependent. The best way forward is that RAN4 does not make any general assumptions for on-device parallel execution of the test and the execution of other active configurations/functionality(ies) and the corresponding test procedures shall not assume it.

RAN4 should not make any general assumptions for on-device parallel execution of the test and the execution of other active configurations/functionalities. It should be handled per use case basis.

The feasibility of Option 1 is strongly related to the considerations on where the testing is performed and the UE capabilities of executing tests. The in-field and on-device post-deployment testing of certain AI/ML-enabled Features configuration may be assumed to be feasible for Release 19 use cases.

Another aspect in Option 1 is whether the potential on-device test (executed in the UE), will be executed with or without assistance or control from the NW-side. Obviously, all UE-side ML model related LCM operations, including potential testing and validation execution, are UE-vendor implementation specific. Nevertheless, for Release 19 use cases and AI/ML-enabled Features, the required input data for the on-device testing procedure is generated based on radio measurements and radio measurements are configured by the NW. In case the on-device testing procedure does require any reconfiguration from NW point of view and does not disrupt the inference for another active configurations/functionality(ies), it can be executed at any time the UE-side LCM algorithm decides that it is necessary and possible. Under any other conditions, the UE would need to be configured to indicate to the NW that such testing is required/imminent and/or when the testing needs additional configuration from NW (e.g. measurement resources).

Furthermore, it is mentioned under Option 1 the possibility to capture ML model input (and if needed other test data such as ground truth) during conformance testing; this stored data can later be used to validate new or updated models on-device in the field. This alternative comes with several complications, among others:

- Collection (configuration, capture and storage) of necessary data during lab testing. This is a new procedure, needs to be adopted by TE vendors and testing labs, while the details remain AI/ML-enabled feature and UE device vendor specific.

- The management and use of the collected data in the field, by various network operators. This requires full- and large-scale inter-operability (with fully specified mechanisms and data formats to be exchanged) between UE vendors, NW vendors and network operators.

- The transfer of the data to the UE for executing the tests in the field. This requires a standardised transfer/delivery mechanism and standardised use of the data by the UE, in order to ensure the reliability and meaningfulness of the test results.

The solution where captured ML model input data (and if needed other test data such as ground truth) during conformance testing is used later for in-field and on-device testing has a very large specification impact and does not seem practical in the timeframe of Release 19 WI.

The solution where captured ML model input data (and if needed other test data such as ground truth) during conformance testing is used later for in-field and on-device testing should not be considered in for release 19 use-cases.

Discussion on Option 2

The FFS under Option 2 addresses the role of the monitoring procedures. While the details of such procedures are use- case specific and are being worked out in RAN1 and RAN2, we consider the NW-side monitoring of the UE performance as the baseline for the post-deployment testing purposes. The main reasons for this consideration are that:

    - This enables solutions for UE-device/vendor independent procedure at the NW side and

    - RAN4 testing of the performance monitoring procedure can be part of the core requirements.

Post-deployment solutions based on NW-side monitoring of the UE performance as the baseline enables UE-device/vendor independent procedure at the NW side and allow RAN4 testing of the performance monitoring procedure as part of the core requirements.

Comparison of Option 1 and Option 2

Comparing Option 1 and Option 2 for the design of an in-field and on-device post-deployment testing procedure, Option 2 has less additional impact on the specifications i.e., in addition to the changes introduced by RAN1 and RAN2 for performance monitoring purposes, while allows for a leaner procedure on both the UE side and the NW side.

RAN1 to confirm that Option 1 feasibility depends on the availability of signalling from the UE about the changes/updates in AI/ML-based feature that should be defined in the other WGs.

Option 2 of post-deployment testing should be adopted based on NW-side monitoring.

Furthermore, we believe that a combination of Option 1 and Option 2 is also a viable way forward. The Option 1 ensures that the functionality configuration can be tested before it is activated (for inference and reporting) by the NW. In Option 2 the performance monitoring at the NW-side is used to detect performance changes of the already activated UE-side AI/ML-enabled feature configurations. The combination of these mechanisms, supported by the signalling from the UE about the changes/updates in AI/ML-based feature, can ensure a high reliability of the post-deployment testing, hence increase the trustworthiness of the AI/ML operation in live networks.

RAN4 to consider a combination of Option 1 and Option 2 as the best way forward solution for the post-deployment testing.

Dependencies on RAN1 and RAN2

From the discussion above on the Option 1 and Option 2 solutions, it is obvious that handling of post-deployment issue is hardly possible without involvement and clarifications from the other WGs. Therefore, we see it important to share potential issues of post-deployment conformance and testing of AI/ML enabled functionalities with RAN1/2 so they could be aware about and consider those in the discussions and designs. At least the following aspects should be highlighted:

- RAN4 has a concern that modified/updated/new AI/ML-enabled Feature (e.g. changes of the underlying ML models) can be used in the field without passing conformance testing explicitly.

  - The value of conformance testing might be compromised if the AI/ML-enabled Feature (or functionality/ configurations) that passed RAN4 conformance testing are not actually used or available in the devices in the real networks (i.e., after deployment in the field).

- The changes in the AIML functionality causing a change in the performance could happen unexpectedly to the network and require additional signalling, e.g., whether the update was tested beforehand or not.

- Whether the monitoring procedures defined in the other WGs can be used for testing in RAN4, e.g., whether the metrics are aligned, and necessary reporting is available.

RAN4 needs to discuss whether the models that were used passed conformance testing shall be kept in the device.

RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML-enabled Feature when designing signalling procedures for functionality/configuration for monitoring/activation/fallback/applicability, for the UE devices already deployed in the field.

Conclusion

In this contribution, we addressed following topics:

- Generalization, number of tests and non-stationary conditions

- Post-deployment validation

Following Observations and Proposals were made in this paper:

Observation 1: The way how test configurations/parameters and requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized in RAN1 and RAN2.

Proposal 1: Each specified AI/ML-enabled Feature shall have RAN4 requirements and UE needs to pass at least one test for specific sub-set of applicable Feature configurations (matching functionalities) as part of conformance testing.

Proposal 2: A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature.

Observation 2: A considerable change of NW-side conditions that require the change of Associated ID may require RRC reconfiguration for the UE.

Proposal 3: RAN4 does not need to define performance requirements that imply the change of NW configuration/Associated ID during the test.

Observation 3: The number of additional requirements per given UE AI/ML-enabled Feature and Associated ID will depend on their concrete definitions of the functionalities/configurations in the other WGs and the generalization capabilities of the UE implementation solutions, and both require further study in per use-case manner.

Proposal 4: RAN4 needs to study the variation in the AI/ML-enabled Feature performance due to the change in test conditions (e.g., propagation conditions) to conclude about the final number of requirements/tests per given AI/ML-enabled Feature and Associated ID combination.

Proposal 5: The static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID shall be assumed as baseline for RAN4 requirements purposes.

Proposal 6: Non-static radio scenarios/conditions for a given UE AI/ML-enabled Feature and/or Associated ID should be discussed separately from the study of AI/ML-enabled Feature generalization to different but static conditions (legacy way of testing).

Proposal 7: In case if Option 1 is agreed, the post-deployment testing procedures should be designed to allow testing in the field, in live radio networks, without the need for any additional lab testing.

Proposal 8: RAN4 should not make any general assumptions for on-device parallel execution of the test and the execution of other active configurations/functionalities. It should be handled per use case basis.

Proposal 9: The feasibility of Option 1 is strongly related to the considerations on where the testing is performed and the UE capabilities of executing tests. The in-field and on-device post-deployment testing of certain AI/ML-enabled Features configuration may be assumed to be feasible for Release 19 use cases.

Observation 4: The solution where captured ML model input data (and if needed other test data such as ground truth) during conformance testing is used later for in-field and on-device testing has a very large specification impact and does not seem practical in the timeframe of Release 19 WI.

Proposal 10: The solution where captured ML model input data (and if needed other test data such as ground truth) during conformance testing is used later for in-field and on-device testing should not be considered in for release 19 use-cases.

Observation 5: Post-deployment solutions based on NW-side monitoring of the UE performance as the baseline enables UE-device/vendor independent procedure at the NW side and allow RAN4 testing of the performance monitoring procedure as part of the core requirements.

Proposal 11: RAN1 to confirm that Option 1 feasibility depends on the availability of signalling from the UE about the changes/updates in AI/ML-based feature that should be defined in the other WGs.

Proposal 12: Option 2 of post-deployment testing should be adopted based on NW-side monitoring.

Proposal 13: RAN4 to consider a combination of Option 1 and Option 2 as the best way forward solution for the post-deployment testing.

Proposal 14: RAN4 needs to discuss whether the models that were used passed conformance testing shall be kept in the device.

Proposal 15: RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML-enabled Feature when designing signalling procedures for functionality/configuration for monitoring/activation/fallback/applicability, for the UE devices already deployed in the field.

- References

- RP-243245, “New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm Incorporated, RAN#106 meeting, 9 – 12 December 2024, Madrid, Spain.

- R4-2501956, “On General Aspects of AI/ML for NR Air Interface”, Nokia, RAN4#114, 17 – 21 February 2025, Athens, Greece.

- RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#106 meeting, 9 – 12 December 2024, Madrid, Spain.

- R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.