3GPP TSG-RAN WG4 Meeting #114                                                              R4-2501968

Athens, GR, 17th – 21st February, 2025

Source:	Samsung

Title:	On the potentially new requirements for SBFD operation for FR1 and FR2-1

Agenda item:			7.23.2.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111 to #113, further agreements are achieved on the potentially new requirements for SBFD operation [6] [8] [10] [12]. In this contribution, accordingly, we would like to further provide our analysis and viewpoints on the potentially new BS RF requirements for SBFD operation for FR1 and FR2-1.

# 2. Transient period requirement for SBFD

### 2.1 Existing agreements

From Rel-18 study, the requirement of transmitter transient period will be introduced for the transmitter mode switching between SBFD and non-SBFD or SBFD reconfigurations if needed. RAN4 has confirmed that the transition period is needed between the non-SBFD slot and SBFD slot and vice versa. Furthermore, RAN4 has concluded that the transient period shall be introduced to BS in SBFD symbols.

For transmitter transient period between SBFD and non-SBFD or SBFD reconfigurations if needed, the requirement shall be introduced to BS in SBFD symbols/slots, by defining the transient period as the time period which the transmitter is changing from the SBFD operation to non-SBFD operation or vice versa, or during SBFD reconfigurations. Regarding the transition period requirement, RAN4 mainly focus on the transition period related with SBFD. Based on the RAN4 study, between the non-SBFD slot and SBFD slot and vice versa, a transition period is needed. If the SBFD configuration between adjacent SBFD slots is the same, then no transition period is needed.

The following agreements related to the transient period between SBFD and non-SBFD are achieved in RAN4#112 [8]:

#### <Agreement achieved in RAN4#112 [8]> #### Issue 2-1-1: Requirement for transient period between SBFD and non-SBFD - Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. #### Issue 2-1-2: Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. #### Issue 2-1-3: Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. #### Issue 2-2-1: Requirement for transient period between different SBFD configurations - Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations.

And the following way forward is achieved in RAN4#112bis [10] and RAN4#113 [12].

#### <Agreement achieved in RAN4#112bis [10]> #### Issue 2-1-1: Transient period requirement applicability on frequency/antenna Way Forward: - FFS the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in R4-2416498. #### Issue 2-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS location of transient period between SBFD and non-SBFD: - Option 1: No need to discuss and decide whether the transmitter transient period shall be located within the SBFD symbol or not. - Option 2: Place the transient period outside of both SBFD and non-SBFD slots. - Option 3: The location of the transient period between SBFD and non-SBFD shall be within the SBFD symbol). <Agreement achieved in RAN4#113 [12]> #### Issue 3-1-1: Transient period requirement applicability on frequency/antenna Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases. #### Issue 3-1-2: Location of transient period between SBFD and non-SBFD Way Forward: - FFS whether the transmitter transient period shall be located within the SBFD symbol or not: - FFS it is an implementation issue and no RAN4 standard impact

It should be noted that RAN1 agreements as below, for both one or two TDD-UL-DL pattern(s) configured, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. And the SBFD can be flexibly configured from/to any symbol of a certain slot.

<Agreements from RAN1#116> Agreement A slot can consist of SBFD symbols and non-SBFD symbols. Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details <Agreements from RAN1#117> Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. <Agreements from RAN1#118> Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

### 2.1 Requirement for transient period between SBFD and non-SBFD

Next issue to be discussed for transient period between SBFD and non-SBFD is how to capture the agreed transient period requirement into core requirement. Based on the agreement “RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD”, here we would like to identify the necessary changes to clause 6.4.2 and clause 9.5.3.

#### 2.1.1 Location of transient period between SBFD and non-SBFD

In our previous contribution [7] [9], different cases (Case-A, B, C and D) are identified and denoted for four cases of transitions to/from SBFD symbols, which is captured in the agreement in RAN4#112 [8].

To better illustrate the Case-A, B, C and D, the following SBFD time-domain configuration is given, in which two TDD-UL-DL patterns are configured, e.g, DXXSU and XXXUU.

Figure 1: Illustration of four cases of transitions to/from SBFD symbols

Corresponding to the above Figure 1, the detailed configurations of SBFD symbols within the above TDD-UL-DL patterns (DXXSU and XXXUU) are provided by the following parameters:

| TDD-UL-DL pattern 1 (DXXSU) | SBFD starting slot index = 1 |
| --- | --- |
|  | SBFD starting symbol index within the starting slot = 0 |
|  | SBFD ending slot index = 2 |
|  | SBFD ending symbol index within the ending slot = 13 |
| TDD-UL-DL pattern 2 (XXXUU) | SBFD starting slot index = 0 |
|  | SBFD starting symbol index within the starting slot = 0 |
|  | SBFD ending slot index = 2 |
|  | SBFD ending symbol index within the ending slot = 13 |

The transient periods for SBFD UE and legacy TDD UE are illustrated in the below figure:

Figure 2: Detailed illustration for transient periods for SBFD UE and legacy TDD UE for four cases

For individual case, the detailed analysis and observation are provided:

Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol(s) |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in SBFD symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in non-SBFD DL symbol(s) |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

Based on the above analysis, we see the location of transient period is in TA_offset or guard symbols reserved in D/F-slot.

Proposal 1: In practical SBFD deployment, the location of transient periods should be located:

(1) Case-A: in TA_offset in SBFD symbol(s);

(2) Case-B: in guard symbol(s) reserved in SBFD symbol(s);

(3) Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s);

(4) Case-D: in TA_offset in non-SBFD UL symbol(s).

For the RAN4 specification impact from the location of transient period, we found there is no need specify the detailed location of transmitter transient period. Specifically, from RAN4 conformance requirement perspective, the test on the “OFF” power on the OFF period is performed (clause 6.4.2.4 in TS 38.141-1):

3)	Measure the mean power spectral density over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the antenna connector or TAB connector centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period – 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

In other words, based on gNB configured TA_offset and gurard symbols, the RAN4 conformance requirement (“OFF” power test) can be performed on the OFF period configured by gNB, which also requires no necessity of specifying the detailed location of transmitter transient period

Proposal 2: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

Particular for the cases in which the transient period is located in guard symbols reserved in Non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband. From RAN1 perspective, another issue is to decide whether the reserved guard symbol should span over the UL subband or the whole subband, and the relevant configuration for that. However, our understanding is this is within the RAN1 scope while has no impact on the RAN4 discussion for transient period requirement.

Proposal 3: For the cases in which the transient period is located in guard symbols reserved in non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband.

Since the discussion on transient period requirement has been discussed for several meetings, we suggest to send out an LS to confirm the above understanding if RAN4 can’t reach the agreement on the location of transient period.

Proposal 4: If RAN4 can’t reach the agreement on the location of transient period, RAN4 shall send out an LS to RAN1 to confirm the understanding. An example of LS is provided in the Appendix.

#### 2.1.2 Transient period requirement applicability on SBFD subbands

As we analysis in our previous contribution, different from the existing transient period for TDD operation (from transmitter OFF period to transmitter ON period or vice versa), the transient period for SBFD operation is more complex in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources: e.g., Case-A (SBFD to DL), the transmitter OFF-to-ON happens only on SBFD UL subband and guardband(s), while the transmitter may continue on the SBFD DL subband.

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements: e.g., Case-A (SBFD to DL), the transmitter OFF-to-ON happens only on the antenna elements in SBFD TX panel.

Observation 2: The transient period for SBFD operation is different from TDD operation in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

In Rel-18 RAN1/4 study, the separate TX/RX antenna array model as below, and three SBFD antenna configuration options (provided in annex A.5 in TR 38.858) have been studied:

Figure A.5-1: Separate-Tx/Rx antenna array model For evaluation and comparison between SBFD and legacy TDD, three options can be used. - SBFD antenna configuration option-1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD.

For SBFD antenna configuration option-1, as illustrated in the below figure, the SBFD-relevant BS transmitter ON-to-OFF could happens in TX and RX antenna panels respectively for Case B (SBFD to UL) and Case C (DL to SBFD), and the SBFD-relevant BS transmitter OFF-to-ON could happens in RX and TX antenna panels respectively for Case A (SBFD to DL) and Case D (UL to SBFD):

Case-D (UL to SBFD)

Case-B (SBFD to UL)

Case-C (DL to SBFD)

Case-A (SBFD to DL)

Figure 1: SBFD-relevant transmitter ON-to-OFF and OFF-to-ON transient period for SBFD antenna configuration Option-1

Case-B (SBFD to UL)

Case-D (UL to SBFD)

For SBFD antenna configuration option-2 and 3, even with different methods of usage of TXRUs and antenna elements (provided in annex A.5 in TR 38.858), the situation is simpler since the BS TX only happens in TX antenna panel. Take the below Option 2 (Method 2-1) as example, SBFD-relevant BS transmitter ON-to-OFF and OFF-to-ON only happens in the TX antenna panel, as illustrated below:

Figure 2: SBFD-relevant transmitter ON-to-OFF and OFF-to-ON transient period for SBFD antenna configuration Option-2 (Method 2-1)

By summarizing the above observations, the transient period for SBFD operation could be further summarized as:

Observation 3: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

#### 2.1.3 Specification implementation for transient period requirement applicability

Based on the agreement achieved in RAN4#113, 	RAN4 shall not define an SBFD-architecture dependent requirement, and the transient period between SBFD and non-SBFD is applicable to all four cases. Firstly, based on WF [10], it is agreed to further study the text proposal for the clause of transmitter transient period for SBFD, e.g., the example provided in our previous contribution [9, R4-2416498]. We have updated the general description for the clause of transmitter transient period for SBFD, by removing the controversial contents related to TX/RX antenna panels.

Proposal 5: The text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

# 3. Discussion on in-channel adjacent subband requirements

### 3.1 In-channel adjacent subband leakage ratio

In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio:

Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

The new requirement is considered to be needed to guarantee the TX leakage between the sub-bands. Based on the discussion, there are different views on how this requirement shall be defined, i.e., ACLR-like or similar to UE in-band emission requirement. It should be noted that the impact of self-interference has been guaranteed by OTA sensitivity requirement already, the in-channel adjacent subband leakage ratio requirement is introduced to guarantee the inter-site gNB-to-gNB CLI, especially if no coordination between gNBs. Based on Rel-18 study, for inter-site gNB-gNB co-channel inter-subband CLI modelling, RAN4 suggest the TX leakage can be modelled as gNB ACLR. Accordingly, we suggest RAN4 to continue this principle by define a ACLR-like requirement for in-channel adjacent subband leakage ratio.

Proposal 6: For in-channel adjacent subband leakage ratio (ICASLR), the requirement shall be defined similar to BS ACLR requirement.

Specifically, in the existing ACLR requirement, it is defined with a square filter of bandwidth equal to the transmission bandwidth configuration of the transmitted signal (BWConfig) centred on the assigned channel frequency and a filter centred on the adjacent channel frequency, where the assigned adjacent channel is either NR of the same BW or 5MHz E-UTRA. For in-channel adjacent subband leakage ratio in SBFD DU case, the assumed UL subband shall be considered as the adjacent subband to be measured, on which the square filter of subband bandwidth can be used.

Proposal 7: For SBFD DU case, the in-channel adjacent subband leakage ratio (ICASLR) requirement shall be defined based on the following configuration:

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio (ICASLR) limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

For DUD case, the existing ACLR requirement specified for the operation in non-contiguous spectrum can be used as baseline.

Proposal 8: For SBFD DUD case, the in-channel adjacent subband leakage ratio (ICASLR) requirement shall be defined by consdieirng the existing ACLR requirement in non-contiguous spectrum or multiple bands as baseline.

### 3.2 In-channel adjacent subband blocking

In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity:

Agreement: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

Based on our evaluation for FR1 UMa-Uma, even without any CLI handling mechanism, co-channel interference from neighboring BS is not significant as adjacent channel interference, for which the existing ACS requirement can be used to mitigate the co-channel gNB-to-gNB interference for more than 95% of the cases. For the remaining high interference cases, RAN1 has introduced gNB-gNB CLI-handling mechanism, by which the co-channel interference from neighboring gNB DL subband transmission could be greatly mitigated.

| Scenario 1: FR1 UMa-UMa |  |
| --- | --- |

Proposal 9: For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement.

# 4. Conclusion

In this contribution, we provided BS RF requirement for SBFD-capable BS, accordingly with the following observations and proposals obtained:

Transient period requirement for SBFD

Observation 1: For the identified four cases of transitions to/from SBFD symbols, the following timing behaviors are expected:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol(s) |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in SBFD symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in non-SBFD DL symbol(s) |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

Proposal 1: In practical SBFD deployment, the location of transient periods should be located:

(1) Case-A: in TA_offset in SBFD symbol(s);

(2) Case-B: in guard symbol(s) reserved in SBFD symbol(s);

(3) Case-C: in guard symbol(s) reserved in non-SBFD DL symbol(s);

(4) Case-D: in TA_offset in non-SBFD UL symbol(s).

Proposal 2: In RAN4 requirement, no need to specify the detailed location of the transmitter transient period (i.e., within the SBFD symbol or not), because in practice the length and location of TA_offset and guard symbols are provided by gNB configuration.

Proposal 3: For the cases in which the transient period is located in guard symbols reserved in non-SBFD DL symbol(s), i.e., Case-C, the explicit guard should be reserved to allow UE transition from receiving to transmitting, in which the transient period, two-way propagation delay shall be accommodated for UL subband.

Proposal 4: If RAN4 can’t reach the agreement on the location of transient period, RAN4 shall send out an LS to RAN1 to confirm the understanding. An example of LS is provided in the Appendix.

Observation 2: The transient period for SBFD operation is different from TDD operation in terms of:

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of frequency resources, and

- Transmitter OFF-to-ON and ON-to OFF happen in part(s) of antenna elements.

Observation 3: The transient period for SBFD operation could be further summarized as below, by elaborating the four different cases for different SBFD antenna configuration:

|  | Frequency regions for transition | SBFD antenna configuration Option-1 (Same panel area as TDD) | SBFD antenna configuration Option-2&3 (Twice panel area as TDD) | BS transmitter behavior during transition |
| --- | --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | SBFD RX antenna panel | N/A | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | SBFD TX antenna panel | SBFD TX antenna panel | BS transmitter OFF-to-ON |

Proposal 5: The text proposal is updated for the clause of transmitter transient period for SBFD:

=================== Start of Text Proposal ===================

#### 6.4.2B	Transmitter transient period for SBFD

##### 6.4.2.1B	General

Transmitter transient period for SBFD requirements apply only to SBFD operation of the BS.

The transmitter transient period for SBFD is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period is illustrated in figure 6.4.2.1-1.

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 6.4.2.1B-1: Transmitter transient period for SBFD

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD symbol to UL symbol) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL symbol to SBFD symbol) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL symbol to SBFD symbol) | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

=================== End of Text Proposal ===================

In-channel adjacent subband requirements

Proposal 6: For in-channel adjacent subband leakage ratio (ICASLR), the requirement shall be defined similar to BS ACLR requirement.

Proposal 7: For SBFD DU case, the in-channel adjacent subband leakage ratio (ICASLR) requirement shall be defined based on the following configuration:

| BS DL subband bandwidth BWSBFD,DL (MHz) | BS UL subband bandwidth BWSBFD,UL (UL) | Filter on UL subband and corresponding filter bandwidth | In-Channel Adjacent Subband Leakage Ratio (ICASLR) limit |
| --- | --- | --- | --- |
| BWSBFD,DL as configured by gNB | BWSBFD,UL as configured by gNB | Square (BWSBFD,UL) | TBD |

Proposal 8: For SBFD DUD case, the in-channel adjacent subband leakage ratio (ICASLR) requirement shall be defined by consdieirng the existing ACLR requirement in non-contiguous spectrum or multiple bands as baseline.

Proposal 9: For in-channel adjacent subband blocking (ICASB), the requirement shall be defined similar to BS in-band blocking requirement.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis

[5] R4-2409765, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411640, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112.

[8] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[9] R4-2416498, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#112bis.

[10] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.

[11] R4-2418098, “On the potentially new requirements for SBFD operation for FR1 and FR2-1”, Samsung, RAN4#113.

[12] R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, Samsung, RAN4#113.

# 6. Appendix – draft LS to RAN1 on SBFD transient period

3GPP TSG-RAN WG4 Meeting #114		R4-25xxxxx

Athens, Greece, 17th – 21st Feb, 2025

Title:	LS on the transient period for SBFD operation

Response to:	N/A

Release:	Rel-19

Work Item:	NR_duplex_evo-Core

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Wang, He (Jackson)

h0809.wang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

## 1	Overall description

On RAN4 requirements for SBFD transient period at gNB, RAN4 has made the following agreements

Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. Agreement: - RAN4 shall not define an SBFD-architecture dependent requirement. - Transient period between SBFD and non-SBFD is applicable to all four cases.

Particularly for the location of transient period, RAN4 has identified four cases of transitions to/from SBFD symbols as illustrated in Figure 1:

Figure 1: Illustration of four cases of transitions to/from SBFD symbols

Accordingly, the expected timing behaviours for four cases are provided in the below table, and illustrated in Figure 2:

|  | Expected timing behavior for DL/UL subbands respectively |  | Location of transient period (if any) |
| --- | --- | --- | --- |
| Case-A (SBFD symbol to DL symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from SBFD symbol to DL symbol. | N/A |
|  | UL subband | TA_offset is still required for SBFD UL subband, which shall be longer than the required transient period (10us for FR1 and 3us for FR2), so it is straightforward that the TA_offset can accommodate the required transient period. | Located in TA_offset in SBFD symbol(s) |
| Case-B (SBFD symbol to UL symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., guard symbols located in D/F-slot is required to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in SBFD symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the follow-up UL symbol, and there is no BS transient period issue at all | N/A |
| Case-C (DL symbol to SBFD symbol) | DL subband(s) | No transient period issue since the BS transmission is kept from DL symbol to SBFD symbol. | N/A |
|  | UL subband | UE shall follow legacy TDD behaviour, i.e., guard symbols should be reserved in D/F-slot to accommodate (1) 10us (or 3us) transient period; (2) TA_offset. | Located in guard symbol(s) reserved in non-SBFD DL symbol(s) |
| Case-D (UL symbol to SBFD symbol) | DL subband(s) | UE shall follow legacy TDD behaviour, i.e., the transient period can be accommodated in TA_offset | Located in TA_offset in non-SBFD UL symbol(s) |
|  | UL subband | TA_offset shall be applied to the timing in SBFD symbol and the preceeding UL symbol, and there is no BS transient period issue at all. | N/A |

Figure 2: Detailed illustration for transient periods for SBFD UE and legacy TDD UE for four cases

Note that RAN4 is still discussing the RAN4 requirement on the transient period, especially how to capture the above understanding on the location of transient period in RAN4 requirement for SBFD capable gNB.

## 2	Actions

To RAN1

ACTION: RAN4 respectfully asks RAN1 to confirm the above RAN4 understanding, and provide further information on the location of transient period if any.

## 3	Dates of next TSG RAN WG1 meetings

RAN4#114bis	7th – 11th April 2025	 	China

RAN1#115		19th – 23rd May 2025	 	Malta, MT