3GPP TSG-RAN WG4 Meeting #114	R4-2501975

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on RRM simulations for < 5MHz

Agenda item:	5.8.3

Document for:	Discussion

Introduction

Currently there is still some work to be done on Less Than 5MHz work item related to RLM and BFD test cases. In this contribution we provide simulation results for the RLM and BFD test cases and propose test case parameters. To be discussed in this meeting.

The simulation results throughout the work item are summarised in the following simulation contribution R4-2501976.

Discussion

Summary of the simulation results

The following table summarises the simulation results for 12, 15 and 20 PRBs.

| Channel | 12 Prb |  | 15 Prb |  | 20 Prb |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Interleaved, 2 os |  | Non interleaved, 3 os |  | Interleaved, 3 os |  |
|  | AL 2 | AL 4 | AL 4 | AL 8 | AL 4 | AL 8 |
|  | Qin | Qout | Qin | Qout | Qin | Qout |
|  | BLER 2% | BLER 10% | BLER 2% | BLER 10% | BLER 2% | BLER 10% |
|  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| TDL-C 300ns, 100Hz | 1.4 | -4.5 | -2.5 | -6.8 | -3.1 | -7.3 |

Table 1: 12, 15 and 20 PRB results

Test case value updates for 12 PRB

The following table shows 15 PRB results for Qin and Qout according to our simulations.

| 12 Prb |  |
| --- | --- |
| Interleaved, 2 os |  |
| AL 2 | AL 4 |
| Qin | Qout |
| BLER 2% | BLER 10% |
| [dB] | [dB] |
| 1.4 | -4.5 |

Table 2: 12, PRB results

Hence, we propose the following update to T1 and T2 parameters.

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR_SSB of set q0 | Config 1 | dB | 2 | -3 | -15 |
| SNR_SSB of set q1 | Config 1 | dB | 2 | -3 | -15 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -98 | -98 | -98 |

Update T1 value to 2dB for 12 PRB test case

Update T2 value to -3 dB for 12 PRB test case

Other than T1 and T2, use legacy values.

Test case update for 15 PRB

The following table shows 15 PRB results for Qin and Qout according to our simulations.

| 15 Prb |  |
| --- | --- |
| Non interleaved, 3 os |  |
| AL 4 | AL 8 |
| Qin | Qout |
| BLER 2% | BLER 10% |
| [dB] | [dB] |
| -2.5 | -6.8 |

Table 3 15 PRB simulation results:

Hence, we propose the following update to T2 and T4 parameters.

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 1 | -4.5 | -15 | -5.5 | 1 |
| SNR_SSB of set q1 | Config 1 | dB | 1 | -4.5 | -15 | -5.5 | 1 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | 1 | -4.5 | -15 | -5.5 | 1 |

In the test case, the signal levels are set according to the following figure:

‘

Figure 1: Test case figure from A.6.5.1.2.1-1: SNR variation for in-sync testing

Update T1 value to -4.5 dB for 15 PRB test case

Update T4 value to –5.5 dB for 15 PRB test case

Other than T1 and T4, use legacy values.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Update T1 value to 2dB for 12 PRB test case

Proposal 2: Update T2 value to -3 dB for 12 PRB test case

Proposal 3: Other than T1 and T2, use legacy values.

Proposal 4: Update T1 value to -4.5 dB for 15 PRB test case

Proposal 5: Update T4 value to –5.5 dB for 15 PRB test case

Proposal 6: Other than T1 and T4, use legacy values.