3GPP TSG-RAN WG4 Meeting #114	R4-2501979

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussion on LP-WUS/R RRM core requirements

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity. Sub-topic 1-3 MR RRM relaxation Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo) DRX cycle length [s]; Scaling factor. 0.32; TBD. 0.64; TBD. 1.28; TBD. 2.56; TBD. Issue 1-3-2: On Neighbour cell and serving cell relaxation factor Agreement: Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state.

Discussion

LR accuracy

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case 1 #1 Fully offloading case | Off | Off | ON |

In case 1 fully offloading scenario, the UE will not be required to perform any measurements using MR. Hence, it is assumed that the MR is fully turned off and UE only measures the serving cell with LR. This obviously means that the UE may not be measuring serving or neighbour cells the MR carrier.

For the fully offloading case we have two different scenarios:

- UE performs measurements using MR (SSB based).

- UE performs measurements using LR (LP-SS based or SSB-based)

This means we have scenarios where, to optimise the UE power saving opportunity, allow the UE to only perform serving cell LR measurements.

For the UE to know when it may fully offload all measurements to LR and hence not perform any measurements using the MR the network needs to configure a suitable threshold. This threshold must be based on SSB-based measurement performed by MR. The purpose of having thresholds is to define when UE can offload some or all measurements to LR. Such threshold can of course only be reliable and set correctly if the reference towards which the given threshold is compared is itself reliable. Reliable here means we have measurement which are accurate. Hence, to enable reliable offloading from MR it is necessary to have absolute measurement accuracy defined for MR.

To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR.

RAN4 have already defined absolute accuracy for idle mode measurement defined. We believe the absolute accuracy requirements defined when EMR was developed can be re-used.

The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes.

Once the UE is fully offloaded to LR and only performing measurements using the LR, it is equally important that the UE return to, and perform measurements with, MR in a timely manner. Performing MR measurements and initiating these early enough is important to ensure the UE connectivity and preventing UE from not being reachable for paging (or accessing the network).

Therefore, there is a need for defining measurement accuracy requirements for LR based measurements.

To enable reliable return to MR from LR in a timely manner, it is necessary to have absolute measurement accuracy requirements defined for LR.

As LR may use either use envelope detector receiver (LP-SS based) or OFDM type receiver (SSB-based) RAN4 need to consider accuracy requirements for both when measurements are performed using LR.

RAN4 should develop LP-SS based measurement accuracy requirements.

RAN4 should develop LR SSB based measurement accuracy requirements.

Using a relative threshold to determine offloading has also been discussed but seems to need more discussion. At least for the fully offloading case, where UE is only measuring using the LR (LP-SS or LR SSB) and is not performing SSB-based measurements using the MR, there is not reference RS signal. Hence, our understanding is that there is not always a measured signal which can be used as reference. This makes it insufficient to only rely on relative offset for offloading decision.

Additionally, under discussion is the matter of whether the LP-WUS/LP-SS is on same carrier or even in same band as the operating MR carrier. This will also complicate using relative offsets. This can be discussed further once a decision is made, but at least RAN4 need to define absolute accuracy requirements for SSB-based MR performed measurements and LP-SS/LR SSB based LR performed measurements.

Hence, for the network, it is important to understand the minimum requirements associated with the accuracy, as these will impact the threshold settings. If the accuracy cannot be guaranteed, the network is not able to configure measurement relaxations for case 1 because of unknown performance

Configurable RRM measurement Scaling factor

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length

It cannot be assumed that UE or the network supports legacy power saving features or Redcap. Hence, the legacy power saving features are independent from LP-WUS discussions.

We cannot assume that either network or the UE supports legacy power saving features on top of LP-WUS.

From the specification 38.133 scaling factor (N) and RRM measurement relaxation factor (k) are different. To clarify things our understanding of scaling factor here in this context means that it refers to RRM measurement relaxation factor.

With scaling factor, we mean RRM measurement relaxation factor (i.e., k in legacy)

In the last meeting scaling factor for MR RRM relaxation was discussed. Configured scaling factor allows the network to have control over the relaxation applied to serving and neighbour cell measurements. This allows network to adapt the scaling factor based on the specific deployment scenario, network conditions, and UE capabilities. For example, if fixed scaling factor is used, it is quite probable that a lower end value is specified, such as 4. If the network controls the scaling factor, the network can set it to higher values, such as 8 or 16, depending on deployment scenario. For instance, in a dense urban environment, a smaller scaling factor is necessary to ensure mobility, while in a rural area, a larger scaling factor can be used due to larger cell size.

RAN4 to specify configurable scaling factor with value range from 4 to 16 with a step size of [2].

Send LS to RAN2 about scaling factor configuration

About joint evaluation of thresholds

The following agreement was done in RAN2.

RAN2 agreement for Procedure and configuration of LP-WUS Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring LP-WUS Entry/exit condition If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements.

WUS monitoring and MR measurement relaxation have different triggering requirements. The threshold to enter LP-WUS monitoring is intended to determine and ensure that UE uses LP-WUS only when reception quality is sufficient to ensure paging performance (no degradation). The threshold for MR relaxation determines when the MR measurements can be relaxed (i.e. performed less frequently) or offloaded to LR.

The main goal of LP-WUS entry/exit monitoring is to allow reduction of UE's power consumption while maintain paging performance by allowing UE to stop monitoring the traditional paging occasion (PO) while monitoring LP-WUS instead. MR relaxation, on the other hand, is focused on reducing the power consumed during the serving and neighbouring cell measurements. These are two separate mechanisms designed to optimize different aspects of power consumption, and their thresholds should be independently tuned to achieve the best results.

Network may want to configure LP-WUS entry and exit based on RSRP, while the network configures MR relaxation thresholds based on RSRQ. For instance, when UE is sectorized cell and approaches  a sector boundary, the RSRP measurements may not reflect the proximity of another cell, hence ned to do re-selection, while RSRQ values could indicate this. When considering the case 1 with RSRP, the UE could end up being out of service after crossing a sector border.

LP-WUS entry / exit and MR measurement relaxation are independent procedures from system perspective. Joint threshold evaluation may occur depending on network configuration but RAN4 should not define only requirements for this case.

Not to specify requirements for joint evaluation of thresholds as the network may configure LR entry exit with RSRP and MR relaxation with RSRQ. Instead, the requirements should be independent.

Figure 1: Example with full offloading

For MR to receive paging, at least one SSB needs to be measured

The purpose of LP-WUS entry/exit thresholds is to allow UE to stop legacy PO monitoring until UE receives LP-WUS indicating wake-up /when above entry) and resume PO monitoring (when below exit).

The purpose of MR offloading / relaxation thresholds is to control when the MR serving / neighbouring cell measurements can be offloaded/relaxed

RAN4 should consider the monitoring of LP-WUS and measurement relaxation and offloading separately. From the network point of view, it is important to be able to set meaningful thresholds for LP-WUS monitoring and measurement relaxation and offloading thresholds for both LP-SS and PSS/SSS.

From the procedure point of view, LP-WUS monitoring may start already before relaxations / offloading occur, hence the requirements should consider LP-WUS monitoring entry/exit independently. Entering LP-WUS monitoring may occur at different time to MR relaxation / offloading entry.

UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, UE shall monitor LP-WUS occasions.

LP-WUS monitoring entry/exit requirements and wake up delay(s) are independently defined from MR relaxation requirements. i.e., depending on network configuration, the UE may start to monitor LP-WUS independently from MR measurement relaxation.

Switching between different RAN4 cases

Issue 1-1-2: Threshold for switching between different cases.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case 1 #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Switch from Case #1 to Case #3: This involves transitioning from the fully offloaded case (case 1) to the relaxed measurement case (case 3), using LR measurements. Switch from Case #3 to Case #1: This transition, from the relaxed to fully offloaded case, can be based on LR measurements alone, or both LR and MR measurements, with the UE stopping neighbour cell measurements when both are below their corresponding thresholds.

Some companies propose that the thresholds for switching cases are up to RAN2 to decide. It's suggested that new LP-based thresholds using LP-RSRP and LP-RSRQ are needed to switch into and out of Case 1. Some companies propose using existing thresholds with offsets for LP-RSRP and LP-RSRQ for the transitions between cases 1 and 3.

When considering that RAN2 has just started to discuss the MR relaxation, it is not clear if this transition is supported by configuration.

Currently RAN2 is defining relaxation conditions currently. Discuss first what has been agreed in RAN2

Measurement requirements to be specified for LP-WUR.

Issue 1-1-3: Measurement requirements to be specified for LP-WUR

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

As stated in the agreement, it is not the intention of RAN1 to introduce new RRM requirements for the overlaid scenario. Hence, RAN4 may consider the following agreement and table as a baseline for defining requirements. If the UE is able to receive PSS/SSS with LR, the requirements to follow should be PSS/SSS based LR requirements. If UE supports OOK based LR, then OOK based requirements should be followed, regardless of whether the scenario is LP-SS overlaid or not.

Define PSS/SSS based LP-WUR measurement requirements and LP-SS based LP-WUR measurement requirements. Consider the following table:

| Receiver type assumption | Reference Signals | RAN4 requirements | Measurement supported |
| --- | --- | --- | --- |
| OFDM receiver based LR to receive SSB | PSS/SSS only | PSS/SSS based LP-WUR measurement requirements apply | PSS/SSS based serving cell measurements |
|  | PSS/SSS and LP-SS |  |  |
|  | PSS/SSS and LP-SS with overlaid OFDM sequence |  |  |
| OOK receiver based LR | LP-SS only | LP-SS based LP-WUR measurement requirements apply | LP-SS based serving cell measurements |

RAN4 related MR Wake up delay

It is expected that MR and LR performances will be different, and that LR will be operated only when radio conditions are good from MR perspective. As discussed, LR is assumed to have worse NF and overall lower performance (due to lower complexity). In other words, when UE can receive LP-WUS, it can be expected to be in very good radio conditions from MR perspective, thereby enhancing the SSB based synchronisation acquisition.

The time (SSBs) required for the UE MR to re-establish time and frequency synchronisation can be reduced by accounting the LR assistance information and radio performance difference between LR and MR.

RAN1 has been discussing about UE capability related to MR wake-up, and considering whether the capability should only cover the MR ramp-up time or whether also MR synchronisation time should be included. As discussed above, MR wake-up should occur in excellent radio conditions from MR perspective and thereby MR synchronisation should require significantly less time than typically considered. Hence, it would seem preferable that RAN4, once determining the LR receiver performance and the delta to MR performance, considers the MR synchronisation time requirement after the wake-up accounting the performance delta.

RAN4 shall define the minimum required time for MR synchronisation and related side conditions.

UE shall be ready to exit after the

LP-WUS-WUR on a different carrier frequency than MR

Issue 1-1-8: LP-WUR operating carrier frequency

RAN plenary  #106 discussed different possible scenarios and solutions for separate band support of MR and LR. In RP-243266 [1] RAN agreed a way forward that WGs should consider possible solutions for LR supporting sub-set of MR bands. One of the considered scenarios is case where UE can be paged (via LP-WUS) on one frequency carrier, but expected to perform RA procedure on other carrier. It is felt that, as the related procedures (e.g. UE supporting LP-WUS being directed to camp on carriers where LP-WUS is deployed) fall under RAN2 jurisdiction, that RAN4 should wait for RAN2 discussion related to feasibility of different solutions.

RAN4 can consider the impact of different solutions for LR supporting sub-set of MR bands, but should wait until RAN2 decides more details on which of the scenarios is supported in Rel-19.

Evaluation of entry – exit

Issue 1-2-3: On requirements for LP-WUR for entry/exit criteria(threshold) evaluation Recommendations: Encourage companies firstly to consider whether to differentiate requirements based on cases Note: For entry condition evaluation requirements, requirements from legacy state to case 1 or case 3 or LP-WUS monitoring will be discussed separately. If No Same requirements will be defined on entry criteria(threshold) evaluation from one case to the other case (excluding scenarios in the note). Same requirements will be defined on exit criteria(threshold) evaluation from one case to the other case. If Yes, consider the following cases one by one Issue 1-2-3a: On requirements for entry/exit criteria(threshold) evaluation for Case 1 Issue 1-2-3b: On requirements for entry/exit criteria(threshold) evaluation for Case 3 Issue 1-2-3c: On requirements for entry/exit criteria(threshold) evaluation for LP-WUS monitoring On detailed evaluation requirements, the following options are available - Option 1: Evaluation requirements for entry/exit evaluation is the same as that of LP-WUR measurement period. - Option 2: Evaluation requirements for entry/exit conditions is longer than that of LP-WUR measurement period. - Option 3: Evaluation requirements for entry evaluation is longer than that of LP-WUR measurement period; evaluation requirements for exit evaluation is the same as that of the LP-WUR measurement period.

When the serving cell RSRP or RSRQ gets worse, the exit or starting MR measurements needs to be fast as there is a delay associated with MR startup. The exit should also be possible right after the entry if the UE measurements indicate that the exit is needed (i.e., RAN4 can assume UE can evaluate the LR measurement based exit in sliding window manner).

Differentiate between MR and LR measurement based entry and LR measurement based exit. Entry can be slower but exit shall be fast.

Measurement in idle mode and LP-WUS measurement relaxation

Issue 1-5-2: LR based RRM with EMR in IDLE/inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

LP-WUS is intended to be used for the benefit of UE power saving. One approach for UE power saving is to offload some or all current measurements performed by the MR to the LR. At least in the rel-19, only serving cell measurements are performed by LR. There are currently existing idle-mode measurements including reselection measurements, which need to be considered when UE is / has been in LP-WUS mode as the idle-mode measurement reporting is impacted if the UE has relaxed the measurements due to LP-WUS. When considering idle-mode inter-frequency measurements, CA/DC measurements should be also considered when UE is in LP-WUS mode and these have been relaxed.

The rel-18 work on fast CA/DC setup also expanded the idle-mode procedures to cover cell reselection measurements.  By using LP-WUS, the benefits of these features would be diminishing as there would be no measurements reports to provide to the network, since some or all measurements can be offloaded to LR and LR may not be able perform neighbour cell measurements. In some case even all MR measurements can be omitted when LP-WUS is used.

Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements

In the last meeting, some companies were proposing that when T331 is running, the UE will keep the EMR measurements running on configured carriers, this would mean that for the first 300 seconds in the idle-mode (T331 timer value), the UE should not enter LP-WUS mode when UE goes into idle-mode, in other words, the T331 condition overrides the MR relaxation condition. From the network point of view this is ok, as the performance in the idle-mode with shorter cycles can be guaranteed.

When T331 timer is configured, the UE will not go to idle-mode while the timer is running.

If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Other issues

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

RAN2 has just recently started to discuss relaxation, hence this can be postponed for now.

Postpone the discussion on Issue 1-1-1 until more RAN2 progress

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

Currently RAN1 has agreed that LP-SS has periodicity of 320ms. From the network point of view this means that the network does not need to transmit LP-SS more often than every 320ms. Before RAN1 agrees other periodicities, the LP-SS periodicity assumption should be 320ms.

RAN1 Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

No need for lower bound, below what RAN1 has agreed. LP-SS periodicity in RAN4 requirements follows RAN1 agreement, which is that LP-SS is transmitted every 320ms.

- Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Check the following: LP-WUR is ON for serving cell measurement, but when, how to turn on LR and the duration of “ON” for serving cell measurement is up to UE implementation, and no LP-WUR requirement will be applied.

From the network point of view, the LP-WUR status is invisible, otherwise than observing a longer paging response delay after presumably the LP-WUS has been in use. From that point of view, it is not clear what is the meaning of this issue. What matters is that when the UE is within LP-WUS monitoring entry / exit thresholds, the UE is assumed to be monitoring LP-WUS, and this should also be tested. When the UE is outside of LP-WUS monitoring thresholds, there should be no LP-WUS associated delay on paging response.

When UE is within LP-WUS, from the network point of view, there is known delay associated with paging response.

When UE is outside of LP-WUS monitoring, there is no LP-WUS associated delay on paging monitoring / response.

LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures

Conclusion

The following Observations and Proposals were made:

Proposal 1: To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR.

Proposal 2: The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes.

Proposal 3: To enable reliable return to MR from LR in a timely manner, it is necessary to have absolute measurement accuracy requirements defined for LR.

Proposal 4: RAN4 should develop LP-SS based measurement accuracy requirements.

Proposal 5: RAN4 should develop LR SSB based measurement accuracy requirements.

Observation 1: We cannot assume that either network or the UE supports legacy power saving features on top of LP-WUS.

Observation 2: With scaling factor, we mean RRM measurement relaxation factor (i.e., k in legacy)

Proposal 6: RAN4 to specify configurable scaling factor with value range from 4 to 16 with a step size of [2].

Proposal 7: Send LS to RAN2 about scaling factor configuration

Observation 3: LP-WUS entry / exit and MR measurement relaxation are independent procedures from system perspective. Joint threshold evaluation may occur depending on network configuration but RAN4 should not define only requirements for this case.

Proposal 8: Not to specify requirements for joint evaluation of thresholds as the network may configure LR entry exit with RSRP and MR relaxation with RSRQ. Instead, the requirements should be independent.

Observation 4: The purpose of LP-WUS entry/exit thresholds is to allow UE to stop legacy PO monitoring until UE receives LP-WUS indicating wake-up /when above entry) and resume PO monitoring (when below exit).

Observation 5: The purpose of MR offloading / relaxation thresholds is to control when the MR serving / neighbouring cell measurements can be offloaded/relaxed

Proposal 9: UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, UE shall monitor LP-WUS occasions.

Proposal 10: LP-WUS monitoring entry/exit requirements and wake up delay(s) are independently defined from MR relaxation requirements. i.e., depending on network configuration, the UE may start to monitor LP-WUS independently from MR measurement relaxation.

Proposal 11: Currently RAN2 is defining relaxation conditions currently. Discuss first what has been agreed in RAN2

Proposal 12: Define PSS/SSS based LP-WUR measurement requirements and LP-SS based LP-WUR measurement requirements. Consider the following table:

Observation 6: The time (SSBs) required for the UE MR to re-establish time and frequency synchronisation can be reduced by accounting the LR assistance information and radio performance difference between LR and MR.

Proposal 13: RAN4 shall define the minimum required time for MR synchronisation and related side conditions.

Proposal 14: RAN4 can consider the impact of different solutions for LR supporting sub-set of MR bands, but should wait until RAN2 decides more details on which of the scenarios is supported in Rel-19.

Proposal 15: Differentiate between MR and LR measurement based entry and LR measurement based exit. Entry can be slower but exit shall be fast.

Proposal 16: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements

Observation 7: When T331 timer is configured, the UE will not go to idle-mode while the timer is running.

Proposal 17: If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

Proposal 18: RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Proposal 19: Postpone the discussion on Issue 1-1-1 until more RAN2 progress

Proposal 20: No need for lower bound, below what RAN1 has agreed. LP-SS periodicity in RAN4 requirements follows RAN1 agreement, which is that LP-SS is transmitted every 320ms.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

Check the following:

Observation 8: When UE is within LP-WUS, from the network point of view, there is known delay associated with paging response.

Observation 9: When UE is outside of LP-WUS monitoring, there is no LP-WUS associated delay on paging monitoring / response.

Proposal 21: LP-WUR status outside of the LP-WUS monitoring does not have impact to legacy procedures

- References

- RP-243266 Way forward on Rel-19 LP-WUS separate band issues RAN2 chair (InterDigital) TSG RAN Meeting #106, Dec 9-12, 2024