3GPP TSG-RAN WG4 Meeting #114	R4-2501982

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On CSI-RS measurements for LTM

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

This contribution discusses the open issues related to CSI-RS measurements for LTM based on RAN4#113 R4-2420101 Way Forward

Discussion

L1 measurement pre-requisite for FR2-1

Issue 3-1-2: Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet <Candidate solutions> - Option 1: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2. (Apple, Huawei, vivo) - Option 2: Remove Rel-18 LTM SSB-based L1-RSRP measurements. (CMCC, Nokia, Samsung, MTK, Ericsson) - Option 2a: Remove L1 measurement for FR2-1 as side condition and consider “The CSI-RS from the target cell configured for L1 measurement remains measurable” as requirement applicability. (Xiaomi) - Option 3: Keep L1 measurement for FR2-1 with bracket in the 2nd bullet and wait for RAN1/RAN2’s progress. (ZTE) <Recommended WF> - Continue discussion.

In rel-18, for FR2-1 it was assumed that L3 measurements are performed with wide beam, and L1 measurements for FR2 are based on narrow beam settings. Similarly, in rel-19, UE is expected to perform SSB based L3 measurements and determine the wide beam. In rel-19, instead of performing Rx beam sweeping on L1 SSB to determine the fine beam, UE can use periodic [or semi-persistent] CSI-RS for the Rx beam sweeping.

On Whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet, our proposal is to remove it. In case some UEs need L1 SSB, RAN4 can determine a new UE capability where CSI-RS based L1-RSRP measurement and reporting needs to be done with SSB-based L1-RSRP measurement in FR2. If a UE supports this capability, SSB based L1 measurements are needed before CSI-RS. For a UE that does not supports this capability, step 2 can be skipped, and the CSI-RS resource should be periodic or semi-persistent.

For FR2-1, as a baseline, no need for rel-18 L1 SSB based measurements before CSI-RS measurements.

  - If some UEs need L1 SSB before CSI-RS, define a capability where UE indicates it needs to perform L1 SSB measurement before performing CSI-RS measurement.

Detectable vs measurable in the known condition

Issue 3-1-3: Whether to use detectable or measurable? <Candidate solutions> - Option 1: The difference of the definition between detectable and measurable need to be further clarified. (CATT) - Option 2: Use detectable. (Apple, ZTE) - Option 3: Use measurable. (Xiaomi, OPPO, Ericsson, Nokia) <Recommended WF> - Continue discussion. RAN4 can first try to align understanding on what “detectable” and “measurable” mean.

The detectable word is used for SSB because there are clear identification requirements for SSB, and it is possible to perform cell search and detect SSB without prior knowledge. The word “detectable” is applied for SSB, because a condition is specified, which is according to the cell identification in clause 9.2. If no condition is specified for CSI-RS, we would, for clarity, prefer to use the word “measurable” instead of detectable.

For detectability, we can define that the SSB associated with the CSI-RS remains detectable. The requirements on the SSB side are clear. Also a side condition can be considered.

The CSI-RS from the target cell configured for L1 measurement remains measurable, and

  - [SSB associated with the CSI-RS remains detectable]

  - [CSI-RS is above [-6] dB]

Side condition for CSI-RS based L1 measurement

Issue 3-1-4: Side condition for CSI-RS based L1 measurement for neighbor cell <Candidate solutions> - Option 1: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB (MTK) - Option 2: It is proposed to use legacy side condition of SINR=-3dB. (Apple, Ericsson, Nokia, Samsung) <Recommended WF> - Continue discussion.

We are ok with -6dB side condition instead of -3dB.

SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB

CSI-RS repetition ‘ON’ / ‘OFF’

#### Issue 3-5: Measurement on CSI-RS with repetition ON <Candidate solutions> - Option 1: Considering the potential large overhead, not support skipping SSB based L1 measurement before performing L1 measurement on CSI-RS resources with repetition ON in FR2-1. (MTK) - Option 2: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell. (Huawei, CATT) - Option 2a: Not to support CSI-RS resources with repetition ON for L1 measurement on an LTM candidate cell. The decision shall be made in RAN1. (vivo) <Recommended WF> - Continue discussion.

RAN1 WF: - Companies are encouraged to study and provide their views on the following issues aiming at the progress at RAN1#119 - The necessity of Rx beam refinement i.e. by allowing repetition is set to “on” - The necessity to provide different configuration on repetition for event triggered reporting and gNB scheduled reporting.

As can be seen from the RAN1 WF, there is still discussion whether to support the repetition “ON” configuration. To include this scenario, we think it’s quite important to also have configuration for it. If RAN1 agrees to have no configuration, then RAN4 doesn’t need to define requirements.

Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements.

A CSI-RS resource set configured with higher layer repetition is used for beam management, where the value of the repetition can be set to ‘ON’ or ‘OFF’. A CSI-RS resource set with repetition set to ‘ON’ is used for the UE to evaluate quality across different DL Rx beams, allowing the UE to determine a fine DL Rx beam and a corresponding UL Tx beam, conditional to UE supporting beam correspondence. The procedure is triggered by the network using dynamic CSI-RSs (semi-persistent or aperiodic), where only a few selected DL Tx beams are used for repeated CSI-RS transmissions with the same DL Tx beam. According to our understanding, semi-persistent and/or aperiodic CSI-RSs support would be needed. As these decisions are still pending in RAN1, RAN4 may start from defining requirements with the assumption that the repetition is set to ‘OFF’.

Although it is possible to configure a CSI-RS resource set with repetition set to 'ON' using periodic CSI-RSs, this may be feasible only for limited scenarios (e.g., with a small number of CSI-RS-based gNB beams), and it’s currently not clear even how many candidate cells UE is measuring before cell switch or how CSI-RS beam selection is made. For instance, if TCI state activation is used, or L3 SSB based selection is used using CSI-RS that is QCL’ed with SSB.  Based on the cell selection, we would also need to consider whether UE has performed L3 and L3 measurements, and what has been the Rx assumption.

If UE measures neighbouring cell CSI-RS with repetition “ON”, it needs to be discussed whether the measurements are applicable after the UE has entered the cell switch interruption. It hardly makes sense to define requirements for early CSI-acquisition procedure if the UE needs to repeat the procedure after the interruption.

As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

CSI-RS resource type

#### Issue 3-7: CSI-RS resource type <Candidate solutions> - Option 1: In addition to periodic CSI-RS, RAN4 also needs to consider semi-persistent CSI-RS transmission and measurements when defining RRM requirements for CSI-RS based L1 measurements for candidate cells. (CATT) - Option 2: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS. (Apple, vivo) - Option 3: Start from periodic CSI RS measurements, and wait until RAN1 agrees on aperiodic and semi-persistent CSI-RS support. (Nokia) - Option 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent. (Samsung) - Option 5: RAN4 should define CSI-RS based L1 measurement requirements for periodic, aperiodic and semi-persistent CSI-RS (ZTE) <Recommended WF> - Continue discussion.

RAN1 #118 Agreement: - Support L1-RSRP measurement based on CSI-RS - FFS: Support L1-SINR measurement based on CSI-RS RAN1 #118 Agreement: For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility RAN1 #118bis Agreement: For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported. Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability.

From the RAN1 working assumption it can be seen that the semi-persistent measurement will be supported via UE capability in Feb RAN1 meeting. From this point of view RAN4 can define requirements for this case when it is formally agreed.

Start from periodic CSI RS measurements and support semi-persistent CSI-RS once RAN1 confirms the working assumption

Supported measurement type

#### Issue 3-8: supported measurement type <Agreement> - CSI-RS based L1 measurement without gap is supported. - FFS on CSI-RS based L1 measurement with gap

It is not clear what are the benefits of supporting CSI-RS based inter-frequency measurements with gap. From the network point of view, it does not seem like a sensible configuration to have CSI-RS for candidate cells with gaps. We suggest postponing this issue to future releases.

CSI-RS based L1 measurement with gap is deprioritised out from rel-19

CSI-RS based L1 measurement w.r.t active BWP

#### Issue 3-10: CSI-RS based L1 measurement w.r.t active BWP <Candidate solutions> - Option 1: CSI-RS based L1-RSRP measurement outside active BWP is a typical scenario and should be considered in R19 LTM. (CATT) - Option 2: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK, Xiaomi, OPPO) - Option 3: Support CSI-RS based L1-RSRP measurement within active BWP and outside active BWP, and same principle can be used as start point, e.g., defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement. (ZTE) - Option 4: If RAN4 agree to define requirements for both within active BWP and outside active BWP, an optional UE capability for CSI-RSs are outside active BWP shall be introduced. Prefer to deprioritize the case where CSI-RS for LTM L1 measurements are outside active BWP(s). (vivo) - Option 5: From RRM requirement perspective, RAN4 further discuss which option is taken as the clarification for ‘the CSI-RS of candidate cell outside active BWP’ in R19 LTM: - Option 5a: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include at least one RB that is outside any active BWP. - Option 5b: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS are all outside active BWP(s) of the UE. - Option 5c: a CSI-RS resource is regarded as outside active BWP if the frequency domain occupied RBs of the CSI-RS include no more than 48 RBs within any active BWP. <Recommended WF> - Continue discussion.

Regarding the frequency location, RAN1 has made the following agreement.

RAN1 118bis Oct: Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement.

Consider also the scenario where CSI-RS based L1-RSRP measurement is outside of the active BWP.

RTD scenarios

Issue 3-12: Scenarios to be considered w.r.t. RTD RTD is the received time difference among CSI-RS and SSB from different cells. - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. - Alt 2: only define requirements for RTD<CP. - NW shall only enable this when RTD<CP is guaranteed.

In rel-18 separate requirements were defined for RTD>CP and RTD < CP cases. The requirements for RTD>CP apply on for UEs that support the capability of L1 measurements with RTD>CP. The problem with these scenarios and requirements is that the network (still) does not know the RTD conditions the UE is observing. Making guarantees on RTD by deployment, for example only supporting intra-DU or collocated scenarios limits severely where LTM can be deployed. Therefore, from the network point of view, it is challenging to configure and use LTM for UEs that support only RTD < CP as there are no requirements for UEs when the actual RTD conditions are above CP.  A natural, but not particularly desirable, option for the network is to only configure LTM for UEs that support RTD > CP capabilities, and to use legacy handover for UEs not supporting the capability.

- Network does not know UE RTD conditions, hence the RTD > CP capability is a prerequisite for LTM support in some networks

- One good option is to only define requirements for UEs supporting RTD > CP in rel-19.

As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability.

#### Issue 3-13: Definition of frequency layer <Candidate solutions> - Option 1: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and the CSI-RS occasions are fully/partially overlapping in time domain. (Ericsson) - Option 2: Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity) (Nokia) - Option 3: If RAN4 concludes not to define CSI-RS based L1 measurement requirements for both RTD>CP case and CSI-RS outside active BWP, it is not necessary to define “frequency layer” (MTK) - Option 4: From RRM requirement perspective, at least if RRM requirements for CSI-RSs outside active BWP are to be considered, RAN4 needs to clarify the frequency layer of the CSI-RS based L1 measurement. (vivo) - The CSI-RSs of the target cells are considered as the same frequency layer if the SSBs of these target cells are of the same SSB frequency, SCS and CP type. - No RRM requirements are defined, if CSI-RSs on the same frequency layer are configured with the different SCS or CP type. - Note: No restriction on the center frequency and/or BW of the CSI-RS. <Recommended WF> - Continue discussion.

Section 9 currently has multiple mentions of frequency layer. Introducing another frequency layer definition would not add much benefit. Therefore, we propose discussing the specific conditions instead.

Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity)

CSI Acquisition

To have a brief update on CSI acquisition progress, the following has been agreed in RAN1.

Agreement The following alternatives are further studied: - Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE. - The report is sent to the serving cell and transferred to the candidate/target cell(s) - Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting

In the last RAN1 meeting, the following working assumption was made regarding early CSI acquisition for candidate cells.

RAN1 Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

The impact of the alternative 1 is mostly on the network side as the network needs to transmit the CSI-Acquisition information to the target cell. From RAN4 point of view alternative 2 and alternative 3 are the ones which have the most RAN4 requirement impact. From our point of view, whether RAN1 supports Alt-2 or Alt-3, the requirements should be clear. Particularly in the Alt-3 RAN4 would need to consider that the UE is performing CSI-acquisition over the time which traditionally has been interruption time. Whether and how this is possible should be discussed, at least briefly, in RAN4.

CSI-acquisition may start before the cell switch (e.g. early CSI-acquisition case) or it may start from the cell switch command (e.g. After SCS CSI-acquisition). RAN4 can start to discuss if and how CSI-acquisition is performed during and after the cell switch.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: For FR2-1, as a baseline, no need for rel-18 L1 SSB based measurements before CSI-RS measurements.

	If some UEs need L1 SSB before CSI-RS, define a capability where UE indicates it needs to perform L1 SSB measurement before performing CSI-RS measurement.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains measurable, and

	[SSB associated with the CSI-RS remains detectable]

	[CSI-RS is above [-6] dB]

Proposal 3: SNR of the CSI-RS from the target cell configured for L1 measurement is above -6dB

Proposal 4: Wait until RAN1 has agreed to support repetition “ON” configuration before agreeing to define requirements.

Proposal 5: As a baseline, define requirements for the case where the repetition is ‘OFF’. Revisit the agreement if RAN1 decides to support “ON” configuration.

Proposal 6: Start from periodic CSI RS measurements and support semi-persistent CSI-RS once RAN1 confirms the working assumption

Proposal 7: CSI-RS based L1 measurement with gap is deprioritised out from rel-19

Proposal 8: Consider also the scenario where CSI-RS based L1-RSRP measurement is outside of the active BWP.

Observation 1: Network does not know UE RTD conditions, hence the RTD > CP capability is a prerequisite for LTM support in some networks

Proposal 9: As the network does not know UE RTD conditions, requirements for RTD > CP shall be defined for rel-19 LTM. Whether the support of these requirements is up to UE capability.

Proposal 10: Not to define new terminology of “frequency layer” for CSI-RS based L1-RSRP measurement. Discuss directly the intra/inter-frequency CSI-RS scenarios where CSI-RS resources have similar properties (SCS, CP type, periodicity)

Proposal 11: CSI-acquisition may start before the cell switch (e.g. early CSI-acquisition case) or it may start from the cell switch command (e.g. After SCS CSI-acquisition). RAN4 can start to discuss if and how CSI-acquisition is performed during and after the cell switch.