# 3GPP TSG-RAN WG4 Meeting # 114                                                          R4-2501984

# Athens, Greece, 17 – 21 February 2025

# Title:	NS_05 NS_48 NS_49 PC2 A-MPR clean-up

# Agenda Item:	6.5.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document discusses how to reflect the recently agreed Rel-19 rules on PC2 A-MPR in [1] into the Rel-17 PC2 A-MPR requirements for NS_05, NS_05U, NS_48 and NS_49.

# Discussion

## Background

At last meeting, it was agreed in Way Forward [1] that, quoting, “The PC2 A-MPR may not exceed the PC3 A-MPR by more than 3dB”.  This rule was motivated by two considerations:

- On one hand, there is no benefit of deploying PC2 if the PC2 A-MPR is more than 3dB higher than the agreed PC3 A-MPR

- On the other hand, reverse-IMD when PC2 is achieved via 2Tx may require 2Tx to be specified as high as PC3 A-MPR +3dB.

These considerations are accounted for in the recently completed PC2 FDD A-MPR requirements throughout the late stage of Rel-18 or early studies for Rel-19.

However, Band n1 was amongst the first PC2 FDD band with A-MPR requirements completed in Rel-17. At that time RAN4 did not have such agreements. For the sake of reducing the TS complexity, it was agreed to specify only 1 set of PC2 A-MPR requirements based on 1Tx and to allow an additional A-MPR relaxation of [2]dB for PC2 achieved with 2Tx using Note 7 in Table 6.2.3.1-1.

This footnote may lead to PC2 2Tx A-MPR requirements which may exceed the PC3 requirements by more than 3dB.

Observation 1: A [2]dB A-MPR relaxation was agreed in Rel-17 to account for the reverse IMD impact for PC2 achieved with 2Tx. This allowance may lead to PC2 2Tx A-MPR requirements which may exceed the PC3 A-MPR by more than 3dB. Hence it is necessary to review the PC2 A-MPR requirements for NS_05, NS_05U, NS_48 and NS_49 A-MPR to ensure the Rel-19 requirements are aligned with the recently agreed WF [1] guidelines.

To account for PC2 2Tx rev-IMD, and also comply with the WF [1] agreement, we propose to remove the [2]dB A-MPR relaxation for NS_05, NS_05U, NS_48 and NS_49 and increase the PC2 1Tx A-MPR by 0.5dB for cases where the PC2 1TxA-MPR < PC3 A-MPR +3dB.

We realize that comparing the PC2 vs PC3 A-MPR is not as straightforward as performing a simple subtraction due to the adoption of different A-MPR regions between PC3 and PC2. We are therefore open to further discuss case-by-case for which regions may this 0.5dB relaxation apply, focusing on 1RB edge, and full RB allocations as first priority. We also realize that such a proposal tightens the Rel-17 and Rel-18 PC2 2Tx A-MPR requirements and may therefore only be implemented in Rel-19.

In the rest of this document, we review each A-MPR requirement, identify A-MPR regions eligible to PC2 2Tx 0.5dB relaxation, and flag cases where the agreed 1Tx PC2 A-MPR > PC3 A-MPR +3dB. We invite interested companies to check if these proposals are acceptable.

Proposal 1: For NS_05, NS_05U, NS_48 and NS_49, and to account for PC2 2Tx rev-IMD:

- Void Note 7 in Table 6.2.3.1-1,

- Replace the [2]dB PC2 2Tx A-MPR relaxation with a maximum of 0.5dB relaxation for cases where the specified PC2 (1Tx) A-MPR < PC3 A-MPR +3dB,

- To avoid tightening the Rel-17 and Rel-18 requirements, these changes are applicable to Rel-19 only,

- Interested companies are invited to verify the proposed corrections.

## Impact on NS_05, NS_05U

Table 1 summarizes the PC2 to PC3 A-MPR difference for A1, A2, A3 where we highlight in yellow differences less than 3dB, and in orange differences greater than 3dB. The A3 inner, the reported data is the difference between the PC3 MPR and the agreed PC2 A-MPR since this region is not specified for PC3.

Table 1: PC3 vs PC2 A-MPR requirements difference (dB) for NS_05 and NS_05U A1, A2, A3.

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner* |
| DFT-s-OFDM | Pi/2 BPSK | 3 | 1 | 2.5 | 2 |
|  | QPSK | 3 | 1 | 2.5 | 2 |
|  | 16 QAM | 3 | 1 | 2.5 | 1 |
|  | 64 QAM | 3 | 1 | 3 | -0.5 |
|  | 256 QAM | 2 | 1 | 2.5 | (MPR) |
| CP-OFDM | QPSK | 3 | 1 | 2.5 | 2.5 |
|  | 16 QAM | 3 | 1 | 2.5 | 2 |
|  | 64 QAM | 3 | 1 | 2 | 0.5 |
|  | 256 QAM | 3 |  | 0 | (MPR) |

Observation 2: For NS_05, NS_05U:

- A1: The DFT-s-OFDM 256QAM A-MPR is not consistent with all other modulations/waveforms,

- A2 and A3 outer: a 0.5dB PC2 2Tx relaxation may be considered except for DFT-s-OFDM 64QAM.

- A3 inner: this inner region is specific to PC2 and to 5MHz CBW. A PC2 2Tx relaxation may not be required for the inner allocations.

Proposal 2: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.4-12. Consider removing the brackets for A1.

Table 6.2.3.4-12: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 13 | ≤ 6.5 | ≤ 7 | ≤ 2 |
|  | QPSK | ≤ 13 | ≤ 6.5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 9 | ≤ 2 |
|  | 64 QAM | ≤ 14 | ≤ 6 | ≤ 9.5 | ≤ 2 |
|  | 256 QAM | ≤ 16 | ≤ 6.5 | ≤ 10 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 64 QAM | ≤ 14 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 256 QAM | ≤ 16 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

For regions A4, A5, A6, A7 we compare the difference for NS_05U between the PC2 and PC3 A-MPR in Table 2 where we highlight in yellow differences less than 3dB and in orange the cases where the PC2 A-MPR is smaller than the PC3 A-MPR. We ignore region A5 inner and the region A8 which is specific to PC2. Based on these differences, we propose the corrections captured in proposal 3.

Table 2: PC3 vs PC2 A-MPR requirements difference (dB) for NS_05U A4, A5, A6, A7.

| Modulation/Waveform |  | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer | Outer | Outer | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | 1 | 0 | 0 | 2 |
|  | QPSK | 1 | 0 | 0 | 2 |
|  | 16 QAM | 1 | 0 | -0.5 | 2 |
|  | 64 QAM | 0.5 | -0.5 |  |  |
|  | 256 QAM |  |  |  |  |
| CP-OFDM | QPSK | 0.5 | 0.5 | 0 | 2.5 |
|  | 16 QAM | 0.5 | 0.5 | 0 | 2.5 |
|  | 64 QAM | 1 | 1 | 0 | 2.5 |
|  | 256 QAM |  |  |  |  |

Proposal 3: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.4-13.

Table 6.2.3.4-13: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 2.5 |  | ≤ 2.5 | N/A | ≤ 8.5 | ≤ 3.5 |
|  | QPSK | ≤ 3.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 8.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 |  | ≤ 3.0 |  | ≤ 2.5 |  | ≤ 8.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |  | ≤ 3.5 |  |  |  | ≤ 8.5 | ≤ 3.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 5.0 |  | ≤ 5.0 |  | ≤ 4.5 |  | ≤ 9.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 5.0 |  | ≤ 5.0 |  | ≤ 4.5 |  | ≤ 9.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 5.5 | ≤ 5.5 | ≤ 4.5 |  | ≤ 9.0 |  |
|  | 256 QAM |  |  |  |  |  |  | ≤ 9.0 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

## Impact on NS_48

Table 3 summarizes the PC2 to PC3 A-MPR difference for NS-48. Changes for consideration are captured in proposal 4.

Table 3: PC3 vs PC2 A-MPR requirements difference (dB) for NS_48.

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 2 | 1 | 0.5 | 1 |
|  | QPSK | 2 | 2 | 1 | 0.5 | 1 |
|  | 16 QAM | 2 | 2 | 1 | 0.5 | 1 |
|  | 64 QAM | 2 | 2 | 1 | 0.5 | 1 |
|  | 256 QAM | 2 | 2 | 1 | 0.5 | 1 |
| CP-OFDM | QPSK | 1.5 | 2 | 1 | 0.5 | 1 |
|  | 16 QAM | 1.5 | 2 | 1 | 0.5 | 1 |
|  | 64 QAM | 1.5 | 2 | 1 | 0.5 | 1 |
|  | 256 QAM | 1.5 | 2 | 1 | 0.5 | 1 |

Proposal 4: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.26-4. Consider removing the brackets for A1.

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |

## Impact on NS_49

Table 4 summarizes the PC2 to PC3 A-MPR difference for NS-49. Changes for consideration are captured in proposal 5.

Table 4: PC3 vs PC2 A-MPR requirements difference (dB) for NS_49.

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 1 | 1 | 1.5 | 1 |
|  | QPSK | 2.5 | 1 | 1 | 1.5 | 1 |
|  | 16 QAM | 2.5 | 1 | 1 | 1.5 | 1 |
|  | 64 QAM | 2.5 | 1 | 1 | 1.5 | 1 |
|  | 256 QAM | 2.5 | 1 | 1 | 1.5 | 1 |
| CP-OFDM | QPSK | 2.5 | 1 | 1 | 2 | 1 |
|  | 16 QAM | 2.5 | 1 | 1 | 2 | 1 |
|  | 64 QAM | 2.5 | 1 | 1 | 2 | 1 |
|  | 256 QAM | 2.5 | 1 | 1 | 2 | 1 |

Proposal 5: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.27-4. Consider removing the brackets for A1.

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |

# Conclusion

This contribution summarizes our views on removing Note 7 from Table 6.2.3.1-1. This is needed since the [2]dB A-MPR relaxation of Note 7 for NS_05, NS_05U, NS_48 and NS_49 may lead to PC2 2Tx A-MPR to exceed the PC3 A-MPR by more than 3dB and therefore violate the recently agreed rules from [1]. We make the following proposals.

Observation 1: A [2]dB A-MPR relaxation was agreed in Rel-17 to account for the reverse IMD impact for PC2 achieved with 2Tx. This allowance may lead to PC2 2Tx A-MPR requirements which may exceed the PC3 A-MPR by more than 3dB. Hence it is necessary to review the PC2 A-MPR requirements for NS_05, NS_05U, NS_48 and NS_49 A-MPR to ensure the Rel-19 requirements are aligned with the recently agreed WF [1] guidelines.

Proposal 1: For NS_05, NS_05U, NS_48 and NS_49, and to account for PC2 2Tx rev-IMD:

- Void Note 7 in Table 6.2.3.1-1,

- Replace the [2]dB PC2 2Tx A-MPR relaxation with a maximum of 0.5dB relaxation for cases where the specified PC2 (1Tx) A-MPR < PC3 A-MPR +3dB,

- To avoid tightening the Rel-17 and Rel-18 requirements, these changes are applicable to Rel-19 only,

- Interested companies are invited to verify the proposed corrections.

Proposal 2: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.4-12. Consider removing the brackets for A1.

Table 6.2.3.4-12: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 13 | ≤ 6.5 | ≤ 7 | ≤ 2 |
|  | QPSK | ≤ 13 | ≤ 6.5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 9 | ≤ 2 |
|  | 64 QAM | ≤ 14 | ≤ 6 | ≤ 9.5 | ≤ 2 |
|  | 256 QAM | ≤ 16 | ≤ 6.5 | ≤ 10 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 64 QAM | ≤ 14 | ≤ 6.5 | ≤ 10.5 | ≤ 4 |
|  | 256 QAM | ≤ 16 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Proposal 3: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.4-13.

Table 6.2.3.4-13: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 2.5 |  | ≤ 2.5 | N/A | ≤ 8.5 | ≤ 3.5 |
|  | QPSK | ≤ 3.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 8.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.0 |  | ≤ 3.0 |  | ≤ 2.5 |  | ≤ 8.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |  | ≤ 3.5 |  |  |  | ≤ 8.5 | ≤ 3.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 5.0 |  | ≤ 5.0 |  | ≤ 4.5 |  | ≤ 9.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 5.0 |  | ≤ 5.0 |  | ≤ 4.5 |  | ≤ 9.0 | ≤ 3.5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 5.5 | ≤ 5.5 | ≤ 4.5 |  | ≤ 9.0 |  |
|  | 256 QAM |  |  |  |  |  |  | ≤ 9.0 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

Proposal 4: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.26-4. Consider removing the brackets for A1.

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 |

Proposal 5: To the account for the voiding Note 7 in Table 6.2.3.1-1, consider the following changes to Table 6.2.3.27-4. Consider removing the brackets for A1.

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤8.0 | ≤6.5 |

# References

- R4-2420342 WF on HPUE for NR bands, 3GPP TSG-RAN WG4 Meeting #113, Orlando, US, Skyworks Solutions, Inc.