3GPP TSG-RAN WG4 Meeting #114	R4-2502005

# Athens, Greece, 17 – 21 February 2025

Agenda item:		6.18.2

Source: 		Qualcomm Inc.

Title:	 	UE-to-UE coexistence and OOB blocking for n252

Document for:		Approval

## 1	Introduction

UE RF requirements for n252 (UL 2000-2020, DL 2180-2200) has been discussed for a few meetings, with main focus on UE-to-UE coexistence requirements towards bands n2 and n25, but final conclusion has not been reached yet. Another band-specific important aspect without conclusion is OOB blocking requirements and whether co-banding is considered similar to n256 requirements. In this contribution these are discussed.

## 2	Discussion

In RAN4#112-bis an agreement on UE-to-UE coexistence was reached

- Agreement:

  - Define a new more relaxed spurious emission requirement for TN-NTN UE to UE coexistence compared to -50dBm/MHz, that allows the NTN service to work effectively, across the full n252 band, whilst still protecting the TN UE at least in the majority of cases

    - Further discuss the actual value and methodology for the requirement (e.g. corresponding separation assumption)

    - Companies are requested to propose what emission level in dBm/MHz could be achieved for frequencies below 1995 MHz, considering the UE RF implementation, different uplink channel bandwidths and RB allocations

    - Further discuss how to capture the solutions in the specification

This was further discussed in RAN4#113, with the following captured the WF

- Tentative Agreement:

    - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2

      - FFS: -30 dBm/MHz for relaxed value.

        - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.”

      - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification

    - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable

    - Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

The work item is scheduled to conclude in March 25, which seems not possible given there is no conclusion on the UE-to-UE coexistence

Observation 1: It has already been agreed that UE-to-UE coexistence requirement is relaxed from -50 dBm/MHz

Observation 2: Work item will need to be extended due to slow progress for UE-to-UE coexistence.

As discussed in earlier meetings, UE-to-UE interference is a statistical phenomenon: UEs need to be near each other and also both UEs need to be scheduled at the same time. For NR NTN it is expected that UL RB allocations are typically narrow to be able to close the UL link. This means that also UL interference, specifically IMD components between wanted RBs and LO and image, are relatively narrow in frequency. Therefore, from a single UE transmission, it is expected that the emissions to the protected frequency range would be near the emission limit only within a narrow frequency range, and most frequencies suffer from much lower interference. This further reduces the impact to UEs within n2/n25 frequency ranges.

To be able to conclude the work within Q2/25, a decision on the protection limit is needed. Given the previous agreements, it is suggested to focus the discussion on range between -30 dBm/MHz and -40 dBm/MHz. When the final value is agreed, A-MPR evaluation can take place for the following meeting.

Observation 3: UE-to-UE co-existence is a statistical phenomenon. A single NR NTN UE is likely to use narrow UL RB allocation, meaning that emissions for most frequencies in the protected region will be well below the emission limit.

Proposal 1: Conclude n2/n25 protection limit by choosing from the candidate values of -30 dBm/MHz and -40 dBm/MHz, and perform A-MPR evaluations for RAN4#114bis

For out-of-band blocking different options were discussed for out-of-band blocking frequency ranges. n252 frequency range is overlapping with n256, and it is extremely likely that in practical UE designs the same filter is used to support both n252 and n256. However, when n256 was specified it was considered to share the same filter with n1/n65. Realistically, a practical design will have only a single duplexer to support these bands. Therefore, n252 out-of-band blocking should be specified to accommodate co-banding with n256 and n65.

Observation 4: n252 can be co-banded with n65 and n256.

Proposal 2: Specify OOB boundaries for n252 as shown in Table 1

Table 1: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 3	Conclusion

In this contribution UE RF requirements for n252 were discussed. Following observations and proposals were made:

Observation 1: It has already been agreed that UE-to-UE coexistence requirement is relaxed from -50 dBm/MHz

Observation 2: Work item will need to be extended due to slow progress for UE-to-UE coexistence.

Observation 3: UE-to-UE co-existence is a statistical phenomenon. A single NR NTN UE is likely to use narrow UL RB allocation, meaning that emissions for most frequencies in the protected region will be well below the emission limit.

Proposal 1: Conclude n2/n25 protection limit by choosing from the candidate values of -30 dBm/MHz and -40 dBm/MHz, and perform A-MPR evaluations for RAN4#114bis

Observation 4: n252 can be co-banded with n65 and n256.

Proposal 2: Specify OOB boundaries for n252 as shown in Table 1

Table 1: Out of-band blocking for n252 and n256

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

## 4	References