3GPP TSG-RAN WG4 Meeting #114	R4-2502015

# Athens, Greece, 17 – 21 February 2025

Agenda item:		5.2

Source: 		Qualcomm Inc.

Title: 		On completing band 254 requirements

Document for:		Discussion

## 1	Introduction

Band 254, also known as LS-band, was introduced in rel-18 and RAN4 RF work was closed in December 2023. The specification still has some TBD, and CRs were brought in previous meeting to address the TBDs. This contribution discussed the missing requirements and how to complete the requirements.

## 2	Discussion

When the RF core work for IoT NTN LS-band was recommended to be closed, a WF was agreed at the same meeting [1]

It can be seen that leaving A-MPR as TBD has been intentional. This explains why there has not been many A-MPR evaluations available.

At the same time, getting feedback from ETSI has taken longer than expected, and 3GPP should start addressing the missing ETSI requirements, both in LS-band as well as in other NTN bands.

The ETSI requirements are in some cases more stringent than 3GPP general requirements, and UE may need accommodations like A-MPR or guard band to meet them. Potential incompliant UE populations gets larger and larger the longer 3GPP waits. Therefore, it would be better to add relevant requirements to 3GPP now, and if necessary remove them later if ETSI in the end does not capture those requirements for the harmonized standards addressing 3GPP NTN operation.

Observation 1: Leaving TBDs for the NS_04N and NS_05N has been intentional.

Proposal 1: 3GPP to start incorporating ETSI requirements for all NTN bands. In case ETSI does not in the end include these requirements to harmonized standard, they can be removed.

Initial proposal for NS_04N and NS_05N A-MPR was made in [2] and [3]. Firstly, it can be noted that the proposed A-MPR does not seem to take into account results provided in [4]. Figures 2 and 3 are taken from [4] and show clear A-MPR need for single tone as well as full 12-tone allocation.

Figure 2: Single tone results in [4]

Figure 3: 12-tone results in [4]

Observation 2: A-MPR proposals in [2] and [3] do not consider results provided in [4]

So far we did not find any results for 3.75 kHz single-tone transmission. It might be even worse than 15 kHz single-tone, as all power is concentrated closer to the channel edge.

Observation 3: Evaluations for 3.75 kHz single-tone transmission have not been done.

We also did initial evaluations on this case with 15 kHz SCS, and found that our results are closer to [4], indicating significant backoff need. For example for NS_04 in-band mask at -2 dBm/30 kHz immediately outside the channel edge, approximately 7 dB backoff was observed. Also for a single tone at the edge, situation was the same, showing approximately 7 dB backoff. For full 12-tone allocation however needed A-MPR was approximately 2 dB.

Given that there are large differences between the results it would be useful to more investigate what is the cause for the difference. We identified at least two potential causes. Firstly, there has not been any commonly agreed set of used assumptions for studying the A-MPR. Secondly, for the requirements right next to the channel bandwidth edge, the baseband waveform assumptions are also meaningful.

In NR side, typically PA calibration point and IQ-impairment levels are commonly agreed to study A-MPR. Similar discussion would be beneficial here. While not agreed for NR, here also the spectral containment of the baseband waveform assumptions would be good to discuss. In principle meeting to NB-IoT general requirements do not need practically any processing to the waveform. Therefore, one could consider this should also be the starting point in evaluating the additional emissions.

Observation 4: Most likely differences in evaluation assumptions have resulted in large difference in A-MPR need. Common assumptions for NB-IoT A-MPR evaluations have not been agreed.

On the other hand, this can result in higher A-MPRs in some cases. To reach better alignment for the A-MPR results and avoid discrepancies in the future, it would be useful to align better on the study assumptions.

Proposal 2: Interested companies should align the evaluation assumptions for PA calibration, IQ impairments and spectral containment of the baseband waveform to be able to better align the result and be able to complete the band 254 requirements.

## 3	Conclusion

In this contribution co-existence between new NTN S-band and adjacent 3GPP bands was discussed. Following observations were made.

Observation 1: Leaving TBDs for the NS_04N and NS_05N has been intentional.

Proposal 1: 3GPP to start incorporating ETSI requirements for all NTN bands. In case ETSI does not in the end include these requirements to harmonized standard, they can be removed.

Observation 2: A-MPR proposals in [2] and [3] do not consider results provided in [4]

Observation 3: Evaluations for 3.75 kHz single-tone transmission have not been done.

Observation 4: Most likely differences in evaluation assumptions have resulted in large difference in A-MPR need. Common assumptions for NB-IoT A-MPR evaluations have not been agreed.

Proposal 2: Interested companies should align the evaluation assumptions for PA calibration, IQ impairments and spectral containment of the baseband waveform to be able to better align the result and be able to complete the band 254 requirements.

## 4	References

- R4-2321817, WF on new FDD band (L+S band) for IoT NTN, MediaTek Inc.

- R4-2411543, (IoT_NTN_FDD_LS_band) CR to 36.102 for IoT-NTN UE RF Multi-tones A-MPR requirements (Rel-18), MediaTek Inc.

- R4-2411544, (IoT_NTN_FDD_LS_band) CR to 36.102 for IoT-NTN UE RF 1-tone A-MPR requirements (Rel-18)

- R4-2321798, Analysis on ETSI emission requirement for NB-IoT in IoT NTN band b254, Sony