3GPP TSG RAN WG4 Meeting #114	R4-2502055

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	5.11.1

Source: 	Ericsson

Title:	Discussion on SR based test case setup for L3 report based SCell activation

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the performance part of the FR2 SCell activation enhancements in R18.

# Discussion

In last meeting following WF is agreed.

- For single SCell Activation of unknown SCell with valid L3 measurement results in FR1, RAN4 to introduce another sub-test with SR transmission in the existing test case for L3 report based fast SCell activation test case.

- Corresponding CR to be submitted in the next meeting.

When the L3 measurement results are available, RAN4 agreed to consider two scenarios of NW scheduling in the test case. In one scenario, UL grant is received at 3ms+THARQ+M-k2 after SCell activation command reception and in other case UL grant is received at [7]ms+ THARQ after SCell activation command is received.

For the SR based sub test, we think second option is relevant. That means UE should be ready to send SR request at the first occasion after 7ms+ THARQ.

- For the L3 report based fast SCell activation test case, for the subtest using SR to get UL grant, UE shall transmit scheduling request on the first SR resource by 7ms+ THARQ + TSR_Periodicity to obtain the UL grant for L3 report transmission.

Scheduling request is periodic resource, and we suggest using following scheduling request config in the test.

| Parameter |  | Value |
| --- | --- | --- |
| schedulingRequestId |  | 0 |
| sr-ProhibitTimer |  | 16ms |
| sr-TransMax |  | n4 |
| periodicityAndOffset |  | Sl2 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | Format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

- RAN4 to use following scheduling request config in the test.

| Parameter |  | Value |
| --- | --- | --- |
| schedulingRequestId |  | 0 |
| sr-ProhibitTimer |  | 16ms |
| sr-TransMax |  | n4 |
| periodicityAndOffset |  | Sl2 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | Format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

# Summary and Conclusion

In this contribution we have provided our views on the FR2 SCell activation delay reduction performance part and made the following proposals:

- For the L3 report based fast SCell activation test case, for the subtest using SR to get UL grant, UE shall transmit scheduling request on the first SR resource by 7ms+ THARQ + TSR_Periodicity to obtain the UL grant for L3 report transmission.

- RAN4 to use following scheduling request config in the test.

| Parameter |  | Value |
| --- | --- | --- |
| schedulingRequestId |  | 0 |
| sr-ProhibitTimer |  | 16ms |
| sr-TransMax |  | n4 |
| periodicityAndOffset |  | Sl1 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | Format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

# References

- RP-220977 “WID on Even Further RRM enhancement for NR and MR-DC”, Apple Oppo

- R4-2406339