3GPP TSG RAN WG4 Meeting #114	R4-2502061

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.8.3

Source: 	Ericsson

Title:	Discussion on low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. WID is described below.

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

-	Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

-	Specify necessary RRM requirements [RAN4]

-	Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

-	Consider the following deployment constraints:

-	The carrier frequency for all cases is <1 GHz

-	Co-located and synchronized network deployment for both carriers

-	Both carriers are in a single TAG

-	SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the potential impact to RAN4 RRM requirements.

# Background

Operators have a rich mid-band spectrum but limited low-band spectrum, which is crucial for coverage in both urban and rural areas [2]. In some of the band combinations of the low band, e.g., n5 plus n29 and n12 plus n29, low band carrier aggregation is not supported at present. The main issue for the lack of support of the low band CA in these BC are listed below.

- Antenna Bandwidth: Supporting a large antenna bandwidth (BW) with a single antenna for certain band combinations is difficult. This is because the antenna needs to cover a wide range of frequencies, which can be technically challenging.

- Duplex Gap and Multiplexer Issues: For some band combinations like n12 and n29, n29 exists in the duplex gap of n12. This means there's no gap for Tx-Rx (transmit-receive) switching, which is essential for the multiplexer to function properly.

Figure 1: Issues for supporting low band CA

3GPP is tasked to specify a switching mechanism to support the low band CA via switching. We summarize low band CA via switching as following.

- when the SCell operation is triggered, UE needs to switch to the SDL carrier (SCell), and during the operation period there is no simultaneous Tx/Rx between the PCell (FDD carrier 1) and the SDL carrier 2 (SCell)

- UE switches back to the FDD carrier 1 (PCell) after the SCell operation is finished

For the rest of the discussion, we refer the FDD carrier 1 as PCell and SDL carrier 2 as SCell.

The main components to support switching can be summarized as the

- Switching pattern

- Switching delay and time mask for the carrier switching

- RRM procedures that might be impacted due to switching pattern and switching delay

# Discussion

## Scope of the work item

In the RAN plenary, two cases were discussed: SCell with SSB and SCell without SSB. We would like to point out that UE supports SSB-less SCell with optional UE capability. Since this WI is proposed by the operators and concerns existing on-field issues, RAN4 should strive to support as much UE capability as possible. Therefore, as a baseline, SCell with SSB should be supported for a UE supporting low band carrier via switching. This means that if we consider SCell without SSB, there should not be two optional UE capabilities (i.e., low band CA via switching with SSB and low band CA via switching without SSB) for supporting low band CA via switching. We are open to considering SCell without SSB after the baseline solution is supported.

- For a UE supporting low band CA via switching, SCell with SSB operation should be mandatorily supported.

- For low band CA via switching, SCell without SSB operation can be considered after the SCell with SSB operation is supported.

## RRM requirements for CA operation

| Requirement | Impacted or not impacted |
| --- | --- |
| RRC idle state mobility | If the SDL carrier can be configured for Idle mode CA/DC measurements (EMR measurements), potentially these measurements may be impacted. However, since the WI concerns connected mode operation, we do not see impact on these requirements. |
| RRC inactive state mobility | If the SDL carrier can be configured for Inactive mode CA/DC measurements (EMR measurements), potentially these measurements may be impacted. However, since the WI concerns connected mode operation, we do not see impact on these requirements. |
| Handover | When the PCell is handed over, RRC is reconfigured (SCells may be kept after HO or may be released based on the RRC reconfiguration) and hence we do not see a potential impact on the HO delay and interruption requirements. |
| Timing requirements | MRTD requirements may be impacted due to the deployment considerations. Further analysis is provided in a separate section. |
| RLM | RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell. Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected. Based on the switch pattern, RLM-RS on the PCell may be impacted or may not be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SCell do not have UL support (SDL), SCell UL is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM procedure on the PCell further. One may argue that RAN1 is the leading working group to define or investigate potential issues concerning the RLM. However, since the RAN1 TU for this WI is limited RAN4 is the leading WG for this WI, we think RAN4 should investigate the potential issues concerning RLM. |
| SCell activation delay | SCell activation delay depends on the SSB availability on the SCell. Due to the switch pattern the time required to receive SSB may be impacted. Due to this we think, SCell activation delay requirements may be impacted. |
| Link recovery procedures | Unlike RLM, BFD and CBD are supported for SCells. BFD and CBD RS can be SSB and/or CSI-RS. Based on the switch pattern, BFD and CBD may be impacted on the PCell and the SCell. Among the BFD and CBD, we think the CBD on the PCell requires special attention as the PCell UL should be active to support the SCell transmission. We suggest RAN4 to further investigate the procedures of CBD on PCell to make sure that it is recovered as fast as possible or at least not impacted due to switch pattern. |
| Active BWP switch delay | BWP switch involves RF and BB retuning, we think the switch pattern do not impact BWP switch delay. |
| Active TCI state switch delay (including unified TCI state switch) | Switch pattern and the availability of the SSB will impact the TCI state switch delay. |
| UL spatial relation info switch | We think it is not applicable to FR1 |
| UE specific CBW change | Similar to BWP switch delay, we think the CBW change should not be impacted by switch pattern. |
| Pathloss reference signal switching delay | The PL-RS is supported for all the active serving cells, and PL-RS can be SSB and/or CSI-RS. We expect the PL-RS to be supported by SCell. Based on the switch pattern, PL-RS may be impacted on the PCell and the SCell. |
| TRP specific link recovery procedures | Can be impacted if supported |
| LTM related procedures | Need to discuss the support for it. Can the n5 or n12 can be configured as LTM candidate cell |
| Measurement related procedures | L3 and L1 Measurement delay: SSB, SMTC, MG, DRX, CSI-RS configuration and location may be impacted by the switch pattern. Since the SSB, and CSI-RS can be supported by PCell and SCell, measurement requirements are impacted and needs further investigation of the impact. Measurement performance: we do not see any impact on the measurement performance. |

As shown in above table, based on the switching pattern design, we observe a significant impact on the RRM requirements. However, we believe it is unnecessary to discuss the number of samples required for the each RRM procedure again. We believe that the RRM impact can be addressed using couple of methods at least.

Method 1: Discussing a method to extend the delay if the SSB/SMTC/MG/CSI-RS/DRX ON periodicity, for different procedures is missed on one cell due to switching to another cell. If we agree on the principle of extending the delay, we can apply this principle to all the aforementioned procedures.

Method 2: At least for some of the requirements, without extending or relaxing the particular requirement, we can specify an applicability condition under which the requirements are valid for low-band CA via switching.

We think RAN4 need not choose between method 1 and 2. Some of the requirement’s impact can be addressed with method 1 and some other requirements impact can be addressed with method 2.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

Among the requirements that might be impacted, we believe the timing requirements need a different approach compared to the others, as they are not dependent on the number of samples. Other aspect RAN4 needs to discuss is the LB CA switching delay and potentiation interruptions. In the next section we discuss the timing requirements impact. And in the subsequent section we discuss the LB CA switching and interruption requirements.

## Timing requirements

In 3GPP TS 38.133, timing requirements are organized into the following subsections:

- UE transmit timing

- UE timer accuracy

- Timing advance

- Maximum Transmit timing difference (MTTD)

- Maximum receive timing difference (MRTD)

Since UE transmit timing is an uplink requirement, as long as there is a SSB within every 160ms, the UE transmit timing requirements should apply to the PCell. We do not foresee any impact on UE timer accuracy, timing advance, and MTTD. However, the MRTD requirement needs to be discussed considering the deployment scenario. The impact on timing requirements can depend on whether the SSB is present on the SCell or not.

### SSB on SDL

When SSB are available on both carriers the UE can maintain a DL synchronization reference to both the FDD-carrier and the SDL-carrier. This allows the UE RX to receive symbols on each carrier correctly, to maximize DL throughput.

Observation 1: When SSB are available on both carriers the UE can maintain a DL synchronization reference to both the FDD-carrier and the SDL-carrier

Observation 2: When SSB are available on both carriers, this allows the UE RX to receive symbols on each carrier correctly, to maximize DL throughput.

### SSB-less SDL

When SSB is available on only FDD-carrier and not on SDL-carrier the UE can synchronize to FDD-carrier and acquire a DL synchronization which is then used also on the SDL-carrier.

Observation 3: When SSB is available on only FDD-carrier and not on SDL-carrier the UE can synchronize to FDD-carrier and acquire a DL synchronization which is then used also on the SDL-carrier.

In 3GPP rel-18 NES WI the following was added to TS 38.133, section 8.3.2:

… For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB ....... and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, …

This means that 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

Observation 4: 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

We can re-use the system development in rel-18 NES since the use case in LB-LB CA with switching in rel-19 when there is no SSB on SDL-carrier is similar to the case we already standardized in RRM TS 38.133 for rel-18 NES inter-band CA without SSB on SCell.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying MRTD between collocated FDD-carrier and SDL-carrier for LB-LB CA via switching in rel-19.

We know that RTD=TAE+ ∆RF propagation. Since LB-LB CA with switching in rel-19 is collocated, then ∆RF propagation=0 and RTD=TAE.

Based on the equation above and the existing NES agreements, we therefore propose MRTD = CP, between collocated FDD-carrier and SDL-carrier for LB-LB CA via switching in rel-19, if the SSB less SCell is considered to be in scope.

- If the SSB less SCell is considered to be in scope, MRTD = CP, between collocated FDD-carrier and SDL-carrier for LB-LB CA via switching in rel-19.

## LB CA switching delay and potential interruptions

As per the WID, RAN1 will specify a semi-static switching pattern based on RRC configuration, and RAN4 to specify the switching delay and time mask for carrier switching.

We believe, a time mask is used to define the timing of signal transmissions to avoid interference and ensure efficient usage of the spectrum. This means that RAN4 need to define when a device should transmit or receive signals in these bands of operation.

Time mask and switch delay:

In the time mask definition, we believe multiple components are involved, including the switch delay. We believe, RF session is the right group to define this than the RRM session. The analysis for the same is provided in our companion paper R4-2501572.

Since the switch pattern, switch delay, and the time mask is known to the UE/NW, we do not think RAN4 need to define additional interruption or scheduling restrictions in the RRM session.

- RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional switch delay or interruption or scheduling restriction in RRM.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For a UE supporting low band CA via switching, SCell with SSB operation should be mandatorily supported.

- For low band CA via switching, SCell without SSB operation can be considered after the SCell with SSB operation is supported.

- RAN4 to further investigate the RRM requirements impact with at least one of the following methods.

- Method 1: Agree on the principle of extending the delay and apply this principle to all the aforementioned procedures.

- Method 2: Specify an applicability condition under which the requirements are valid for low-band CA via switching

Observation 1: When SSB are available on both carriers the UE can maintain a DL synchronization reference to both the FDD-carrier and the SDL-carrier

Observation 2: When SSB are available on both carriers, this allows the UE RX to receive symbols on each carrier correctly, to maximize DL throughput.

Observation 3: When SSB is available on only FDD-carrier and not on SDL-carrier the UE can synchronize to FDD-carrier and acquire a DL synchronization which is then used also on the SDL-carrier.

Observation 4: 3GPP RAN4 has already standardized SSB less cases as part of NES rel-18 WI.

- If the SSB less SCell is considered to be in scope, use NES rel-18 WI as baseline for specifying MRTD between collocated FDD-carrier and SDL-carrier for LB-LB CA via switching in rel-19.

- If the SSB less SCell is considered to be in scope, MRTD = CP, between collocated FDD-carrier and SDL-carrier for LB-LB CA via switching in rel-19.

- RRM to reuse the RF defined switch delay and time mask. RAN4 RRM not to define additional switch delay or interruption or scheduling restriction in RRM.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537