3GPP TSG RAN WG4 Meeting #114	R4-2502062

Athens, Greece, 17 Feb. 2025 – 21 Feb. 2025

Agenda Item:	7.22.3.1

Source: 	Ericsson

Title:	Discussion on UE initiated BM requirements

Document for:	Discussion

# Introduction

In this contribution we analyze the potential RAN4 RRM impacts for NR MIMO Phase 5.

# Discussion

Event triggered measurement requirements

Event triggered measurement delay:

For defining event triggered measurement reporting requirements, RAN4 needs to discuss and decide on the start point, end point, measurement period, the event evaluation time, and the measurement report preparation time.

Regarding the start point and end point, following was agreed in last meeting [R4-2420096].

- Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air.

- Ending point is the point when the UE transmits a first PUCCH triggered by the event.

Further, regarding the measurement period definition following WF is agreed in last meeting.

- Measure both the current beam and new beam after the starting point, FFS the details.

- Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion.

- Start the RAN4 RRM discussion for FR2.

- Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

We believe the agreement regarding the measurement period results from the following two scenarios:

- Scenario 1: The new beam periodicity is equal to the current beam periodicity.

- Scenario 2: The new beam periodicity is not equal to the current beam periodicity.

Considering these scenarios, we can define the measurement period for event evaluation based on when the event is detected over the air. This leads to four possible cases for UE event realization, as illustrated in Figure 1:

Figure 1: Different points for UE event realization.

- Case A: Only the current beam needs to be measured for the UE to realize the event.

- Case B: Both the current beam and the new beam need to be measured for the UE to realize the event.

- Case C: Only the new beam needs to be measured for the UE to realize the event.

- Case D: Both the current beam and the new beam need to be measured for the UE to realize the event.

The primary reason for these four cases is the difference in periodicity between the new and current beams. If the periodicities are the same, we think these four cases do not need to be considered. Additionally, a similar issue exists in legacy event-triggered reporting. In legacy L3 event triggered reporting, we do not distinguish whether the UE realizes the event after the serving cell measurement or the neighbour cell measurement.

However, RAN4 requirements are defined keeping testability in mind. In the test, we change the signal strength of the target cell to meet the event condition and check if UE can evaluate the event under the specified delay. We think the similar approach can be followed here. Hence, we think the measurement period definition does not need to not consider the current beam periodicity but the new beam periodicity. if we define the event triggered reporting as per case C or case D, we cannot use the same delay equation to the other events than the event 2. We list the other event agreed in RAN1 below.

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise The additionally supported events will be lower priority compared to event 2.

Since the UE can evaluate the event entering condition at any point, we propose that RAN4 define a generic requirement (similar to L3 event-triggered reporting) where the UE is expected to evaluate the event after every measurement sample completion. We think RAN4 should specify the UE measurement period of a RS that triggers the measurement report. The RS that may trigger the measurement report may be current beam or new beams.

Measurement period of a RS whose measurement may triggered the measurement report: In last meeting it was agreed to start the discussion with FR2 for discussing the measurement period requirements. For further discussion, the legacy measurement reporting delay for FR2 is copied below for the reference (Table 9.5.4.1-1 of TS 38.133).

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

UE measurement occasions for L1-RSRP depend on following factors:

- Report periodicity (TReport)

- SSB periodicity (TSSB)

- Number of samples (M)

- Rx beam sweeping factor (N)

- Sharing factor (P Sharing with inter-cell L1 and L3)

- DRX period (TDRX)

According to the current L1 measurement period requirement, measurement occasion depends on the measurement reporting periodicity. This is because, even if the UE measures frequently, it can only report as often as the measurement report period. However, we think the same doesn't apply to L1 event-triggered reporting, since the UE doesn't know when the event entering condition will be met.

For event-triggered reporting, we think all parameters, except for TReport in the measurement delay, can stay the same. In other words, for L1 event-triggered reporting, the UE needs to measure at every measurement occasion to complete the measurement of a RS.  Considering the existing measurement scaling rules, we can define the SSB-based L1-RSRP measurement period for a RS that may trigger the measurement report can be defined as shown in the following table.

| Configuration | TL1-RSRP_event triggered_measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. |  |

- SSB based L1-RSRP measurement period for the SSB configured for event triggered measurement is given by following table.

| Configuration | TL1-RSRP_event triggered_measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. |  |

Similar to the SSB, CS-RS measurement period can be defined as shown in below proposal.

- CSI-RS based L1-RSRP measurement period for the CSI-RS configured for event triggered measurement period is given by following table.

| Configuration | TL1-RSRP_ event triggered _measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TCSI-RS |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TCSI-RS) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

Though RAN1 did not agree on the filtering, RAN1 agreed on defining a counter or timer (e.g., similar to time to trigger) to prevent the false event triggered reports. RAN1 agreed the timer or counter as M. However, since RAN4 L1-RSRP measurement requirements already consist of a parameter ‘M’ we suggest using different parameter name than the M, e.g., M1.  When NW configures the timer or counter, UE supposed trigger the UE initiated beam report after the event is met over M1 L1-RSRP samples over a period of time. That means, UE may need to evaluate the event for M1 number of times before sending the event triggered measurement report. However, for the RAN4 requirements, RAN4 need not consider the counter or timer as TTT was not considered in L3 event triggered measurement reporting requirements. Moreover, the counter is a NW configurable parameter and if the UE can meet the RAN4 requirement without counter, UE can meet the RAN4 requirement with counter being configured too.

- RAN4 not to consider counter or timer in the event triggered reporting as L3 measurements did not consider that. That means M1=1 in RAN4 requirements.

Other components:

Since the L1-RSRP from the event triggered report is used for beam management, UE should send the report as soon as event condition is met. Event triggering conditions can be changed as fast as the measurement occasions. Hence, UE shall evaluate the event after every measurement occasion to check if any of the configured event is met. We understand that RAN1 is also discussing this issue, and we believe RAN1 should decide on the same.

- Event evaluation period for a SSB or CSI-RS can be decided in RAN1.

Once the event is met, UE needs to send the indication on the first UL channel to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. RAN4 to further discuss the components and criteria for defining TTime to first UL channel.

- RAN4 to further discuss the components and criteria for defining TTime to first UL channel.

Based on the above discussion we suggest the event triggered measurement reporting delay as TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel or TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel.

- Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel.

Inter-cell scenario for UEIBM

Whether to consider intra-cell and inter-cell scenario for defining event triggered reporting for CSI-RS was discussed in last meeting. We provide our view on the inter-cell scenario in this section.

In Rel-15, SSB based L1-RSRP requirements were defined for intra-cell scenario where all the SSB in a cell are associated with single PCI. In Rel-17, RAN1 introduced inter-cell beam management and inter-cell multi-TRP scenario where SSB of the cell can be associated with different PCI, where the SSB with different PCI are typically transmitted from different TRP associated with same serving cell. In Rel-17, RAN4 defined L1-RSRP measurement requirements for inter-cell multi-TRP scenario where one TRP can be associated with serving cell PCI and other TRP can be associated with different PCI than the serving cell PCI. However, in that WI, only SSB was considered, and CSI-RS based inter-cell L1-RSRP measurements were not considered during that WI.

CSI-RS based L1 measurement requirements are defined in Rel-15 and CSI-RS is assumed to be QCL’ed with SSB. As per the applicability of CSI-RS requirements shown below, SSB which is QCLed with CSI-RS can be intra-cell and inter-cell.

For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set. For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON.

One of the first agreement RAN1 made for this work item is shown below. Since supporting intra-cell and inter-cell scenario was one of the first agreement by RAN1, it signifies the importance of considering both the scenarios.

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Hence, we propose

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

When UE is configured with CSI-RS measurements in inter-cell scenario, SSB associated with CSI-RS may be from different TRP (different PCI) and in FR2, they may need to be received with different QCL Type D. Since multi-Rx may not always be assumed, UE may be able to measure only one TRP CSI-RS at a time when the CSI-RS from different TRP are overlapping in time. RAN4 may need to define sharing or scaling rules for the inter-cell CSI RS measurements. One may argue that SSB based inter-cell L1-RSRP measurements are defined during a dedicated WI in Rel-17 and may contain big workload to define CSI-RS based inter-cell L1-RSRP measurement requirements. We think that RAN4 can reuse the principles of sharing agreed for SSB based measurements to derive the requirements for CSI-RS based inter-cell L1-RSRP measurements.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements.

Impact on unified TCI state switching delay

In last meeting there was a discussion on whether to define new TCI state switching delay or existing requirements are sufficient for the unified TCI state switching based on event triggered reporting. To analyze the requirements, we first look at the events agreed in RAN1 to discuss whether there is an impact on the unified TCI state switch delay.

RAN1 agreed on following events (event 1 and 7 are working assumption)

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

  - M is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

The intention or use case of each of the event is provided below

- Event-1: when the quality of the current beam is lower than certain threshold NW may request the aperiodic report and schedule a beam change.

- Event 2: when quality of the new beam is better than the current beam, NW can trigger the beam switch based on the report from the UE.

- Event-7: When a new beam which was not in the active TCI state list is better than beams in active TCI state list, based on the UE report, NW update the active TCI state list.

Currently RAN1 supports two alternatives for unified TCI state activation. Based on the use case for which the events are defined, we think the RAN1 defined events support the existing TCI state activation framework listed below.

Alternative 1:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE activating some of the TCI states from the RRC configured TCI states

- Step 3: DCI indicating one of the MAC CE activated TCI states.

Alternative 2:

- Step 1: RRC configuring large pool of TCI states

- Step 2: MAC CE directly activating a TCI state from the RRC configured TCI states

In some implementations, after receiving the measurement report when the event 2 trigger conditions are met, if a new beam which was not in the active TCI state list is better than the current beam, then NW may decide to switch the current beam to the new beam. In this scenario, Alternative 1 may cause extra delay for switching the current beam to best beam. However, we think that this can be supported with Alternative 2 as described above. Hence, we do not see a need for new requirements definition.

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

Other enhancements

Following two issues are brought as part of the other enhancements during last meeting. Though these are good to have, we think both of the issues are out of scope for this WI and do not see a need for specifying them in Rel-19 as they are not specific to event triggered beam reporting.

Whether to enhance beam sweeping in event triggered L1 beam reporting?

<Way forward>

•	FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management?

<Way forward>

•	Option 1: No new accuracy requirements for existing L1 measurements are required.

•	Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

- RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements.

UE capability for UE IBM

Capabilities for Support of Event Triggering and Reporting Criteria? - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Apple) - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - Option 3: (Ericsson) - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP. - Option 4: (MediaTek) - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency.

Since the UE IBM is for the MIMO and MIMO generally is defined for single serving cell. Even for the ICBM or multi-TRP, the TRPs are considered as a single cell. For a single cell, as per our understanding RAN4 generally does not define intra-frequency or inter-frequency. We agree with proposal 2 in the above WF that RAN1 should handle the capability definition.

- UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

Compatibility with LTM:

In LTM serving cell can be configured as a candidate cell and event triggered measurement reporting can be configured on serving cell. That means UE may be configured with UEIBM for serving cell and event triggered reporting for the same cell from LTM candidate cell perspective. Though the cell may be same, the event frameworks between MIMO and LTM are different. To avoid frequent reports due to fast fading, MIMO agreed on counter-based mechanism and LTM agreed on timer-based mechanism, We think UE may implement them as a different HW capability. Though the same RS and thresholds are configured, the counter and timer should be implemented separately. Hence, we think it may be reasonable to assume they need to be treated as different event from the UE capability perspective.

Having said that, there are further details like which layer maintain or evaluate the events are still discussed in RAN1/2. Hence, we think it may be little premature to agree on the UE capability without all the details in place. We also would like to point that UE capability discussions are takes place generally in RAN1 at the end of the core part of the WI. Hence, we think RAN4 should leave the UE capability discussion to RAN1. If RAN1 do not define this, then RAN4 can define.

- RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- SSB based L1-RSRP measurement period for the SSB configured for event triggered measurement is given by following table.

| Configuration | TL1-RSRP_event triggered_measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = is the periodicity of the SSB-Index configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. |  |

- CSI-RS based L1-RSRP measurement period for the CSI-RS configured for event triggered measurement period is given by following table.

| Configuration | TL1-RSRP_ event triggered _measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TCSI-RS |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TCSI-RS) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement that may trigger the measurement report. TDRX is the DRX cycle length. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- RAN4 not to consider counter or timer in the event triggered reporting as L3 measurements did not consider that. That means M1=1 in RAN4 requirements.

- Event evaluation period for a SSB or CSI-RS can be decided in RAN1.

- RAN4 to further discuss the components and criteria for defining TTime to first UL channel.

- Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel.

- Event triggered measurement report requirements are applicable for intra and inter-cell scenario for SSB and CSI-RS.

- RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2.

- Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.

- RAN4 not to specify new Rx beam sweeping factor or new accuracy requirements as part of the event triggered measurement requirements.

- UE Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added.

- RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility.

# References

[1]		R4-2420096, “WF on RRM requirements for NR_MIMO_Ph5”, Samsung

[2] 	RP-234007, New WID: NR MIMO Phase 5