3GPP TSG-RAN WG4 Meeting #114 	R4-2502083

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item: 7.23.3.1

Source: MediaTek inc.

Title: Discussion on SBFD simulation performance

Document for: Discussion

# Introduction

In the previous meeting RAN4 agreed to run simulation [1] to provide performance results based on the agreed simulation assumptions [2]. In this document, we provide our simulation performance.

# Simulation performance

Following the agreed simulation assumptions, we have performance based on the mandatory simulation assumptions:

- Channel: AWGN Channel

- SCS: 15kHz, 30kHz, 60kHz, and 120kHz SCS

- SNR: 1dB/3dB SNR

- num. of measurement sample: 1/2/3 L1 sample

- SRS BW: 48 PRBs

- SRS symbol: 1 OFDM symbol

- performance metric: 5%/50%/95%-tile delta RSRP

Table 1: simulation performance for SR-RSRP measurement accuracy (delta RSRP) for Case 1 (4.67us for FR1, 3.67us for FR2)

| FR | SCS | SRS BW | Sample | MTK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |  |  |
|  |  |  |  | 1 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% |
| FR1 | 15 | 48 | 1 | -0.85 | 0.15 | -0.78 | -0.01 |
|  |  |  | 2 | -0.75 | -0.05 | -0.7 | -0.17 |
|  |  |  | 3 | -0.71 | -0.13 | -0.67 | -0.226 |
|  | 30 | 48 | 1 | -1.43 | -0.34 | -1.35 | -0.53 |
|  |  |  | 2 | -1.32 | -0.57 | -1.27 | -0.69 |
|  |  |  | 3 | -1.28 | -0.65 | -1.23 | -0.75 |
|  | 60 | 48 | 1 | -2.84 | -1.56 | -2.74 | -1.77 |
|  |  |  | 2 | -2.71 | -1.81 | -2.66 | -1.96 |
|  |  |  | 3 | -2.68 | -1.92 | -2.63 | -2.05 |
| FR2 | 60 | 48 | 1 | -2.18 | -1.02 | -2.09 | -1.21 |
|  |  |  | 2 | -2.08 | -1.24 | -2.02 | -1.38 |
|  |  |  | 3 | -2.04 | -1.34 | -1.98 | -1.46 |
|  | 120 | 48 | 1 | -5.34 | -3.56 | -5.2 | -3.85 |
|  |  |  | 2 | -5.17 | -3.87 | -5.07 | -4.1 |
|  |  |  | 3 | -5.08 | -4.03 | -5.01 | -4.22 |

Table 2: simulation performance for SR-RSRP measurement accuracy (delta RSRP) for Case 2 (1.67us for FR1, 0.67us for FR2)

| FR | SCS | SRS BW | Sample | MTK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |  |  |
|  |  |  |  | 1 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% |
| FR1 | 15 | 48 | 1 | -0.5 | 0.45 | -0.43 | 0.31 |
|  |  |  | 2 | -0.4 | 0.26 | -0.35 | 0.15 |
|  |  |  | 3 | -0.37 | 0.184 | -0.33 | 0.1 |
|  | 30 | 48 | 1 | -0.68 | 0.29 | -0.62 | 0.13 |
|  |  |  | 2 | -0.59 | 0.09 | -0.55 | -0.03 |
|  |  |  | 3 | -0.56 | 0.02 | -0.52 | -0.08 |
|  | 60 | 48 | 1 | -1.09 | -0.07 | -1.01 | -0.22 |
|  |  |  | 2 | -0.98 | -0.27 | -0.94 | -0.39 |
|  |  |  | 3 | -0.95 | -0.35 | -0.91 | -0.45 |
| FR2 | 60 | 48 | 1 | -0.61 | 0.35 | -0.54 | 0.2 |
|  |  |  | 2 | -0.51 | 0.16 | -0.47 | 0.05 |
|  |  |  | 3 | -0.48 | 0.08 | -0.44 | -0.01 |
|  | 120 | 48 | 1 | -0.92 | 0.07 | -0.86 | -0.08 |
|  |  |  | 2 | -0.83 | -0.12 | -0.78 | -0.51 |
|  |  |  | 3 | -0.79 | -0.21 | -0.75 | -0.3 |

# LS in from RAN1

In meeting#XX, RAN4 sent an LS to RAN1 asking whether to consider intra-cell and/or inter-cell measurements. In this meeting, RAN4 received response from RAN1 which can be summarised as:

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

In order to limit RAN4 workload, the requirements can be defined only for intra-cell UE-to-UE CLI measurements. Therefore, only Case 2 should be considered.

- RAN4 to consider the scenario of for intra-cell UE-to-UE CLI measurements only.

# Summary

Simulation performance is provided based on the agreed simulation assumptions.

Proposal 1: RAN4 to consider the scenario of for intra-cell UE-to-UE CLI measurements only.

# References

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Hefei, China, Oct 2024.

- R4-2416910, “Simulation assumptions for L1 SRS-RSRP measurement”,